3GPP TSG-SA3 Meeting #102-e 	S3-210462-r1

- e-meeting, 18 - 29 January 2021											Revision of S3-20xxxx

- Source:	Samsung

- Title:	Key Issue on Authorization for MUSIM optimizations

- Document for:	Approval

Agenda Item:	5.19

# 1	Decision/action requested

It is requested to approve the proposed key issue on the need for authorization to perform MUSIM optimizations.

# 2	Rationale

Unauthorised exposure of the private information (like, MUSIM assistance information indicating that the UE is holding multiple active subscriptions), to the serving network (may be to the same operator or to a different operator) and/or to the 3rd party servers is a privacy concern. Therefore, it is required for the system to have a mechanism to verify the authorizations before providing /obtaining the MUSIM optimizations.

# 3	Detailed proposal

********* START OF CHANGES *********

- 5.a	Key issue #x: Authorization for MUSIM optimizations

- 5.a.1	Key issue details

The MUSIM procedures related to performance improvement are detailed in TR 23.761 [2], for example, Paging request messages includes Paging cause, Busy indication, like so. TR 23.761 [2] requires Multi-SIM capable UE sharing the details to the serving network about presence of the other USIMs in the device, in order to obtain MUSIM optimizations (for example, UE providing the MUSIM assistance information to the SN).

As the USIMs within a UE may belong to different purposes, subscriber and/or home network may have privacy concerns in indicating that Multi-USIM device is with multiple active subscriptions to the serving network, for e.g., to avoid poaching of its subscriber, subscriber does not want to reveal that he/she holding multiple subscriptions to avoid mapping of personal and business subscriptions.

Providing the information (like, UE is holding multiple active subscriptions), which may be a private information of the UE and/or of the home network, to others (may be same/another operator or 3rd party servers) is privacy concern without authorization.

This key issue covers both, authorization of the UE to obtain the MUSIM optimizations from the serving network and authorization of the home network to provide MUSIM optimizations to the UE.

- 5.a.2	Threats

Unauthorised exposure of the private information (like, MUSIM assistance information indicating that the UE is holding multiple active subscriptions), to the serving network (may be to the same operator or to a different operator or 3rd party servers) is a privacy concern.

- 5.a.3	Potential security requirements

MUSIM optimizations shall be provided by the serving network to the UE, only if authorised by the UE and also by the home network.

The UE shall request MUSIM optimization from the network, only if the home network and the user authorize the UE to obtain the MUSIM optimization services.

The UE shall exchange the MUSIM optimization related information with the network, only if the home network and the user authorize the UE to exchange the information related to the MUSIM optimization.

*********** END OF CHANGES *************