3GPP TSG-SA3 Meeting #102-e 	S3-210471r1

- e-meeting, 18th - 29th January 2021												Revision of S3-20xxxx

- Source:	Qualcomm Incorporated

- Title:	Proposed solution for UAV authorisation when connected to 4G

- Document for:	Approval

Agenda Item:	5.7

# 1	Decision/action requested

This contribution propose a new solution for a UAS connected to 4G for the UAS TR

# 2	References

[1]	TR 33.854 v0.3.0 (S3-203467)

# 3	Rationale

This contribution proposes a solution for authorisation when a UAV is connected to 4G.

It is proposed that SA3 approve the inclusion of the solution in the TR [1].

# 4	Detailed proposal

It is proposed that SA3 approve the inclusion of the below pCR in the TR [1].

**** START OF CHANGES ****

## 6.X	Solution #X: Authorisation of UAV/UAVC when connected to EPS

### 6.X.1	Solution overview

This solution addresses Key issues #1: and Key issue #2: Pairing authorization for UAV and UAVC

### 6.X.2	Solution details

#### 6.X.2.1	General

The solution uses a UAV Flight Enablement Subsystem (UFES) as a single point of contact between the PLMN and USS/UTM in order to limit the impact on the 3GPP system, although it is not strictly necessary for authentication and authorisation solution to work. The authentication and authorisation procedures are shown when connected to EPS and the authentication/authorisation takes place during PDN connection establishment. The procedure in 6.X.2.2 are used to authenticate and authorize a UAV.

#### 6.X.2.2	Authentication and authorisation

Figure 6.X.2.2-1 shows how the UAV can be authenticated and authorised by the USS/UTM when connected to EPS.

Figure 6.X.2.2-1: Authentication and authorisation of a UAV connection to EPS

The steps are as follows:

1. The UAV sends an Attach Request to the MME. The UAV includes the Aviation Connectivity payload which contains the allocated CAA-Level UAV ID and flight/pairing information in the message.

2. The MME authenticates the UAV and establishes the security.

3. The MME determines the subscription is an aerial subscription and selects the SMF+PGW-C to establish the default bearer.

4. The MME sends a Create Session Request message to the SMF+PGW-C. The message includes the Aviation Connectivity payload.

5. The SMF+PGW-C responds with a Create Session Response. At this point the UAV is restricted from sending user plane traffic.

6. The MME sends an Attach Accept message to the UAV.

7. The UAV responds with an Attach Complete message to the MME.

8. The SMF+PGW-C requests a UAV authentication and authorisation from the UFES and includes the Aviation Connectivity payload in the request.

9. The UFES forwards the information to the USS/UTM.

10a. and 10b. There can be several round trips required for authentication of the UAV by the USS/UTM depending on the authentication method used by the USS/UTM and UAV. The authentication method and the content of messages used for authentication are out of scope of 3GPP. The content of the messages is carried in containers that are passed along and not processed by the entities between the UAV and USS/UTM.

11. On a successful authentication and authorisation of the UAV, the USS/UTM stores the 3GPP UAV ID with the CAA-level UAV ID. The USS/UTM informs the UFES that the UAV has been successfully authenticated and authorised by the USS/UTM. The USS/UTM includes authorisation information for both the network and the UAV.

12. The UFES further informs the SMF+PGW-C that the UAV has been successfully authenticated and authorised by the USS/UTM. The UFES passes the received authorisation information onto the SMF+PGW-C. The SMF+PGW-C stores the network authorisation information as part of the UE context. The network authorisation information further contains the information whether USS/UTM authentication and authorisation is required during future registrations and whether to allow UE to establish PDN connections(s) dedicated for the UAS service with or without further USS/UTM authentication and authorisation. The network part of the authorisation data contain authorisation information applicable to existing PDN connections, which influence SMF+PGW-C decisions for the traffic on these connections. For example, the information may indicate to disable all connectivity of the UAV except for the connectivity to USS/UTM.

13. The SMF+PGW-C sends the Update Bearer Request message to the MME and include the UAV authorisation information. The MME responds with the Update Bearer Response message. The SMF+PGW-C also set the traffic filters to allow traffic based on the received authorisation information.

14. The MME passes the UAV authorisation information to the UAV to inform the UAV that the authorisation was successful. The UAV authorisation information contains any needed aviation information, e.g. a new CAA-level UAV ID.

15. If using different PDN connections for C2 traffic, the UAV triggers a PDN connection set-up procedure which may include a further UAV authentication and authorisation.

16. C2 traffic can start to pass between UAV and UAVC,

#### 6.X.2.3	Revocation

TBD

#### 6.X.3	Solution evaluation

TBD

**** END OF CHANGES ****