3GPP TSG-SA3 Meeting #102e 	draft-S3-210499-r1

e-meeting, 18-29 January 2021

- Source:	Ericsson

- Title:	New WID on 3GPP profiles for cryptographic algorithms and security protocols

- Document for:	Approval

Agenda Item:	4.21

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 3GPP profiles for cryptographic algorithms and security protocols

## Acronym: CryptPr17

## Unique identifier: 	XXXXXX

Potential target Release: Rel-17

Note that this field above indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan.

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No |  |  |  |  |  |
| Don't know | X |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
|  |  |  | None |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  | None |  |

Dependency on non-3GPP (draft) specification:

Internet draft draft-ietf-tls-dtls13: "The Datagram Transport Layer Security (DTLS) Protocol Version 1.3"

Internet draft draft-westerlund-tsvwg-dtls-over-sctp-bis: " Datagram Transport Layer Security (DTLS) over Stream Control Transmission Protocol (SCTP)"

## 3	Justification

3GPP specifications use several security protocols from other standards bodies, for example IPsec (IKEv2, ESP), TLS, DTLS, DTLS/SCTP, EAP-TLS, SRTP, JOSE, and Internet X.509, which are standardized by the IETF. For each of these protocols, the 3GPP security protocol profiles specify which version, options, extensions and cryptographic algorithms to support and use. In addition to updates from IETF, attacks and recommendations on how to use these protocols are published by academia and other organizations. The 3GPP security protocol profiles need to be regularly updated to stay up to date and remain secure.

DTLS 1.3 is expected to be published in early 2021 obsoleting DTLS 1.2 and bringing similar security and performance benefits as TLS 1.3. Like TLS 1.3, DTLS 1.3 is expected to be implemented and deployed very quickly. Similar to TLS 1.3, 3GPP should mandate support of DTLS 1.3 as soon as possible. TLS 1.3 is mandatory for network since Rel-15 and for UE since Rel-16.

RAN3 has discovered that the 16 kB message size limitations of DTLS/SCTP (RFC 6083) will be problematic for the largest message sizes expected in 5G. To mitigate this problem, IETF has started work on RFC6083bis (draft draft-westerlund-tsvwg-dtls-over-sctp-bis). 3GPP should replace RFC6083 with RFC6083bis.

In Rel-16, 3GPP aligned with recommendations from ANSSI, BSI, and NIST to frequently perform Diffie-Hellman key exchange in order to limit the impact of a key compromise. More frequent use of Diffie-Helman increases the latency requirements on the Diffie-Hellman algorithm. 3GPP should introduce Diffie-Hellman group 31 (Curve25519) as should support. Curve25519 was chosen for SUCI protection due to its security properties and high performance and NIST plan to standardize it (Draft SP 800-186).

During Rel-16 it was discovered that some use of (D)TLS in 33.501 lacked certificate profiles, requirements on mutual authentication, and allowed weak and outdated versions and algorithms. To mitigate this from happening in the future, the 3GPP cryptographic profiles include a certificate profile that apply when certificates are used. The 3GPP crypto profiles should also apply to all uses of (D)TLS to protect 3GPP interfaces unless otherwise stated. Just like today, specification could make exceptions to the general profiles.

The ECDSA signature algorithm use a per-message nonce, which typically has been completely randomly generated. However, weaknesses in the random number generation can lead to leakage of the private key. RFC 6979 specifies how the per-message nonce is generated in a deterministic way. RFC 6979 is recommended by e.g., TLS 1.3 and NIST plan to also standardize its use (Draft FIPS 186-5). RFC 6979 should be recommended for 3GPP X.509 and IKEv2. 3GPP should at least reference RFC 6979 as an optional implementation. Deterministic ECDSA (and all other ways to generate the per-message nonce) are compatible with existing implementations as the receiver does not have to be aware of how the nonce is generated. Note that deterministic ECDSA is problematic in IoT devices where it may enable an attacker to perform side-channel and fault attacks (draft-mattsson-cfrg-det-sigs-with-noise-02).

The IMS specification (used by VoLTE/VoNR) make use algorithms such as MD5, SHA-1, and key exchange without Diffie-Hellman, which are weak and no longer recommended to use. 3GPP should mandate support of strong and recommend algorithms such as SHA-256, so that MD5 and SHA-1 can be phased out long-term. If possible, 3GPP should refer to RFCs or Internet-drafts. 3GPP should also make a common profile for all use of SRTP that mandates support for AES-GCM and DTLS-SRTP. Support for these are currently only mandatory for some use cases.

CA certificates typically use algorithms with a higher security than end-entity certificates. The current certificate mandates support of RSA-4096, which is typically used in CA certificates. For ECDSA, no algorithm with a higher security is mandated and for both RSA and ECDSA there is no hash function with higher security is mandatory to support. 3GPP has recommended ECDSA over RSASSA-PKCS1-v1_5 for a long time. While high security for IPsec and TLS is should support, algorithms for CA certificates should be shall support. 3GPP should make SHA-384 and ecp384r1 shall be supported.

To enable mutual authentication in an interoperable way, 3GPP specification need to specify which identity field(s) in the X.509 certificates that should be checked during authentication, how these fields are formatted, and what the fields should be checked against. The authenticated information is then used for authorization and policy control. 3GPP should make the profiling of the non-cryptographic identity fields in X.509 certificates stricter to enable interoperability between different implementations, this is especially true for SBA, which is used between operators.

Several 3GPP specifications are tightly bound to Certificate Revocation Lists (CRLs). Since the introduction of Internet X-509 Certificates and CRLs (RFC 5280), CRLs has been complemented by OCSP and OCSP stapling. Many systems in use several revocation mechanisms to enable the best possible trade-off between security, availability, and latency. In Rel-16, 3GPP introduces OSCP stapling as should support for TLS based on NIST’s must support requirement. 3GPP should change its revocation requirements to “do revocation checking” instead of “do revocation checking with CRL”. 3GPP should also make and OCSP profile similar to the CRL profile as well as any necessary updates to the certificate profiles.

## 4	Objective

Update of the 3GPP security protocol profiles for security algorithms from other standards bodies (e.g., IETF), considering the following:

-	Update the usage of (D)TLS according to other standards bodies’ works, e.g., new IETF works.

-	Update the elliptic curve crypto profiles in IKEv2.

-	Update on X.509 profiles, and make revocation more flexible by mandating revocation but not CRL.

-	Remove weak and non-recommended algorithms in the IMS specifications. Mandate support of the strong and recommended algorithms.

-	Add default certificate profiles to the 3GPP security protocol profiles. Add statement that the 3GPP security profile apply for all uses of (D)TLS to protect 3GPP interfaces unless otherwise stated.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
|  |  | None |  |  |  |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 33.141, TS 33.203, TS 33.210, TS 33.220, TS 33.222, TS 33.303, TS 33.310, TS 33.328, TS 33.501, | Update of 3GPP profiles for cryptographic algorithms and security protocols | SA#93 (Sep 2021) |  |

## 6	Work item Rapporteur(s)

Pinar Comak, Ericsson, pinar.comak@ericsson.com

## 7	Work item leadership

SA3

## 8	Aspects that involve other WGs

RAN3 for aspects related to DTLS/SCTP

CT1 for aspects related to IMS

## 9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval.}

Supporting IM name

Ericsson

Deutsche Telekom

NCSC