3GPP TSG-SA3 Meeting #102-e 	draft_S3-210517-r1

- e-meeting, 18 - 29 January 2021														Revision of S3-200517 and S3-210334

- Source:	Thales, Nokia, Nokia Shanghai Bell

- Title:	TR 33.846: comparison of candidate solutions

- Document for:	Approval

Agenda Item:	5.5

# 1	Decision/action requested

Comparision of candidate solutions for 3GPP TR 33.846.

# 2	References

[1]	3GPP TR 33.846 v0.9.0.3	Rationale

In order to ease discussions to agree on conclusions for 3GPP TR 33.846 [1], this contribution proposes a comparison of the candidate solutions, taking into account UE impacts, network impacts and addressed key issues.

There is no solution proposed for key issues #1.X, #3.2, and 3.2, as shown in Table 6.0-1 "Mapping of solutions to key issues" in clause 6.0 of 3GPP TR 33.846 [1]. Consequently, the comparison table below takes into account only Key Issues #2.1, #2.2, and #4.1.

Comparison table

|  | UE impacts | Network impacts |  | Key Issues addressed |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions |  | AMF/ SEAF | UDM/ ARPF | #2.1 | #2.2 | #4.1 |
| Solutions for resilience against identifier linkability |  |  |  |  |  |  |
| #2.1: Handling of Sync failure by AUTS encryption | ME | X | X | X |  | X |
| #2.2: Encryption of authentication failure message types by UE with new keys derived from K_AUSF | Probably ME | X | X | X |  | X |
| #2.3: Unified authentication response message by UE | USIM | X | X | X |  |  |
| #2.4: MAC-S based solution | USIM | X | X | X |  | X |
| #2.5: Encryption of authentication failure message with SUCI method | USIM or ME | X | X |  | X |  |
| #2.6: Certificate based encryption of unicast NAS message | USIM and ME | X | X | X |  | X |
| Solutions on re-synchronisation in AKA |  |  |  |  |  |  |
| #4.1: Using MACS as freshness in the calculation of AK | USIM |  | X |  |  | X |
| #4.2: Using symmetric encryption function to protect SQN during a re-synchronisation procedure in AKA | USIM |  | X |  |  | X |
| #4.3: SQN protection by concealment with SUPI in USIM | USIM |  | X | X | X | X |
| #4.4: SQN protection during re-synchronisation procedure in AKA | USIM and ME |  | X |  |  | X |

# 4	Detailed proposal

It is proposed that SA3 review and approve the following pseudo-CR to 3GPP TR 33.846 [1].

**************** START OF CHANGES

# 7	Conclusions

Editor’s Note: This clause contains the agreed conclusions that will form the basis for any normative work.

## 7.0	Overall evaluation aspects

### 7.0.1	Assessment of attack risk

Editor’s note: This clause should provide for all KIs addressed a clear statement (in a condensed way) on the security threat, i.e. the likelihood of the attack to happen under which conditions, level of severity.

|  | Security threats | Comments (e.g likelihood, level of severity) |
| --- | --- | --- |
| Key Issues |  |  |
| Key Issue #2.1 |  |  |
| Key Issue #2.2 |  |  |

### 7.0.2	Impact of solutions

Table: Comparison table of UE and network impacts per solution

|  | UE impacts | Network impacts |  | comments |
| --- | --- | --- | --- | --- |
| Solutions |  | AMF/ SEAF | UDM/ ARPF |  |
| Solutions for resilience against identifier linkability |  |  |  |  |
| #2.1: Handling of Sync failure by AUTS encryption | ME | X | X |  |
| #2.2: Encryption of authentication failure message types by UE with new keys derived from K_AUSF | Probably ME | X | X |  |
| #2.3: Unified authentication response message by UE | USIM | X | X |  |
| #2.4: MAC-S based solution | USIM | X | X |  |
| #2.5: Encryption of authentication failure message with SUCI method | USIM or ME | X | X |  |
| #2.6: Certificate based encryption of unicast NAS message | USIM and ME | X | X |  |
| Solutions on re-synchronisation in AKA |  |  |  |  |
| #4.1: Using MACS as freshness in the calculation of AK | USIM |  | X |  |
| #4.2: Using symmetric encryption function to protect SQN during a re-synchronisation procedure in AKA | USIM |  | X |  |
| #4.3: SQN protection by concealment with SUPI in USIM | USIM |  | X |  |
| #4.4: SQN protection during re-synchronisation procedure in AKA | USIM and ME |  | X |  |

NOTE: A mapping table on which solution is addressing which key issue is provided in clause 6.0.

**************** END OF CHANGES