3GPP TSG-SA3 Meeting #103-e 	S3-211485-r3

- e-meeting, May 17th – 28th 2021	Revision of S3-21xxxx

- Source:	InterDigital

- Title:	TR 33.847 Update for solution #24

- Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

It is proposed to approve this pCR in TR 33.847.

# 2	References

# 3	Rationale

The contribution proposes to address the following ENs and add an evaluation:

Editor’s Note: whether S-NSSAI is sent by a remote UE is to be in line with SA2.

The option for the Remote UE to transmit S-NSSAI is removed to align with TR 23.752 clause 8.3 on UE-to-Network Relay conclusion:

For the PDU Session relaying the Remote UE's traffic, the Relay UE can determine PDU session parameters based on RSC requested by the Remote UE according to the association between RSC and PDU session parameters as described in Sol#35. The Remote UE can select the Relay UE according to the association between RSC and PDU session parameters as described in Sol#35.

Editor’s Note: procedure for network controlled authorization is to be in line with SA2.

The procedure for network-controlled authorization is based on Solution #10 which builds on top of TR 23.752 Solution #47 with necessary enhancements to support NSSAA. TR 23.752 clause 8.3 on UE-to-Network Relay conclusion states that:

NOTE 2:	The procedures to support authentication of Remote UE and Relay UE by the network will be determined by SA WG3.

-	For the Remote UE to use the network resources (e.g. PDU Session and Network Slice) of the Relay UE's serving network, the network-controlled authentication and authorization procedures (e.g. as proposed in Solution #40 and Solution #47) will be determined in the normative phase with coordination with SA WG3. The alignment with the associated security procedures to authenticate the Remote UE and Relay UE will be carried out in normative phase via coordination with SA WG3.

NOTE 3:	No dedicated solution is documented in SA WG2 for the Network Slice Specific Authentication and Authorization (NSSAA) for a Remote UE. Whether and how to support of NSSAA for a Remote UE will be determined by SA WG3.

This above is further confirmed in SA2 incoming LS S3-211401.

Observation-1: SA2 listed TR 23.752 Solution #47 as a reference for network-controlled authentication and authorization procedure.

Observation-2: SA2 did not document any solution for NSSAA.

Observation-3: It is expected that SA3 will determine how to support authentication and authorization of Remote UE and Relay UE in general and in particular how to support NSSAA.

Based on the above it is proposed to remove the EN.

# 4	Detailed proposal

It is proposed to approve the following changes for inclusion in TR 33.847.

***	BEGIN OF CHANGES	***

## 6.24	Solution #24: NSSAA for Remote UE with L3 UE-to-Network relay

### 6.24.1	Introduction

The contribution proposes a solution to address KI #4: Authorization in the UE-to-Network relay scenario. The solution describes how to support a Network Slice-Specific Authentication and Authorization (NSSAA) procedure for a Remote UE connecting via a L3 UE-to-Network relay.

This solution proposes to use a Network-controlled authorization of Remote UE procedure as described in sol#10 (based on TR 23.752 Solution #47) with enhancements to enable the Remote UE to perform NSSAA for a given S-NSSAI (no dedicated solution is documented in TR 23.752[2] for the support of NSSAA for a Remote UE). The S-NSSAI subject to NSSAA is associated with the connectivity service (RSC) requested by the Remote UE. The RSC that requires Network-controlled authorization is configured in the Remote UE and the UE-to-Network relay which determines the associated S-NSSAI based on RSC requested by Remote UE (as per TR 23.752[2], Sol#35). The authentication messages for NSSAA of Remote UE are exchanged securely during or after the PC5 link establishment. Upon successful completion of NSSAA procedure by the Remote UE, the relay provides Remote UE access to the slice.

This solution assumes that the AMF serving the relay is always able to serve the Remote UE (same as solution #10, clause 6.10.1). Moreover, the S-NSSAI associated with the connectivity service provided by the Relay is assumed to be the same for the Remote UE and Relay UE or that it can be mapped to the same S-NSSAI in the Relay serving PLMN.

### 6.24.2	Solution details

#### 6.24.2.1	PC5 link establishment with L3 UE-to-Network relay to use an S-NSSAI subject to NSSAA

The procedure for PC5 link establishment with L3 UE-to-Network relay to use an S-NSSAI subject to NSSAA is depicted in Figure 6.24.2-1.

Figure 6.24.2.1-1: Procedure for PC5 link establishment with L3 UE-to-Network relay to use an S-NSSAI subject to NSSAA

0. Remote UE is provisioned with authorization parameters to act as Remote UE. Relay UE is provisioned with authorization parameters to act as a Relay UE. Relay has registered for the S-NSSAI(s) associated with the services that are provided by the relay, including for the S-NSSAIs that are subject to NSSAA.

1. Remote UE and Relay perform a discovery procedure.

2. The Remote UE determines from the configuration provided in step 0 that the relay service code/type discovered in step 1 is associated with a S-NSSAI that is subject to NSSAA (e.g., based on an indication parameter for the S-NSSAI). Based on this determination, the Remote UE sends a DCR message including the RSC, a Remote UE's identity (e.g.,) and NSSAA capabilities.

3. Upon receiving the DCR message, the Relay may determine that a network-controlled authorization based NSSAA for Remote UE is required to provide Remote UE access to the slice. The determination may be based on any of the following conditions:

- Relay has performed NSSAA for the S-NSSAI (e.g., as performed in step 0). For example, during NSSAA procedure, S-NSSAI is marked with an indication that it is subject to NSSAA.

- Based on configuration from step 0, the service provided (i.e., RSC) is associated with a particular S-NSSAI that is subject to NSSAA (e.g., based on an indication parameter for the S-NSSAI) or an indication that network-controlled authorization is required to use the relay service.

4. Upon receiving the DCR message including a Remote UE identity (e.g., SUCI), the relay decides to trigger a network-controlled authorization of Remote UE. The relay sends a NAS request message that includes the Remote UE id, the S-NSSAI and Remote UE's NSSAA capabilities.

NOTE 2: If the Remote UE has a 5G native context established with the Relay UE's serving PLMN, it provides its 5G-GUTI (instead of SUCI) and the primary authentication steps below may be skipped as described in solution #10.

5. The AMF checks that the relay is authorized to act as a relay and is authorized to provide access to the S-NSSAI (e.g., S-NSSAI is part of Relay's UE Allowed NSSAI). Upon successful check, the AMF decides to trigger a primary authentication of Remote UE via the relay.

6. The Remote UE performs a primary authentication procedure via the relay. Authentication messages are transported over NAS messages between the AMF and Relay. The NAS messages include an indication (e.g., Remote UE's GPSI, Remote User Id or any id provided by Remote UE in message 2) that the authentication messages are for the Remote UE. The relay forwards those messages transparently between Remote UE and AMF.

7. Upon successful authentication procedure, the AMF checks with Remote UE's UDM that Remote UE is authorized to use the relay and has S-NSSAI as part of its subscription. If S-NSSAI is subject to NSSAA, AMF verifies that Remote UE supports NSSAA from the capabilities received from the Relay. AMF registers with Remote UE's UDM (including information about serving Relay) to handle further UDM subscription notifications or to handle revocation/re-authentication requests for Remote UE from AAA-S.

Following successful subscription-based authorization checks, AMF generates key material to authorize and enable secure communication between the Relay and Remote UE. The generated key material is derived from the key material generated during the primary authentication with the Remote UE. Upon successful authentication procedure, the Remote UE generates key material for securing communication with relay the same way as AMF.

8. Upon successful completion of the authentication procedure, the AMF sends a NAS response message that includes the Remote UE id (e.g., GPSI, SUPI), the generated key material, the S-NSSAI, NSSAA status which    indicates whether NSSAA is to be performed, ongoing or successful (e.g., if initiated or performed successfully from a previous registration). If primary authentication or subscription-based authorization check fails, the NAS message indicate a failure cause (e.g., S-NSSAI not authorized).

NOTE 3: Remote UE may have performed successfully or initiated NSSAA for the S-NSSAI from a previous Registration with Relay UE's AMF or another AMF. In that case the Relay UE's AMF may retrieve the current NSSAA status from Remote UE context (as described in sol#10).

9. The relay establishes the PC5 link security with the Remote UE using the key material generated from step 7. This step is skipped in case of failure indication in message 8.

10. The relay may send an ack message to confirm the PC5 link security establishment to the AMF. AMF may trigger the NSSAA procedure upon receiving this message illustrated in Figure 6.24.2.2-1.

11. The relay sends a DCA message including indication for NSSAA status (e.g., pending, required, successful) and S-NSSAI to Remote UE. In case of failure indication in message 8, the relay sends a reject message to the Remote UE including the failure cause.

12. If NSSAA is required, a NSSAA procedure for Remote UE via Relay may be triggered by Relay as illustrated in Figure 6.24.2.2-1.

13. Upon successful completion of the NSSAA procedure, the relay sends a PC5 message (e.g., a PC5 Link Modification Request) that includes a successful NSSAA indication, the authorized S-NSSAI. If the NSSAA procedure fails, the relay may release the PC5 link indicating the failure cause.

In the above procedure the NSSAA procedure is triggered after the relay sends the DCA message to the Remote UE. Alternatively, the NSSAA procedure may be triggered during the PC5 link establishment (e.g., NSSAA triggered after successful completion of DSMC procedure and before sending DCA to Remote UE). Upon successful NSSAA procedure, the relay sends a DCA message that includes a successful NSSAA indication and the authorized S-NSSAI. If the NSSAA fails, the relay may send a PC5 reject message indicating the failure cause.

Editor's Note: Whether and how to verify that the Remote UE is authorized to access the DN associated with rqquested RSC is FFS.

#### 6.24.2.2	NSSAA of Remote UE connecting via L3 UE-to-Network relay

The procedure for NSSAA of Remote UE connecting via L3 UE-to-Network relay is depicted in Figure 6.24.2.2-1.

Figure 6.24.2.2-1: Procedure for NSSAA of Remote UE connecting via L3 UE-to-Network relay

1. The relay UE or the AMF may decide to trigger an NSSAA procedure for the Remote UE according to conditions as described in Figure 6.24.2.1-1 (i.e., successful completion of network controlled authorization procedure, confirmation of PC5 link security establishment from Relay). If the procedure is triggered by AMF steps 2 and 3 are skipped. If the procedure is triggered by Relay step 8 is skipped.

2. The Relay sends a NAS request message to the AMF that includes the Remote UE id (e.g., SUPI, GPSI) and S-NSSAI.

3. The AMF checks that the relay is authorized to act as a relay and is authorized to provide access to the S-NSSAI (e.g., S-NSSAI is part of Relay's UE Allowed NSSAI) and that NSSAA is to be performed for Remote UE to use S-NSSAI (e.g., based on NSSAA status associated with S-NSSAI/Remote UE stored in Relay UE context). Upon successful check, the AMF decides to trigger a primary authentication of Remote UE via the relay.

4. The Remote UE performs a NSSAA procedure via the relay. Authentication messages are transported over NAS messages between the AMF and Relay. The NAS messages include an indication (e.g., Remote UE's GPSI, SUPI) to inform the Relay that authentication messages are for the Remote UE. The relay forwards those messages transparently between Remote UE and AMF. AMF may receive authorization information from AAA-S for the Remote UE to use the S-NSSAI (e.g., a time limit).

5. Upon successful completion of the NSSAA procedure, the AMF updates the S-NSSAI status information associated with Remote UE in Relay UE context (e.g., mark S-NSSAI as Allowed for Remote UE).

6. The AMF sends a NAS message to the relay indicating the result of the NSSAA procedure, including an identity of the Remote UE, S-NSSAI and optionally authorization information associated with S-NSSAI (as provided by the AAA-S).

7. On the condition of successful NSSAA, the relay stores S-NSSAI authorization information for Remote UE.

8. The relay sends a NAS message to acknowledge message 6 from AMF.

The Relay UE proceeds with the rest of PC5 link setup with relay using S-NSSAI subject to NSSAA as illustrated in Figure 6.24.2.1-1.

#### 6.24.2.3	AAA-S triggered Authorization Revocation to use S-NSSAI for Remote UE

The procedure for AAA-S triggered Authorization Revocation to use S-NSSAI for Remote UE in Figure 6.24.2.3-1.

Figure 6.24.2.3-1: Procedure for AAA-S triggered Authorization Revocation to use S-NSSAI for Remote UE

0. Remote UE is connected to the Relay UE and authorized to use an S-NSSAI subject to NSSAA.

1. NSSAAF receives an authorization revocation request from AAA-S. The request includes the GPSI of the Remote UE.

NOTE: Existing protection mechanisms for communications between NSSAAF and AAA-S as per TS 33.501 [14] are assumed (e.g., for privacy of GPSI, S-NSSAI).

2. NSSAAF requests UDM to get the ID of the AMF serving the Remote UE. NSSAAF receives AMF ID and Relay UE GPSI (UDM has obtained info about Relay UE and its AMF prior, see clause 6.24.2.1, step 7).

3. NSSAAF notifies AMF about the S-NSSAI authorization revocation for Remote UE, providing both Remote UE and Relay UE's GPSI.

4. AMF locates the Relay UE context and removes the association of Remote UE with S-NSSAI in the Relay UE context. If no other S-NSSAI is used for Remote UE via the Relay UE, the AMF removes the Remote UE information from the Relay UE context. AMF de-registers from the Remote UE's UDM if Remote UE info is removed from the Relay UE context.

5. AMF sends a NAS command message to the Relay including the Remote UE ID (e.g., GPSI) and S-NSSAI to revoke authorization of Remote UE to use S-NSSAI.

6. The Relay UE discards S-NSSAI authorization information associated with Remote UE including Krelay/Krelay ID.

7. The Relay UE initiates a Link Release procedure and provides Remote UE with the appropriate link release cause.

8. The Relay UE sends a response to the AMF to ack the command.

The re-authentication/authorization procedure is similar to the authorization revocation procedure above. The differences are as follows:

- Upon receiving a re-authentication request from AAA-S via NSSAAF for the Remote UE, the AMF triggers NSSAA of Remote UE via Relay procedure as illustrated in clause 6.24.2.2.

- If NSSAA fails, Figure 6.24.2.3-1 steps 4 – 8 are performed.

#### 6.24.2.4	AAA-S triggered Authorization Revocation to use S-NSSAI for Relay UE

The procedure for AAA-S triggered Authorization Revocation to use S-NSSAI for Relay UE. The messages 1-3, 5 and  8 are the same as TS 33.501[14], Figure 16.5-1 steps 1-4.

Figure 6.24.2.3-1: Procedure for AAA-S triggered Authorization Revocation to use S-NSSAI for Relay UE

0. Remote UE is connected to the Relay UE and authorized to use an S-NSSAI subject to NSSAA.

1. NSSAAF receives an authorization revocation request from AAA-S. The request includes the GPSI of the Relay UE.

NOTE: Existing protection mechanisms for communications between NSSAAF and AAA-S as per TS 33.501 [14] are assumed (e.g., for privacy of GPSI, S-NSSAI).2. NSSAAF requests UDM to get the ID of the AMF serving the Relay UE. NSSAAF receives AMF ID.

3. NSSAAF notifies AMF about the S-NSSAI authorization revocation for Relay UE, providing Relay UE's GPSI and S-NSSAI.

4. AMF locates the Relay UE context and for each Remote UE associated with the revoked S-NSSAI in the Relay UE context the AMF removes the association of Remote UE with S-NSSAI (e.g., NSSAA status) from the Relay UE context. If no other S-NSSAI is used for Remote UE, the AMF removes the Remote UE information from the Relay UE context. AMF de-registers from the Remote UE's UDM if Remote UE info is removed from the Relay UE context.

5. AMF initiates a UCU procedure with the Relay to revoke the authorization for the relay to use the S-NSSAI.

6. For each Remote UE associated with the revoked S-NSSAI, the Relay UE discards S-NSSAI authorization information associated with Remote UE including Krelay/Krelay ID.

7. For each Remote UE connected to the relay and associated with the revoked S-NSSAI, the relay initiates a link release procedure.

8. The Relay completes the UCU procedure. If the Relay is left with no Allowed NSSAI and no default S-NSSAI can be provided to the Relay UE. The Relay is de-registered by the AMF, and the Relay UE releases any active link with Remote UE(s).

### 6.24.3	Evaluation

This solution proposes a mechanism to authorize a Remote UE to access a specific slice that requires NSSAA via a L3 UE-to-Network Relay using a network-controlled authorization procedure. This solution fully addresses the requirements in KI#4 in the case of slice access that requires NSSAA.

In this solution AMF is required to support triggering NSSAA for the Remote UE as part of a new network controlled authorization procedure instead of Registration procedure.

The solution supports Remote UE operating in or out of coverage.

Editor's Note: the impact on 5GC and the existing procedures needs to be evaluated.

Editor's Note: Further evaluation is FFS.

### ***	END OF CHANGES	***