3GPP TSG-SA3 Meeting #103-e 	draft_S3-211525-r1

- e-meeting, 17 - 28 May 2021

- Source:	LG Electronics

- Title:	Evaluation of Solution 13

- Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

It is requested to approve this modification for TR 33.847.

# 2	References

[1]	3GPP TR 33.847: “Study on Security Aspects of Enhancement for Proximity Based Services in 5GS (Release 17)”

# 3	Rationale

This contribution proposes to remove an EN in solution#13 as no further evaluation for this solution is expected. And one minor editorial correction is also proposed.

# 4	Detailed proposal

*****Start of change*****

### 6.13.3	Evaluation

The Solution #13 proposes to support secondary authentication of the Remote UE. With this solution the AAA server in the DN can authorize the Remote UE to give an access to the services using the PDU session of the L3 UE-to-NW Relay UE via PC5 link. In this solution the Remote UE and the UE-to-Network Relay UE exchange the EAP message via PC5 signalling and the UE-to-Network Relay UE relays the message between the Remote UE and SMF. SMF obtaining Remote UE's SUPI is assumed to be performed as part of a procedure addressed in solutions for KI#4.

Performing the secondary authentication for the remote UE over L3 UE-to-NW Relay UE may increase both NAS and PC5 signalling compared to not performing but it can be performed only when it is needed based on the subscription of the Remote UE and local configuration of the SMF.

- Editor's Note: Further evaluation is FFS

*****End of change*****