3GPP TSG-SA3 Meeting #103-e 	S3-211738

- e-meeting, 17 - 28 May 2021											Revision of S3-20xxxx

- Source:	Lenovo, Motorola Mobility

- Title:	Update to Solution #6

- Document for:	Approval

Agenda Item:	5.17

# 1	Decision/action requested

This pCR provides an update to solution #6 in TR 33.864.

# 2	References

[1]	3GPP TR 33.864 ‘Study on the security of Access and Mobility Management Function (AMF) re-allocation’.

# 3	Rationale

The pCR adds an example for clarification and provides few editorial corrections.

# 4	Detailed proposal

SA3 is kindly requested to agree the pCR below to TR 33.864.

*****Start of Change*****

## 6.6	Solution #6: Solution to provide Security context to AMF capable of serving the UE to ensure system availability

### 6.6.1	Introduction

The solution addresses Key issue #1 on Security of AMF re-allocation procedure. The solution enables provisioning of UE security context to the AMF capable of serving the UE to ensure system availability.

### 6.6.2	Solution details

The solution is based on the principle of verifying the initial AMF's capability and the UE's subscription information (i.e., slice subscription data), before the UE's security context can be provisioned to the initial AMF during the primary authentication to prevent system availability issues described in Key Issue#1. The Figure 6.6.2-1 describes the AMF capability and slice isolation requirement-based UE Security handling during primary authentication as follows. The essential adaptations required over initiation of primary authentication and authentication procedure is described here in Figure 6.6.2-1 and the rest follows similar to Clause 6.1.2 and clause 6.1.3 of TS 33.501.

Figure 6.6.2-1: AMF Serving Capability based Security handling during primary authentication

Step 1.		The UE sends the Registration Request with SUCI or 5G-GUTI to the initial AMF.

Step 2a-b.	The initial AMF  sends authentication request to the SEAF with SUCI by including the AMF Slice serving Capability set as ‘unknown’ (example., unknown can be represented with a value set to ‘0’). The AMF can set the AMF serving serving capability as unknown based on SUCI and if there is no slice related information (example. slice selection information or reroute due to slicing information) available for the UE. The AMF sends AMF slice serving capability as unknown if it has no available information which can enable the network to select the appropriate slice.

The SEAF further sends a Nausf_UEAuthentication_Authenticate Request message to the AUSF, which can contain SUCI, SNN and AMF Slice Serving Capability.

Step 3.		The AUSF stores the received SNN and SUCI temporarily in its local memory. The AUSF sends to the UDM a Nudm_UEAuthentication_Get Request containing, SUCI, and SNN.

Step 4.		The UDM/UDR performs SUCI deconcealment and authentication method selection as in TS 33.501 Clause 6.1.2. Further the UDM, determines, if there is any slice isolation required based on the available subscription information (ex., slice selection subscription data). If a slice isolation requirement information is available, then the UDM determines to provide the related information to the AUSF.

Step 5.		The UDM sends a Nudm_UEAuthentication_Get Response to the AUSF, which contains an AV (i.e., EAP-AKA' AV/5G HE AV as in 33.501 based on authentication method), SUPI and Slice Isolation Required indication.

Step 6.		The AUSF performs method specific message exchange with the UE to perform primary authentication as in 33.501 Clause 6.1.3 (steps 3-8/9 for EAP-AKA' or 3-11 for 5G AKA). The solution proposes no changes to the challenge request/response message exchanges involved in the primary authentication.

Step 7a.	The AUSF if finds that the RES* verification (if 5G AKA) or Authentication challenge verification (EAP-AKA') is successful, then the 5G network considers the primary authentication as successful.

Step 7b.	Post successful authentication verification at the AUSF, if an AMF slice serving capability unknown was received from AMF/SEAF in step 2, then the AUSF determines to hold the UE security context until an acknowledgement is received from the initial AMF/SEAF that it is capable to serve the UE. As an alternative option 1, the AUSF can provide security context as in the existing system in addition to sending AMF serving capability check Required Indication and Slice Isolation Required Indication.

Step 8.		The AUSF sends to AMF/SEAF, a Nausf_UEAuthentication_Authenticate Request message which includes authentication result as success, SUPI and an AMF serving capability Check Required indication and Slice Isolation Required Indication (if received from UDM). As an alternative option 1, the Anchor Key is also provided.

Step 9a-b.		The AMF/SEAF on receiving an AMF serving capability Check Required indication and Slice Isolation Required Indication (if received), the AMF determines to check its serving capability and it performs steps 3a-6b (as applicable) of clause 4.2.2.2.3 Registration with AMF re-allocation as specified in TS 23.502. The initial AMF based on local policy and subscription information if decides to reroute via RAN (based on TS 23.502 clause 4.2.2.2.3 step 7B), then performs step 10a-15. Also, the AMF determines not to use the received security context (if received as in alternative option 1). Else if the AMF finds that it is capable to serve the UE (i.e., a reroute via RAN is not needed), and if no security context is received in addition to AMF serving capability check required indication, then the AMF/SEAF can perform step 10a'-10b' to fetch UE security context and then to continue with NAS SMC as in existing system. Else if, the AMF finds that it is capable to serve the UE (i.e., a reroute via RAN is not needed), and if a security context is received as in alternative option 1, then the AMF continue with NAS SMC as in existing system.

Step 10a.	The AMF/SEAF, if determines to perform Reroute via RAN, then it sends SUPI, AMF Serving Capability Result set as '0', AMF reroute security required indication and target AMF information in an AUSF service operation message to the AUSF. As an alternative, in addition the AMF/SEAF can also send the ABBA (i.e., ABBA previous used in authentication message exchange with UE) to AUSF in this step.

Step 10b. The AUSF on receiving an AMF Serving Capability Result set as '0' (as in step 10a) with AMF reroute security required indication, determines that it need to provide the security to new Target AMF. Further as a reroute security context is requested, the AUSF derives an AMF authentication token (AMF_AUTN) using hash of AUSF key, SUPI and RAND. AUSF locally stores the AMF_AUTN along with the SUPI, SUCI, Kausf, ABBA (if received) and Kseaf (if available). The AUSF further sends AMF_AUTN in AUSF service operation message to the AMF/SEAF.

If No Reroute via RAN determined at AMF (then only steps 10a' and 10b' are applicable):

Step 10a'. The AMF if determines that it is capable to serve the UE (i.e., no reroute via RAN), sends SUPI and AMF Serving Capability Result set as '1' to the AUSF.

Step 10b'. The AUSF on receiving SUPI and AMF Serving Capability Result set as '1' determines to provide the UE Security context to AMF/SEAF and then AUSF can send SUPI, Authentication Result and Anchor Key to the AMF/SEAF as in the existing system.

Step 11a.	The initial AMF based on local policy and subscription information if decides to reroute via RAN (based on TS 23.502 clause 4.2.2.2.3 step 7B) at step 9b, it further sends the Reroute NAS message to the NG-RAN, which contains the initial UE message, Routing Information (to select the AUSF instance holding the UE context), and AMF_AUTN. As an alternative, in addition the initial AMF can also send the ngKSI (i.e., ABBA previous used in authentication message exchange with UE) in the Reoute NAS message.

Step 11b.	The NG-RAN forwards the received Reroute NAS message to the Target AMF with initial UE message, reroute due to slicing, AMF_AUTN, ngKSI (if received) and Routing Information.

Step 12.	The Target AMF on receiving the Reroute NAS message with AMF_AUTN will attempt to contact the right AUSF (either directly or via co-located SEAF) based on the routing Information. The Target AMF sends to appropriate AUSF, the Nausf_UEAuthentication_Authenticate Request containing the SUCI, SNN, and the received AMF_AUTN (to authenticate itself with AUSF to fetch the UE security context). The target AMF if received a ngKSI, then that ngKSI can be used to identify the partial native security context created with the successful primary authentication.

Step 13.	The AUSF verifies the received AMF_AUTN based on the UE authentication information locally stored. If the Target AMF provided AMF_AUTN matches with the locally stored AMF_AUTN for a SUCI, then the AUSF considers the AMF_AUTN verification (i.e., Reallocated AMF authentication) as successful and determines to provide the anchor key.

Step 14.	The AUSF sends to SEAF of the Target AMF, the Nausf_UEAuthentication_Authenticate Response containing authentication result, SUPI and Kseaf (the anchor key). Further the SEAF sends the ABBA parameters, authentication result as success, and Kamf key to the Target AMF as in the existing system. As an alternative, if the AUSF inaddition received ABBA in step 11a, the AUSF sends the ABBA received to SEAF and the SEAF is required to use the received ABBA.

NOTE 2: The ABBA parameter usage can be same as in the existing system. TS 33.501 specified the ABBA parameter value as ‘0x0000’. The UE shall use the ABBA parameter provided by the SEAF in the calculation of KAMF as specified in TS 33.501 Clause A.7.1 ABBA parameter values.

NOTE 3: For EAP-AKA', perform step 11 as specified in TS 33.501 clause 6.1.3.1. For 5G AKA, perform step 12 as specified in TS 33.501 clause 6.1.3.2.0.

Step 15.	The Target AMF on receiving the Kamf and authentication result can trigger the NAS Security mode command (NAS SMC) procedure with UE when required to activate the UE NAS security context as in TS 33.501.

Adaptations for Registration Mobility Update:

This section describes the simple adaptations required to address security handling in AMF-reallocation during registration mobility update procedure as shown in Figure 6.6.2-2.

Figure 6.6.2-2: Security context handling during registration mobility update

Step 1. If the Registration Request contains 5G-GUTI (pointing to an old AMF), the initial AMF performs the following accordingly for various scenarios mentioned in Clause 4.3 'Architecture and security assumptions', of this TR,

Case 1-2.a.i) N14 interface exists only between Initial AMF and Old AMF:

Step 2a-b. The Initial AMF based on TS 33.501 Clause 6.9.3, fetches SUPI and security context from the old AMF by providing the 5G-GUTI and the registration request. Further SUPI and subscription information (fetched from UDM as shown in 2b.3) can be used to decide whether to reroute the Registration Request according to TS 23.502 Clause 4.2.2.2.3 step 2. If a reroute is required, the initial AMF, if fetched the UE context (as in TS 23.502 Clause 5.2.2.2.2) from the old AMF, it can use the slice information to perform (2b.4) Nnssf_NSSelection service operation based on 23.502 4.2.2.2.3 step 4a-b. Also, for the case, where the current 5G security context is fetched, the AMF may decipher the NAS container with the same security context, and get the initial NAS message and so if there is any Requested NSSAI provided by UE, it can also be used in Nnssf_NSSelection service operation (even though it is an optional IE according to TS 23.502 Clause 5.2.16.2.1). If an indirect AMF allocation and reroute via RAN is required, the initial AMF does not send any NAS message to the UE (i.e., it does not use current/new 5G security context fetched from old AMF), in turn can perform reroute via RAN.

NOTE 1:	According to TS 33.501 Clause 6.9.3 for the case where source/old AMF sends new 5G security context to Initial AMF, it states, 'The source AMF subsequently deletes the 5G security context which it holds.'

Step 2c.1 If another AMF is selected, the initial AMF sends a reject indication to the old AMF and the old AMF continues as if the Namf_Communication_UEContextTransfer had never been received.

Step 2c.2. Target AMF discovery can be based on 23.502 clause 4.2.2.2.3 steps 6a-b.

Step 3-4. The initial AMF performs reroute via RAN as in 23.502 Clause 4.2.2.2.3 step 7B.

Step 5a-b. The Target AMF on receiving the initial NAS message with 5G-GUTI finds that it is not able to identify the related old AMF and considers that it cannot identify the UE with 5G-GUTI and so initiates identity request procedure with UE and get SUCI.

Step 6. The target AMF initiates primary authentication by sending authentication request with SUCI (without AMF slice serving capability IE, because the target AMF is aware that this is reroute due to slicing received in Reroute NAS message as in TS 23.502 Clause 4.2.2.2.3 step 7B).

The target AMF will not include any AMF slice serving capability IE as unknown, so, the AUSF soon after successful response verification will provide Anchor key to the AMF/SEAF as in existing system. If the UDM provides a Slice Isolation Required Indication during step 5 as in Figure 6.6.2-1, the AUSF also sends the received Slice Isolation Required Indication to the AMF/SEAF along with the authentication result, SUPI and anchor key. Alternatively, if the AUSF does not receive any AMF slice serving capability unknown indication from the AMF, the AUSF can skip sending Slice Isolation Required indication to the AMF (if received from the UDM). The Target AMF checks the SUPI and subscription information and then accordingly performs NAS SMC with the UE.

Case 2-2.a.ii)		No N14 exists between Initial AMF and old AMF and also no N14 exists between target AMF and old AMF: As the initial AMF lack N14 with old AMF, the UE cannot be identified by means of a temporary identity (5G-GUTI) and so the initial AMF performs Subscription identification procedure (as shown in step 2.b-1) with UE and continues with primary authentication (as shown in step 2b.2) with the adaptations described in Figure 6.6.2-1 steps 2a-13.

Case 3-2.b.i)	N14 exists between Initial AMF and old AMF and also between target AMF and old AMF: The initial AMF having N14 with old AMF will act similar to Case 2-2.a.i steps 1-4.

Step 5-6. The Target AMF on receiving the initial NAS message with 5G-GUTI finds it can contact the corresponding old AMF. Based on local policy and reroute due to slicing indication received in reroute NAS message, the target AMF may fetch security context from old AMF (or) determines not to fetch security context from old AMF and perform Identity request/response procedure and primary authentication accordingly (as in case 2.a.i).

If the target AMF fetches security context from old AMF, either based on local policy the target AMF may use current/new security context fetched from old AMF or the target may determine to perform primary authentication with SUPI (as in case 2.a.i, but SUPI is used instead of SUCI).

NOTE 2:	Further the case 2.b.i may not be feasible according to the vertical slice isolation requirement and it may need to be aligned with the Clause 4.3.

Case 4-2.b.ii)	N14 exists only between target AMF and old AMF: As the initial AMF lack N14 with old AMF, the UE cannot be identified by means of a temporary identity (5G-GUTI) and so the AMF performs Subscription identification procedure  (as shown in 2b.1) with UE and continues with primary authentication (as shown in 2b.2) based on the adaptations described in Figure 6.6.2-1 steps 2a-13. The target AMF on receiving the reroute NAS message (i.e., Registration Request with 5G-GUTI), based on local policy, received authentication information (AMF_AUTN) and reroute due to slicing indication, determine to fetch security context related to an authentication information received in the reroute NAS message from the AUSF.

*****End of Change*****