3GPP TSG-SA3 Meeting #103-e 	draft_S3-212050-r1

- e-meeting, 17 - 28 May 2021											Revision of S3-20xxxx

- Source:	Ericsson

- Title:	Update to solution #2

- Document for:	Approval

Agenda Item:	5.7

# 1	Decision/action requested

Approve the pCR to TR 33.854 [1] below.

# 2	References

[1]	3GPP TR 33.854 "Study on security aspects of Unmanned Aerial Systems (UAS)"

[2]	3GPP TS 23.256 “Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2”

# 3	Rationale

This contribution proposes the following updates to solution #2 in TR 33.854 [1]:

- introduction of a revocation mechanism,

- a clarification to one of the procedure steps in clause 6.2.2.2, and

- resolution of the following ENs:

“Editor’s Note: Further evaluation is FFS whether mutual authentication is supported and whether this solution addresses 4th requirement (prevention of unauthorized USS).”

“Editor’s Note: The impacts related to the web-based interface is FFS.”

- The requirement related to authentication and authorization in KI#1 states the following:

The 3GPP system shall enable UAV or networked UAVC authentication and authorisation by the UTM/USS utilising the 3GPP system.

As seen from this statement, there is no requirement related to mutual authentication between UAV/UAVC and USS/UTM. 3GPP is responsible to provide a mechanism to enable authentication and authorization of UAV/UAVC by USS/UTM. The authentication and authorization mechnanism is out of scope of 3GPP. Thus, the existency of such a mutual authentication and the method are out of scope.

- The following texts in the solution addresses the 4th requirement (prevention of unauthorized USS) in KI#1.

If the check is successful, the UAS AF determines the USS/UTM serving the UAV/UAVC based on the USS/UTM identity provided in the request in Step 3 and the predefined list stored in UAS AF with valid USS/UTM identities including URLs to corresponding requests. If the requested identity is not in the list, the request from the UAV will be rejected.

…

If the check is successful, the UAS AF based on the UTM/USS identity, looks up the corresponding UTM/USS URL and triggers a request towards the UTM/USS.

- For the interfaces between UAS AF and USS/UTM, the following is stated in TS 23.256 [2]:

The UAS Network Function is supported by the NEF and used for external exposure of services to the USS. The UAS-NF makes use of existing NEF/SCEF exposure services for UAV authentication/authorization, for UAV flight authorization, for UAV-UAVC pairing authorization, and related revocation; for location reporting, and control of QoS/traffic filtering for C2 communication.

A dedicated NEF may be deployed to provide only the UAS NF functionality, i.e. to support the UAS specific features/APIs and the NEF features/APIs that are specified for capability exposure towards the USS.

# 4	Detailed proposal

*** Start of 1st Change ***

### 6.2.1	Solution overview

This solution addresses the key issue #1. It introduces a new 3GPP Application Function placed in user-plane (UAS AF) which validates that the UAV/networked-UAVC (networked-UAVC is the UAVC connected via 3GPP) has a valid UAV subscription and includes relevant UAV subscription information and UAV application information to be sent to the USS/UTM to support the USS/UTM for the authentication and authorization of the UAV/networked-UAVC. Throughout this key issue, unless otherwise specified, “UAVC” is used for “networked-UAVC”.

This solution assumes that each UAV or UAVC is provisioned with a PLMN UE ID and the corresponding credentials to be used in primary authentication by the PLMN as a normal UE. Also, the UEs are provisioned with a CAA level ID and corresponding credentials to be used in UAS authentication and authorization (UAA) by USS/UTM. The credentials used in UAS A&A and A&A method are out of 3GPP scope.

Also, this solution includes an authorization revocation mechanism that allows the authorization revocation triggered by USS/UTM.

*** End of 1st Change ***

*** Start of 2nd Change ***

#### 6.2.2.2	Procedure for EPS

Figure 6.2.2.Y-1: UAV Authentication and Authorization procedure for EPS

1.	Attach and a PDN connection activation procedures are performed. As the UAV has a subscription indicating UAV capability the PCRF generates a PCC rule indicating UAV application and header enrichment policy.

2.	UAV sends UAS A&A request to the UAS AF.

3.	PGW detects the UAV application traffic and adds UAV subscription information.

NOTE:  In case of end-to-end security between UAV and UAS AF, to enable the PGW to add UAV subscription information, a HTTP(S) proxy functionality in the PGW is integrated under the assumption that TLS is used for end-to-end security. The certificate of the UAS-AF needs to be provisioned to the PGW. The PGW terminates TLS for HTTPS-requests towards to the UAS-AF and then apply header-enrichment.

4.	UAS AF checks if the UAV has a valid aerial subscription based on the subscription information received from the PGW.

If the check is successful, the UAS AF based on the UTM/USS identity, looks up the corresponding UTM/USS URL and triggers a request towards the UTM/USS. If the check is unsuccessful a response is sent to the UAV rejecting the request. The UAS AF can include information to the UTM/USS needed for further interaction between UTM/USS using network APIs and EPS regarding the PDU session.

5.	UTM/USS performs authentication and authorization steps which are transparent to the network. In this communication, the UAS AF and PGW relay the traffic between UAV and UTM/USS.

6.	If the A&A in step 5 is successful, UTM/USS triggers a accept response to UAS AF acknowledging the request including an application specific information such as a security context to be used in the establishment of secure connection between UAV and UTM/USS. The transfer of this information is transparent to the network and the content of it is out of 3GPP scope.

If the A&A is un-successful, a response is sent to the UAV rejecting the request.

7.	The UAS AF sends the response to the UAV.

8.	UAV triggers a set-up of a secure connection to UTM/USS e.g. using the information received in step 7.

*** End of 2nd Change ***

*** Start of 3rd Change ***

#### 6.2.2.X	Authorization revocation mechanism

When the USS/UTM wants to trigger authorization revocation, it calls the NEF API (AsSessionWithQoS) used to activate, modify, and revoke policies for specific data flows on a specific PDU-session/PDN-connection. UAS AF provides related information needed by the USS/UTM to call that API, during the authentication and authorization procedure.

During the authentication and authorization procedure, UAS AF provides the UE-IP of the UAV to the USS/UTM. After successful authentication and authorization by the USS/UTM, the USS/UTM calls AsSessionWithQoS to activate the authorized policies for the PDU-session and then waits for a notification on that they have been successfully enforced. Then the USS/UTM sends the A&A result back to the UAS-AF which will forward the result to the UAV.

*** End of 3rd Change ***

*** Start 4th Change ***

### 6.2.3	Solution evaluation

This solution fully addresses key issue #1 having no effect on the current NAS procedures and UAV UE proposing a UP solution.

This solution provides a user plane solution while TR 23.754 [3] concludes on using control plane based UUAA procedures.

This solution requires a new function (UAS AF) that requires updates on AMF and PCF for the subscription information requests.

It uses web-based interface for the communication between UAS AF and USS/ UTM.

*** End 4th Change ***