3GPP TSG-SA3 Meeting #103-e 	S3-211415

# e-meeting, 17 - 28 May 2021

Source:	3GPP SA2

# Title:	Reply LS on Prevention of attacks on sliced core network

# Document for:	Information, Discussion

Agenda Item:  4.19

3GPP TSG SA WG 2 Meeting 144e 		TDoc S2-2103213

Online, April 12 - 16 2021

Title:	Reply LS on Prevention of attacks on sliced core network

Response to:	LS S2-2102954 from GSMA FSAG

Release:	N/A

Work Item:	N/A

Source:	SA2

To:	GSMA FSAG

Cc:	CT4, SA3

Contact person:	Alessio Casati

Alessio.casati@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

# SA2 thanks GSMA FSAG for their LS on "Prevention of attacks on sliced core network". SA2 relies on the security expertise of 3GPP SA3 for the analysis of the issue in your LS. However, for the following aspects in your LS SA2 would like to provide the following answers:

Is it allowed to have two slices with the same SST (the SD is after all optional and not a conditional element)?

SA2 answer: An S-NSSAI identifies uniquely a network slice within a network. If a network slice is identified by an S-NSSAI consisting only of a value in the SST field and no SD field, there is only one network slice in the PLMN identified by such S-NSSAI. If network slices are identified by both the SST and the SD fields, such network slices can share the same SST value but must have a different SD value. An S-NSSAI consisting only of an SST field value (and no SD field) and an S-NSSAI consisting of the same SST field value but also including an SD field value identify different network slices.

Is every slice-id which has a SD automatically non-standard? Even if it has a standard SST value?

SA2 answer: A standardized S-NSSAI is comprised only of a standardised SST value. The Standardised SST values are defined in clause 5.15 of TS 23.501. The values of SST 0-127 are reserved for standardization. all other SST values are operator specific. Any S-NSSAI with a SD field is operator-specific even if the SST field is one of the standardised values.

# 2	Actions

To GSMA FSAG

ACTION: 	Please take the above information into account

# 3	Dates of next TSG SA WG 2 meetings

the upcoming SA2 meetings can be found in the SA2 Meetings calendar