# 3GPP TSG SA WG3 (Security) Meeting #103-e	S3-211464

# 17-28 May 2021, Online

# Source:	Johns Hopkins University APL, US National Security Agency, CISA ECD, InterDigital, Fraunhofer SIT, CableLabs, AT&T, Verizon Wireless, NIST

# Title:	Solution on Remote Attestation for 5GS

# Document for:	Approval

# Agenda Item:	5.16

# 1	Decision/action requested

It is requested to approve this contribution for a new solution in TR 33.866.

# 2	References

[1] H. Birkholz, D. Thaler, M. Richardson, N. Smith, and W. Pan, “Remote Attestation Procedures Architecture,” draft-ietf-rats-architecture-12, April 2021,  https://datatracker.ietf.org/doc/html/draft-ietf-rats-architecture-12

# 3	Rationale

Remote attestation provides the ability to interrogate a remote system for evidence of its trustworthiness to ensure that the system is doing what it is intended to and nothing else, e.g., not producing corrupt data. Evidence can be based on identity or other characteristics of the system (e.g., hardware components, software components, configuration, etc.). Data collection frameworks, i.e., the NWDAF (TS 23.288) in the 5GC, the MDAS (TR 28.809) on OAM, and the DCCF (clause 6.9, TR 23.700-91) can use attestation results about data sources to mitigate the threat of tampered data in AI/ML analytics. Remote attestation is particularly appropriate for collecting data from NFs because the SBA spins new NFs up and takes older NFs down as needed.  This produces a fresh set of evidence that can be used in determining the trustworthiness of the NFs.

Key Issue #1.2: Successful development and usage of AI/ML analytics requires data that can be trusted. Fundamentally, data produced by an untrustworthy component cannot be trusted.  Developing AI/ML analytics requires training data. For greatest effect, training data should be free from malicious manipulation, otherwise corrupt data will become normalized, and misrepresentative of the specific 5GS. Not having training data that accurately represents the 5GS could lead to analytics that flags normal operations as anomalies and vice-versa. Furthermore, tampering with input data during the inference phase of analytical operations can evade abnormal behavior detection, degrade performance management, or disclose information about proprietary algorithms.

Key Issue # 2.2: Implementing remote attestation within the 5GS is not just limited to identifying the trustworthiness of a data source for analytics. The attestation results can also be used as an analytics data source for identifying anomalous NF behavior. Remote attestation can be used for verifying the tampering of a NF including the platform that is running the NF.

# 4	Detailed proposal

****Start of 1st Change****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 23.700-91: "Study on enablers for network automation for the 5G System (5GS); Phase 2".

[2]	3GPP TS 33.867: "Study on user consent for 3GPP services".

[3]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[4]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services ".

[5]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[6]	Draft NISTIR 8269: "A Taxonomy and Terminology of Adversarial Machine Learning"; https://doi.org/10.6028/NIST.IR.8269-draft

[7]		ETSI SAI: "AI Threat Ontology"; https://docbox.etsi.org/ISG/SAI/70-DRAFT/001/SAI-001v008.docx.

[8]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[9]	3GPP TR 28.809: "Study on enhancement of management data analytics".

[yy]	H. Birkholz, D. Thaler, M. Richardson, N. Smith, and W. Pan, “Remote Attestation Procedures Architecture,” draft-ietf-rats-architecture-12, April 2021, https://datatracker.ietf.org/doc/html/draft-ietf-rats-architecture-12

****End of 1st Change****

****Start of 2nd Change****

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of solutions to key issues

| Solutions | Key Issues |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 Key issues related to securing the data provided to any type of analytics function |  |  |  |  |  | 2 Key issues related to detection of cyber-attacks and anomaly events by analytics function |  |  | 3 Key issues related to data transfer protection |  |  |
|  | 1.1 | 1.2 | 1.3 | 1.4 | 1.5 | 1.X | 2.1 | 2.2 | 2.Y | 3.1 | 3.2 | 3.Z |
| #1: UE data collection protection |  |  |  |  | X |  |  |  |  |  |  |  |
| #2: Network Analysis Framework for DDoS Attack |  |  |  |  |  |  | X |  |  |  |  |  |
| #3: Usage of current SBA mechanisms to protect data in transit |  |  |  |  |  |  |  |  |  |  | X |  |
| #4: DCCF determining if NF Service consumer is authorized to invoke a service to a Data Producer NF for data collection |  |  | X |  |  |  |  |  |  |  |  |  |
| #5: Providing the security of data via Messaging Framework |  |  |  | X |  |  |  |  |  |  |  |  |
| #6: Integrity protection of data transferred between AF and NWDAF | X |  |  |  |  |  |  |  |  |  |  |  |
| #7: Detection of anomalous NF behaviour by NWDAF |  |  |  |  |  |  |  | X |  |  |  |  |
| #8: Privacy preservation of transmitted data |  |  |  |  |  |  |  |  |  | X |  |  |
| #Y: Remote Attestation for 5GS |  | X |  |  |  |  |  | X |  |  |  |  |

****End of 2nd Change****

****Start of 3rd Change (All Text Are New)****

### 6.Y	Solution #Y: Remote Attestation for 5GS

### 6.Y.1	Introduction

This solution addresses Key Issue # 1.2 “Processing of tampered data” by protecting a 5GS analytics function from processing unsanitized or tampered data by using remote attestation. Remote attestation will allow the NWDAF to know if a 5G System (5GS) component is in a proper operational state before accepting data from it. This solution also partially addresses Key Issue 2.2 “Anomalous NF behaviour detection by NWDAF” by providing remote attestation results as additional data points for detecting anomalous NF behaviour.

6.Y.2	Solution details

Remote attestation provides cryptographic evidence of the integrity of the firmware, software, composition, and configuration of a device. The integrity of a device asserts the integrity of the data produced by the device. Typically, this evidence includes trusted integrity assertions, such as signed measurements and other  statements about system characteristics and events.  Conceptually, this evidence provides proof of a trusted boot cycle that starts with a root of trust and extends to the operational environments. Remote attestation also includes proof that the evidence comes from the intended device; this is often done with a permanent cryptographic identifier.

6.Y.2.1 Remote Attestation Description

This solution proposes implementing remote attestation in 5GS components including, but not limited to, UEs, gNBs, Virtual Machines hosting a NF, Containers hosting a NF, and physical servers hosting a NF. This solution requires three new roles: Attester[yy], Verifier[yy], and Relying Party[yy].  The Attester is implemented on each desired 5GS component that produces data for the NWDAF . The Attester performs integrity measurements from a trusted environment and securely stores those measurements so they cannot be subverted. Attestation measurements are sent as authenticated evidence to a Verifier and compared to reference values to determine the level of confidence in the trustworthiness of the component. Based on the attestation results generated by appraisal procedures via comparison, the Relying Party, i.e., the protected 5GS component, can decide to consume and process data from the attested component. The reference values of the integrity measurements are changed along with attested component updates so that remote attestation is robust to the typical component updates.  It is worth noting that remote attestation does not prevent exploitation of a component, but it does detect the exploitation through the integrity measurements and allows the Relying Party to protect itself from the impacts. The role of the Verifier and Relying Party may coexist on the same system. Diagram 1 illustrates the three roles and the flow of information in a remote attestation architecture.

It is recommended that the NWDAF host the role of Verifier for remote attestation since it is already designed to consume and produce data from and to each 5GS component.

Diagram 1

Editor’s Note: Details on how to integrate the Verifier role for remote attestation into NWDAF are F.F.S.

6.Y.2.1.1 Security Procedures for 5G Components

The 5G components should adhere to the following procedures:

- Measurements should be sent from the Attester to the Verifier in a manner that ensures freshness, confidentiality, integrity, and authenticity.

- The Attester should run in a trusted environment with complete access to the system so that it can take accurate and comprehensive measurements even when measuring a corrupted component.

- The Attester should run on component boot up.

- Measurements on a component should be stored in a secure manner to prevent unauthorized modification prior to being sent to the Verifier.

- The Verifier should run in a trusted operating environment.

- Attestation results should be sent from the Verifier to the Relying Party in a manner that ensures freshness, confidentiality, integrity, and authenticity.

6.Y.2.2 Base Stations

Base stations are implemented on either purpose-built hardware or commodity hardware (e.g., O-RAN architecture). During the boot up of purpose-built hardware, the Attester application will take measurements of the hardware and firmware on disk.

During the boot up of commodity hardware, the Attester application will take measurements of the hardware, boot/bios image, Operating System (OS) image on storage hardware, and base station application on storage hardware.

6.Y.2.2.1 Security Procedures for Base Stations

Editor’s Note: Details on the specifics of measurements for base stations are F.F.S.

The underlying hardware platform provides a trusted environment within which the Attester application runs.

6.Y.2.3 Servers Hosting Network Functions (NFs)

During boot up, the Attester application will take measurements of the hardware, boot/bios, OS image on disk, and Network Function (NF) software on disk.

6.Y.2.3.1 Security Procedures for Servers Hosting NFs

Editor’s Note: Details on the specifics of measurements for servers are F.F.S.

The server’s hardware platform provides a trusted environment within which the Attester application runs.

6.Y.2.4 Virtual Machines Hosting NFs

Upon start-up of the virtual platform, the Attester will take measurements of the hardware and hypervisor software. After successful attestation of the hardware and the hypervisor, attestation will be performed against the guest OS and NF software. This attestation process is called layered attestation and will occur for every instance of a NF that is started up.

6.Y.2.4.1 Security Procedures for Virtual Machines Hosting NFs

Editor’s Note: Details on the specifics of measurements for virtual machine platforms are F.F.S.

The virtual machine hardware platform provides a trust environment within which the Attester application runs.

The hypervisor software provides a virtual trust environment within which the Attester application runs.

6.Y.2.5 Containers Hosting NFs

Upon start up, the Attester will take measurements of the hardware, OS, and container software. Once the hardware platform, OS, and container software has been attested, the container software can then follow its own method of attestation. By design, container software runs an image of the process it is hosting. For these purposes, the process is a NF. Upon creation each image is signed. Prior to running the image, the Attester application will verify the signature.

6.Y.2.5.1 Security Procedures for Containers Hosting NFs

Editor’s Note: Details on the specifics of measurements for container platforms are F.F.S.

Editor’s Note: Cryptographic details on the container image signing process are F.F.S.

The hardware platform provides a trust environment within which the Attester application runs.

The container software supports remote attestation.

6.Y.2.6 UEs

UEs contain a baseband processor for the sole purpose of communicating with the RAN and an application processor for user or product applications. For the purposes of Quality of Service (QoS), RAN security, and control plane security the focus is on the baseband processor. The application processor and OS is the source for user plane security. Upon boot up the Attester application will take measurements of the hardware, baseband firmware, boot loader, and OS on disk.

6.Y.2.6.1 Security Procedures for UEs

Editor’s Note: Details on the specifics of measurements for UEs are F.F.S.

The Baseband provide a trusted environment within which the Attester application runs.

6.Y.3	Evaluation

Editor’s Note: To be added later

****End of 3rd Change****