# 3GPP TSG SA WG3 (Security) Meeting #103-e	S3-211466

# 17-28 May 2021, Online

# Source:	InterDigital

# Title:	Changes to TR 33.866

# Document for:	Approval

# Agenda Item:	5.16

# 1	Decision/action requested

It is requested to approve this contribution proposing changes to TR 33.866.

# 2	References

[1] TR 33.866 v040

# 3	Rationale

This pCR proposes editorial changes for Clauses 4-5 of TR 33.866.

# 4	Detailed proposal

# ****Start of Changes****

# 4	Overview of eNA

Editor's Note: This clause will contain a brief overview on eNA based on SA2's study (TR 23.700-91), including architectural assumptions, etc.

3GPP TS 23.288 [4] provides the Stage 2 architecture enhancements for 5G System (5GS) to support network data analytics services in 5G Core network, which forms the baseline for the present study on security aspects of enablers for Network Automation (eNA) for the 5G system (5GS).

The Network Data Analytics Function (NWDAF) as specified in 3GPP TS 23.501 [5] interacts with different entities within 5GS for data collection based on subscription to events, retrieval of information from data repositories, retrieval of information about NFs (e.g. from NRF for NF-related information) and on demand provision of analytics to consumers. The NWDAF provides analytics to 5GC NFs and OAM. Analytics information is either statistical information of the past events or predictive information.

3GPP TR 23.700-91 [1] is an architectural study on enhancements for analytics and NWDAF, for which any security impact will be documented in the present document. There is a particular security impact for UE data collection protection, detection of cyber-attacks and anomaly events supported by NWDAF and its related functions, on the protection of data transfer in inter-NWDAF/NWDAF cases.

# 5	Key issues

Editor's Note: This clause contains all the key issues identified during the study.

## 5.1	Key issues related to securing the data provided to any type of analytics function

Editor's Note: This clause is for key issues on UE data collection protection to fulfil the NWDAF functionalities including privacy consideration, data authenticity, data integrity, accessibility aspects requirements, according to the first objective of the SID.

### 5.1.1	Key Issue #1.1:Integrity protection of data transferred between AF and NWDAF

#### 5.1.1.1	Key issue details

The 5GS supports the collection and utilisation of the UE data and delivering it to the NWDAF as an input to generate the analytic information (to be consumed by other NF).

As per KI#8 in TR 23.700-91 [1], there is no direct interface between the UE and the NWDAF. When AF is used for the communication between the NWDAF and UE for data collection, there is a need to study the security aspects for the data provided by the UE to NWDAF via AF and vice versa.

This key issue studies the integrity aspects on data collection and utilization of UE data in order to derive the analytics.

#### 5.1.1.2	Security Threats

If the data shared between  AF and NWDAF is not secured, it may lead to the following issue;

Data can be modified and replayed by unauthorized parties.

#### 5.1.1.3	Potential Requirements

Integrity and replay protection shall be supported on the interface between AF and NWDAF for the UE data collection.

### 5.1.2	Key Issue #1.2: Processing of tampered data

#### 5.1.2.1	Key issue details

5GS is expected to use ML. Two centralized frameworks currently responsible for ML-based analytics, e.g., abnormal behavior analytics, are NWDAF (TS 23.288 [4]) in 5GC and MDAS (TR 28.809 [9]) on OAM. Furthermore, decentralized AI/ML is used in several use cases, such as efficiency optimization in RAN. In addition, a new data collection framework DCCF (clause 6.9, TR 23.700-91 [1]) is proposed for Rel-17.

Network data analytics is including the following steps:

- Request of analytics by consumer

- Collection of data by analytics function

- Processing of collected data by analytics function

- Reply containing analytics output to a consumer by analytics function

While 3GPP provides sound security on the network level, the data used by AI/ML is not being subject to security controls. This key issue seeks solutions to remedy several attacks against a 5GS involving tampered data.

#### 5.1.2.2	Security threats

Editor's Note: Threats need to be revisited if in line with NIST 8269 [6] and ETSI SAI [7] terminology.

Data used by AI/ML is not being subject to security controls. This allows for a number of attacks against a 5GS with severe impact on performance up to denial of service (DoS) conditions:

- Adversarial examples are generated by slightly perturbating input data. The data is perturbated in a space in which AI/ML algorithms are sensitive to change, leading to severe performance degradation and misclassifications in the inference process. This attack is well-known in human-centric use cases, such as image/audio classification.

- During training, tampered training data can lead to model skewing. Skewed models will provide false results in inference.

- Tampered data may also lead to the information disclosure of confidential/proprietary AI/ML algorithms by inference.

- In more simple attacks, perturbations may not be slight (as those generated by adversarial example methods). In non-human-centric use cases (as most are in 5GS), the perturbations may just be false data to force misinterpretation.

Unprotected analytic functions are subject to:

- Decreased efficiency, e.g. power consumption, load balancing, QoS optimization

- System failure (DoS scenario)

- Inference of confidential ML algorithms employed by 5GS

- Leakage of privacy-related data derived from AI/ML models

#### 5.1.2.3	Potential security requirements

A 5GS analytics function shall be protected from processing unsanitized or tampered data.

### 5.1.3	Key Issue #1.3: Authorization of NF Service Consumers for data access via DCCF

#### 5.1.3.1	Key issue details

A Data Collection Coordination Function (DCCF) is used to coordinate the collection of data from one or more NF(s) based on data collection requests from one or more Consumer NF(s). DCCF and Data Repository Function (DRF) can be standalone NFs, possibly co-located with NWDAF, or can be hosted by NWDAF. Data Collection notification to one or more Consumer NF(s) may be supported via a Messaging Framework. Adaptors supporting 3GPP services allow NFs to interact with the Messaging Framework. Only the interface between 3GPP entities and the adaptors is in 3GPP scope. This includes 3GPP services offered by adaptors to allow NFs to interact with the Messaging Framework.

TR 23.700-91-100 conclusion mentions that "Additional authorization for Consumers to access data from a Data Source via the DCCF and to access data from DRF (directly or via DCCF) needs to be coordinated with SA3". According to SA2 KI#11 conclusions, if a consumer subscribes to analytics notifications to the DCCF, the DCCF can subscribe itself to the data source and notify the data source that notifications are to be sent directly to the consumer. The data source will then send notifications to the consumer via the MF or the DCCF.

This key issue addresses the authorization of data consumer to access the data (via DCCF) from the data source or the DRF, and the authorization aspects of the DCCF being allowed to subscribe the data on behalf of the consumer at the data source or the DRF, i.e. the security aspect on usage subscription/notification mechanisms for a consumer to receive notifications on a different path (as adapted in SA2 conclusions) will be studied.

#### 5.1.3.2	Security threats

DCCF introduces a new path for an NF Service Consumer (NFc) to access the data from data sources or an NF Service Producer (NFp). Due to the introduction of DCCF between consumer and producer, the existing security mechanism will not be sufficient, and the following threats need to be addressed:

An unauthorized data consumer could request service from the DCCF.

NOTE: This threat can be addressed by existing SBA mechanisms. Therefore, no related requirement is added.

Based on a request from a DCCF, the Messaging Framework may provide data from a producer to a requesting data consumer, even though the consumer is not authorized to receive this data.

Based on a request from a DCCF data received from a data producer is stored in the DRF. When the data are later retrieved, the DCCF may provide the stored data to a non-authorized consumer if requested.

A DCCF could subscribe for data from the data source on behalf of the data consumer without the data consumer authorizing DCCF to do so.

The data producer may be unable to correctly verify the identity of the data consumer since the data request is coming from DCCF on behalf of the consumer.

#### 5.1.3.3	Potential security requirements

The data consumer shall be authorized to access the data from the data source.

Authorization of the DCCF shall be supported to access a service of a data source on behalf of a data consumer.

Editor’s Note: Whether the service request including the URI, service name, etc., needs to be verified by the data source is FFS.

### 5.1.4	Key Issue #1.4: Security protection of data via Messaging Framework

#### 5.1.4.1	Key issue details

In [1], in the conclusions for the Key Issue #11 ‘Increasing efficiency of data collection’, DCCF (Data Collection Coordination Function) is defined for efficient data collection in 5GS. The DCCF is a control-plane function that coordinates data collection and triggers data delivery to Data Consumers.

TR 23.700-91 [1] lists several agreed principles for normative work, some of which are as follows:

-	"When a Data Collection Coordination Functionality (DCCF) is deployed, it is used to coordinate collection of data from one or more NF(s) based on data collection requests from one or more Consumer NF(s)."

-	"Data Collection notification to one or more Consumer NF(s) may be supported via a Messaging Framework. Adaptors supporting 3GPP services allow NFs to interact with the Messaging Framework.

-	Data Collection notifications may also be supported directly from the data provider to one or more Consumer NF(s), or via DCCF and not via the Messaging Framework.

NOTE 2:	The Messaging Framework is outside the scope of 3GPP. Adaptors are not expected to be standardized by 3GPP, only the interface between 3GPP entities and the adaptors is in 3GPP scope. This includes 3GPP services offered by adaptors to allow NFs to interact with the Messaging Framework."

-	"The DCCF coordinates data collection so the same data is not requested multiple times from the same data source."

The Messaging Framework is not expected to be standardized by 3GPP. It contains Messaging Infrastructure that propagates event information and data (e.g.: streaming and notifications) from data sources to data consumers. A Data consumer that uses the Data Management Framework sends requests to the DCCF rather than to a data producer.

Data notifications from data sources go through the adaptor on the producer side, 3PA, the Messaging framework, and the adaptor on the consumer side, 3CA. The adaptor 3PA recognizes notifications from a Data Source and delivers them to the Messaging Framework. The adaptor 3CA obtains data from the Messaging Framework and sends notifications to the Data Consumer. 3CA can condense multiple notifications into a single notification.

In TR 23.700-91 [1], Figure 6.9.3-1: "Data Collection & Distribution for Event Notifications (Subscribe/Notify)" shows how the data can be transferred via Messaging Framework from data sources to data consumer.

Since the Messaging Framework is not expected to be standardized by 3GPP, it may not be trusted.

As concluded in clause 8 in TR 23.900-71 [1], Data Collection Coordination Function (DCCF) and Data Repository Function (DRF) and the related interfaces (interfaces between 3GPP entities and the adaptors) are to be standardized.

The DCCF and the Messaging Framework decouple the data collection between the data consumer and the data source; however, this may induce a security problem because the data consumer cannot verify that the data from the data source is not modified by the Messaging Framework and the confidentiality of the data cannot be guaranteed by the Messaging Framework.

#### 5.1.4.2	Threats

An attacker may eavesdrop or manipulate or replay the communication or initiate the MitM attacks on the interface.

If the integrity of the data collected from the data source is not protected, then the Messaging Framework may modify the data, which results in producing wrong analytics.

If the confidentiality of the data collected from the data source is not protected, then the Messaging Framework may access the sensitive data, which may cause privacy leakage.

Replay attacks may lead to the utilization of the same data more than once, and therefore, it may cause wrong analytic results.

#### 5.1.4.3		Potential security requirements

The transfer of the data between the data source and data consumer via the messaging framework shall be confidentiality, integrity, and replay protected end-to-end between the data source and data consumer.

Confidentiality protection, integrity protection, and replay-protection shall be supported on the new interfaces between 3GPP entities and the adaptors.

Editor's Note: It is ffs if this requirement can be fulfilled, since the task of adaptors (e.g. 3CA) is to process and format data before the data is sent as notification to the data consumer.

Editor's Note: Current understanding is that adaptors are not expected to be standardized by 3GPP. To be checked with SA2.

### 5.1.5	Key Issue #1.5: UE data collection protection at NF/NWDAF

#### 5.1.5.1	Key issue details

UEs register to 5GS and request services, e.g. the initial registration request to AMF.  For fulfilling the service, but also for analytics purposes, 5GS NFs will collect data about the UE being served, e.g. AMF needs to maintain a mapping between SUPI and 5G-GUTI and for accounting the time window for the service used. UE-related data, processed by one NF, may also need to be transferred to another NF to fulfill a service request or for analytics purposes. UE can also provide privacy-sensitive data such as positioning information, user profiling info, etc to NFs, which may be transferred to NWDAF.

This KI is about NF/NWDAF collecting information about the UEs (e.g., UE mobility events, UE registration failures) from the 5G NFs (e.g., AMF, 5G RAN) and determines the threats and requirements for the protection of data related to UE, which are collected by core NFs.

#### 5.1.5.2	Security threats

If the communication between UE and network is not confidentiality protected, then sensitive information about UEs may be leaked to unauthorized entities.

If the integrity of the data collected from UE is not protected, the analytics may not be accurate.

Replay attacks may lead to the utilization of the same UE data more than once, and therefore, it may cause wrong analytic results.

UE-related data stored in an NF or transferred between different NFs may be altered by a malicious entity. The attacker may provide false or modified information to other NFs or an analytics function such as NWDAF. For instance, the malicious entity can modify the UE information statistics or logs sent to the NWDAF.

In case of the network is not authenticated by the UE, the UE may send UE-related data to an unauthorized entity, which may lead to leakage of sensitive data from the UE.

If an unauthenticated UE is sending data, it may send erroneous data to NF/NWDAF. This can compromise the efficiency, performance, and output of analytics algorithms implemented in the analytics functions. If the NF/NWDAF which is receiving UE data is not properly authenticated and authorized, the sender may transfer the UE-related data to an unauthorized NF or analytics function.

#### 5.1.5.3		Potential security requirements

1. UE and network shall mutually authenticate each other.

2. The communication between UE and the network shall be confidentiality protected.

3. The data collected from UE shall be integrity protected.

4. Data transferred from UE to NFs and from NFs to the analytics function shall be protected against replay attacks.

5. Authorization of NFs and analytics functions to receive, send, or transfer UE-related data shall be assured.

## 5.2	Key issues related to the detection of cyber-attacks and anomaly events by analytics function

Editor's Note: This clause is for key issues on detection of cyber-attacks and anomaly events supported by NWDAF and its related functions, specifically to define parameters provided by UE to help detect attacks and abnormal behaviours, according to the second objective of the SID.

### 5.2.1	Key Issue #2.1: Cyber-attacks detection supported by NWDAF

#### 5.2.1.1	Key issue details

NWDAF has been defined to offer automatic network analytics and alarming, with possible capabilities of artificial intelligence and machine learning to help proactively manage the 5G network. 3GPP TR 23.700-91[1] has identified the use case of NWDAF detecting cyber-attacks by monitoring events and data packets in the UE and the network, with the support of machine-learning algorithms. To achieve cyber-attacks detection, the NWDAF can collaborate with UE and any other NFs to collect related data as inputs and  providing alerts of anomaly events as outputs to OAM and other NFs which have subscribed to them so that they could take proper actions.

This key issue describes what kind of cyber-attacks can be detected by NWDAF. In order to mitigate the identified cyber-attacks, the data/parameters collected by NWDAF need to be studied.

The specific cyber-attacks for which an analytics function may provide detection support include but are not limited to the following examples:

(1) MitM attacks on the radio interface: MitM attacks or fraudulent relay nodes may modify or change messages between the UE and the RAN, resulting in failures of higher layer protocols such as NAS or the primary authentication. The NWDAF may detect MitM attacks.

(2) DoS attacks: 5G has high performance requirements for system capacity and data rate, improved capacity and higher data rate may lead to much higher processing capability cost for network entities, which may make some network entities (e.g. RAN, Core Network Entities) to suffer from DDoS attack. The NWDAF may also enable the detection of DDoS attacks.

#### 5.2.1.2	Security threats

Cyber-attack may not be detected by the 5G network, thus further attacks could be conducted.

Anomaly events may not be detected by the 5G network, thus further attacks could be conducted.

#### 5.2.1.3	Potential security requirements

The 5GS system shall support the detection of cyber-attacks by providing related inputs or collecting output analytics using an analytics function such as NWDAF.

Editor's Notes: The requirement may be updated according to SA2's feedback.

### 5.2.2	Key Issue #2.2: Anomalous NF behaviour detection by NWDAF

#### 5.2.2.1	Key issue details

The 5GC supports different NF deployments that could be in distributed or redundant fashion so that the NF provides the services from several locations and several execution instances. When these NFs are distributed across diverse cloud infrastructure, it is possible that the NFs may behave in an undefined manner. The undefined behaviour of the NF may be caused by internal errors such as configuration mistakes or internal data corruption. This misbehaviour may impact one or more UE services based on the type of NF. Thus, the correlation of which NF is handling which UE data is an important aspect, such that NWDF is enabled to conclude from the reported UE-related data, which NF may have anomalous behaviour.

In all such instances, it is imperative that an analytics function such as NWDAF monitors the behaviour of all the NFs and ensures that the NFs behave as defined. If the NFs behave erroneously, it should be possible to detect the anomaly so that appropriate steps can be taken, e.g. by an operator, to control the potentially damaging behaviour.

Note, it is up to the operator to define the details of what NFs should report if such monitoring and detection by NWDAF is desired. However, there is a need to enable NWDAFs to receive or request reports by NFs which serve the detection of anomalous NF behaviour.

#### 5.2.2.2	Security threats

Different NFs may behave in an undefined manner. Anomalous or malicious NF behaviour could be, for instance, include attempts to access NF/NF service which was not authorized to a NF as NF/NF service consumer, to unusually high consumption of network or compute resources for NF as either NF/NF service consumer or producer, to triggering of DoS attack on NF service producer by continuously sending malicious messages, e.g. ill http request, etc., attempts to exhaust connections of http server.

The above conditions can happen either due to internal data corruption, configuration errors, or cross communication between NFs from different vendors. Based on the NF type, such NFs could cause damage to either one or multiple UEs. For example, in the case of an AMF or SMF dedicated to a network slice, the service for all UEs within the whole network slice could be affected. Even the whole network slice could get out of service.

An erroneous NF may succeed in knocking the whole network out of service by sending wrong messages to other NFs, causing other NFs to get out of service.

The NFs within the 5GC are already authenticated and allowed to communicate with each other based on successful authentication and authorization. If the NF is misconfigured or has internal data corruption, etc, the assumption of trust becomes invalid and causes potential threats.

#### 5.2.2.3	Potential security requirements

It should be possible for the network to detect anomalous NFs using the data collected from UE and NFs.

NOTE: By this requirement, it is only assured that specific data can be collected by and/or reported to an analytics function. Which AI/ML is used is implementation-specific and out of scope in 3GPP.

## 5.3	Key issues related to data transfer protection

Editor's Note: This clause is for key issues on protection of data transferring (e.g. privacy consideration) in the inter-NWDAF/NWDAF instances, according to the third objective of the SID.

### 5.3.1	Key Issue #3.1: Privacy preservation for transmitted data between multiple NWDAF instances

#### 5.3.1.1	Key issue details

In the case of Multiple NWDAF Instances, during the transfer of data/metadata/analytics output, it needs to be ensured that the privacy of the user is preserved.

It needs to be ensured that appropriate measures are taken by the sender NWDAF to protect any information which can reveal the privacy of the user, such as positioning information, user profile information, etc., before sending privacy-related data to another NWDAF instance. Privacy-related information that has been allowed by the User for analysis should not be transferred without sufficient protection mechanism.

#### 5.3.1.2	Security threats

Information that can reveal the identity of the user can compromise privacy when transmitted unprotected.

The transfer of personally identifiable information without adequate protection measures constitutes a threat against user privacy and possibly violates regulations on data protection.

Editor's Note: Description of the attacker model is FFS.

#### 5.3.1.3	Potential security requirements

Any information which can reveal the identity of the user, such as positioning information, user profile information, etc, should be securely protected before data is being shared or transferred to other NWDAF Instances.

### 5.3.2	Key Issue #3.2: Protection of UE data in transit

#### 5.3.2.1	Key issue details

The UE is providing the core network functions with data, which are reported to or requested by analytics functions. The transfer of any data between core network functions needs to be protected.

According to TS 23.288 [y] the NWDAF collects data from various data sources and provides Analytics Output to different NWDAF data consumers. In addition, according to the solutions for KI#2 "Multiple NWDAF Instances" proposed in TR 23.700-91 [x] the analytics data or the analytics output can be transferred from one NWDAF instance to another NWDAF instance.

Data in transit needs to be protected while in transfer between NWDAFs, NF to NWDAFs, and NWDAF to another entity, e.g. DCCF.

This key issue addresses security for data in transit involving an analytics function.

#### 5.3.2.2	Security Threats

If data is transferred between NFs or different NWDAF Instances, a MitM (for instance a malicious SCP) can compromise data by eavesdropping or modification.

A rogue NWDAF Instance can send wrong or modified data to another NWDAF instance.

#### 5.3.2.3	Potential security requirements

Data transferred between core network functions shall be integrity, confidentiality, and replay protected.

### 5.3.3	Key Issue #3.3: Ensuring restrictive transfer of ML models between authorized NWDAF instances

#### 5.3.3.1	Key issue details

In 3GPP TR 23.700-91 [1], Key Issue 19 describes trained model sharing between multiple NWDAF Instances. It has been concluded that "Sharing of models or model meta data is limited to single vendor environments."

Since machine learning models are trained using proprietary algorithms, and sometimes are also trained using sensitive data, securing them and ensuring restricted usage and secure transfer is paramount. Therefore, this key issue will study how to ensure that trained model sharing is only allowed among authorized NWDAF instances.

#### 5.3.3.2	Security Threats

If ML models are shared with an NF, which is not authorized, proprietary and sensitive implementation-specific information may be leaked.

#### 5.3.3.3	Potential security requirements

Only authorized NWDAF instances should be allowed to consume ML models from other NWDAF instances.

****End of Changes****