3GPP TSG-SA3 Meeting #103-e	S3-211609

# e-meeting, 17 – 28 May 2021													        	Revision of S3-20xxxx

# Source:	Huawei, HiSilicon

# Title:	pCR – Conclusions for KI#2

# Document for:	Approval

Agenda Item:	5.7 FS_UAS_SEC

# 1	Decision/action requested

Approve the proposed conclusions to KI#2 for TR33.854

# 2	References

[1]

# 3	Rationale

This contribution proposes to conclude the KI#2 study and starts normative work

(KI#2: Pairing authorization for UAV and UAVC)

# 4	Detailed proposal

pCR

***	BEGINNING OF 1st CHANGES    ***

## 7.2	Conclusions for KI#2

Pairing Authorization for UAV and UAVC is recommended for the normative work based on the following solutions and principles:

- Pairing authorization is performed after successful UAA between UAV and USS/UTM

- Pairing is performed either during registration or PDU session establishment (5G solution #5, #11, #14, #15 as basis)

- Both SMF and USS/UTM may trigger pairing authorization. USS/UTM may trigger revocation of pairing authorization

- For EPS: solution #13 is chosen as the basis for normative work, with similar principles as for 5GS above.

***	END OF 1st CHANGES	***