3GPP TSG-SA3 Meeting #103-e 	S3-211615

e-meeting, 17 - 28 May 2021

Title:	Reply LS on SLIC

Response to:	LS S3-211382 on User location identification from Carrier Aggregation secondary cell activation messages from GSMA FSAG

Release:	-

Work Item:	-

Source:	Ericsson [to be SA3]

To:	GSMA FSAG, RAN2

Cc:

Contact person:	Prajwol Kumar Nakarmi

prajwol (dot) kumar (dot) nakarmi (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

DISCUSSION FOR SA3. TO BE REMOVED BEFORE SENDING TO RAN2. GSMA FSAG's LS refers to a paper that describes the so-called SLIC attack. After analysing the paper, we agree with the observations made by GSMA in the LS. We have following observations on the four countermeasures listed in the SLIC paper: - The first countermeasure is to encrypt the MAC-CE. We do not believe that similar other low-risk side-channel attacks can be completely thwarted by just encrypting the SCell activation/deactivation MAC CE. Furthermore, introducing encryption in the MAC layer requires a fundamental design change which is not proportionate to the risk. - The second countermeasure is to add noise to the CA side channel. We suggest that RAN2 comments on the efficacy of this countermeasure. - The third countermeasure is to frequently change the UE identifiers. 5GS already ensures strict refreshment of 5G-GUTI and EPS allows implementation specific refreshment rate. - The fourth countermeasure is SLIC aware cell planning. We suggest that RAN2 comments on the efficacy of this countermeasure. Furthermore, we identified new countermeasures that may be simpler – which are best discussed in RAN2. For example: - Sending SCell activation/deactivation MAC CE over an SCell instead of the PCell so that the attacker is forced to be in a close vicinity of the SCell – in which case, the attacker can determine the victim’s location without going through the complex SLIC attack. - Allocating different C-RNTIs to the same UE over SCells and PCells so that the attacker cannot link the observed traffics to the same victim by merely using the radio level identifier. Therefore, we suggest that SA3 answers on security/privacy aspect and refers to RAN2 for the rest.

GSMA FSAG sent LS (S3-210006) to both SA3 and RAN on User location identification from Carrier Aggregation secondary cell activation messages. The LS refers to a paper that describes the so-called SLIC attack.

SA3 agrees with the observations made by GSMA FSAG that the practical risk of the SLIC attack is quite low.

SA3 has following comments on the countermeasures listed in the SLIC paper:

- Encrypting MAC-CE is a major design change unproportionate to the risk.

- Refreshing GUTI is already a part of standards.

- It is in the remits of RAN2 to comment on adding noise to the CA side channel and planning cells with SLIC awareness.

SA3 kindly requests GSMA FSAG and RAN2 to take SA3's comments into account.

# 2	Actions

To GSMA FSAG:

ACTION: 	SA3 kindly requests GSMA FSAG to take SA3's comments on MAC-CE and GUTI into account and refer to RAN2's reply on adding noise and planning cells.

To RAN2

ACTION: 	SA3 kindly requests RAN2 to take SA3's comments on MAC-CE and GUTI into account and reply to GSMA on adding noise and planning cells.

# 3	Dates of next TSG SA WG 3 meetings

SA3#103Bis-e	5 - 9 ~July 2021		Electronic meeting (TBC)

SA3#104-e	16 - 27 August 2021	Electronic meeting