3GPP TSG-SA3 Meeting #103-e 	S3-211625

# e-meeting, 17 – 28 May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.180 | CR | 0170 | rev | <Rev#> | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Security solution for temorary – broadcast group call procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | S3 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Work item code: | MCXSec2 |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The key management for group communication is based on the GMK which is distributed with the “Notify group configuration request”at group management phase before the group communication happens. However, when it comes to the temporary group – broadcast group call procedure, the existing security mechanim defined in 3GPP TS 33.180 is not applied as the group communication is not over a single group that already formed. The GMK is used to derive the SRTP Master Key/Salt which is used to establish protection of the group-broadcast group media. The security solution for the for temporary group – broadcast group call procedure is to provide a common SRTP Master Key for all the groups included in the temporary group call. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Propose a security solution for temporary group – broadcast group call procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no security solution for temporary group – broadcast group call procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.x(new); |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of the Change ****************

7.3.X	Temporary group – broadcast group call

Temporary Group-broadcast group call is described in clause 10.6.2.5.3 of 23.379 [2] and consists of a temporary group transmission to a set of groups rather than to a set of users.

In order to provide a common Temporary Group Master Key (TGMK) for all the groups included in the temporary group call, a TGMK can be generated by the initiating MC service client which is also the transmitting user, and distributed to all the other partcipants in the temporary group – broadcast group call. Figure 7.3.X-1 illustrates the temporary group – broadcast group call security procedure.

Figure 7.3.X-1: Group call setup and key management for temporary group – broadcast group call

1. 	The initiating MC service client generates the Temporary Group Master Key (TGMK) for the temporary group – broadcast group call media.

2.	The initiating MC service client initiates the group call by sending the group call request to the controlling server of the temporary group – broadcast group call. The initiating MC service client encrypts the TGMK using GMK of group A and GMK of group B respectively. The encrypted TGMK is attached to the group call request.

3.	The controlling server of the temporary group sends the TGMK encrypted by GMK of group A to the MC server A. The encrypted TGMK is attached to the group call request.

4.	The MC server A sends the group call request towards the MC clients in group A. The message contains the temporary group indicator. The message also contains the TGMK encrypted encrypted by GMK of group A.

5.	The receiving MC clients are notified about the incoming group call. The users verify they are authorized to participate in the temporary group call. The MC clients decrypt and extract the TGMK.

6.	The receiving MC clients accept the group call request and send group call response to the MC server A. This response may contain an acknowledgement.

7.	The MC server A sends the group call response to the controlling server of the temporary group to inform about successful call establishment.

8.	Steps 3-7 are repeated by the MC Server B for the MC clients in group B.

9.	MC clients establish media plane and floor control resources. The MCPTT media plane is protected using identical TGMK.

*************** End of the Change ****************