3GPP TSG-SA3 Meeting #103-e	S3-211632

# e-meeting, 17 – 28 May 2021

# Source:	Huawei, HiSilicon, China Broadcasting Network

# Title:	Update to the solution 11

# Document for:	Approval

Agenda Item:	5.11

1	Decision/action requested

This pCR proposes update to the solution 11 in TR 33.850.

2	Rationale

Based on the analysis, the attacker is hard to guess the SUPI if generating fake response to SEAF.

3	Detailed proposal

SA3 is kindly requested to agree to the below pCR to TR 33.850.

********************** First Change ****************************

6.11	Solution #11: Update the keys used to protect the MBS traffic

6.11.1	Introduction

This solution addresses Key issue #2 to update the keys to protect the MBS traffic. The MBS traffic is protected between MBSF-U and UEs. The basic idea is to use the signalling messages for the key update procedure. The UEs, which belong to a multicast group, acquire the same group keys as in the MBSF-U. The security protection is enabled in service layer.

6.11.2	Solution details

Figure 6.11.2 -1: Procedure for MTK update

1. The UE registers to 5GS and establishes a MBS session, during which MBSF-C generates MTK1&KID1 and distributes them to MBSF-U and UEs. MTK1&KID1 are used to protect the MBS traffic. Details can be found in solution 3 or solution 8.

2. MBSF-C subscribes to the UDM/UDR on the changes of the multicast information including the authorization information. The UDM/UDR notifies the MBSF-C when the authorization for a UE to join the multicast service/application is revoked. The MBS Session ID and UE info (i.e. GPSIs, number of UEs) are included in the notification. The MBS Session ID may be a Temporary Mobile Group Identifier (TMGI) or a multicast address.

3. MBSF-C triggers the MTK update based on the changes of authorization info or the key lifetime of MTK1. MBSF-C generates MTK2 and KID2 for this MBS session.

4-5. MBSF-C distributes MTK2, KID2 and MBS Session ID to (MB)-SMF and MBSF-U respectively.

6. (MB)-SMF distributes MTK2, KID2 and MBS Session ID to the authorized UEs whose MBS sessions are not released. Confirmation for update notification is responded to (MB)-SMF. With the confirmation info, (MB)-SMF determines to activate MTK2 based on local policy.

7. If the procedure to update the key in UE side is finished,  (MB)-SMF indicates MBSF-U to activate the MTK2 for the corresponding MBS session.

8. MBSF-U uses MTK2 and KID2 to protect the MBS traffic.

The above text describes the security handling when SMF and MB-SMF are co-located. If SMF and MB-SMF are deployed separately, MBSF-C distributes MTK2, KID2 and MBS Session ID to MB-SMF. Afterwards, MB-SMF distributes MTK2, KID2 and MBS Session ID to the authorized UEs via SMF. MB-SMF indicates MBSF-U to activate the MTK2

6.11. 3	Evaluation

This solution addresses Key issue #2 to update the key for protecting the MBS session. The solution protects the MBS traffic between MBSF-U and UEs.

MBSF-C decides to trigger the MTK update procedure based on the changes of authorization info or the key lifetime of MTK. The new MTK and KID are sent to UE and MBSF-U respectively.

********************** End of pCR********************