3GPP TSG-SA3 Meeting #103-e 	S3-211709

# e-meeting, 17- 28 May 2021

# Source:	Apple

# Title:	Evaluation on solution#9

# Document for:	Approval

Agenda Item:	5.8

# 1	Decision/action requested

It is proposed to add the evaluation for solution#9 in MEC TR 33.839.

# 2	References

[1]	3GPP TS 23.558: "Architecture for enabling Edge Applications (EA)"

# 3	Rationale

This pCR proposes to add the evaluation on solution#9.

# 4	Detailed proposal

****START OF CHANGES ***

## 6.9	Solution #9: Authentication and authorization between EEC and ECS based on AKMA

### 6.9.1	Introduction

This solution addresses the key issue #2. It is assumed that the key used for authentication between EEC and ECS is negotiated based on AKMA. Then, the EEC should initiate the service provisioning request with EEC ID included. To prevent EEC ID impersonation, the ECS should verify the authenticity of UE’s EEC ID before performing authorization based on the EEC ID. Considering the ECS can determine the authenticity of UE’s A-KID based on AKMA procedure, it can confirm the authenticity of UE’s EEC ID in case the association between A-KID and EEC ID can be verified. This solution further transforms the association between A-KID and EEC ID to the association between A-KID and GPSI based on the pre-configured association between EEC ID and GPSI in ECS. Afterwards, the ECS interworks with 5GC to verify the association between A-KID and GPSI.

After successful verification, the ECS may retrieve the edge computing related profile for the EEC from the 3GPP Core Network or from its local database for edge computing. Then, the ECS can determine the EEC’s authorization based on the profile.

### 6.9.2	Solution details

Figure 6.9.2-1: Authentication/Authorization between Edge Enabler Client and ECS

Pre-conditions:

-	The EEC and ECS have shared A-KID and KAF via AKMA (as specified in TS 33.535 [1]).

-	The ECS or the 5GC is configured with the edge computing related profile for the EEC.

-	The ECS and the 5GC share an UE identifier (i.e., GPSI) to identify the EEC.

-	The ECS stores the association between EEC ID and UE identifier. This association is pre-configured in the ECS by the ECS administrator.

Step 1: UE initiates the service provisioning procedure with EEC ID included (as specified in clause 8.3 in TS 23.558 [2]).

Step 2: The ECS retrieves GPSI from EEC ID according to the preconfigured association.

Step 3: In order to prove the authenticity of the UE’s GPSI, the ECS sends an association check request to UDM (if the ECS is located out of 5GC, the request should be sent via NEF), including the GPSI and A-KID.

Step 4: In order to verify the association of GPSI and A-KID, the UDM first contacts the AUSF to obtain the corresponding SUPI of the A-KID. Afterwards, the UDM verifies the association of the GPSI and A-KID according to the association between SUPI and GPSI.

Step 5: The UDM sends the association verification response back to the ECS.

Step 6: On successful verification, the ECS retrieves the edge computing related profile for the EEC either from the 5GC or from its local database. Afterwards, the ECS can determine the EEC’s authorization based on EEC’s profile.

Step 7: The ECS sends the provisioning response back to EEC.

### 6.9.3	Solution Evaluation

This solution fully addresses the security requirement for authentication between EEC and ECS in the key issue #2 based on the AKMA.

This solution requires the ECS or the 5GC is configured with the edge computing related profile for the EEC and the ECS and the 5GC share an UE identifier (i.e., GPSI) to identify the EEC

This solution also requires that the EEC and ECS shall support the AKMA. The solution assumes the association between EEC ID and UE ID (i.e., GPSI) which is pre-configured in the ECS by the ECS administrator and verifies the association during service provisioning which requires updates to the service of UDM and AAnF.

This solution doesn’t apply to the case when there are multiple EECs in one UE.

Editor’s Note: It is ffs whether this solution fully addresses the key issue.

****END OF CHANGES ***