3GPP TSG-SA3 Meeting #103-e 	S3-211731

# e-meeting, 17 - 28 May 2021

# Source:	Nokia, Nokia Shanghai Bell

# Title:	Resolution of Editor's Note in Solution 19 related to UDM involvement

# Document for:	Approval

Agenda Item:	5.12

# 1	Decision/action requested

The contribution proposes to resolve an Editor's Note in Solution 19 related to UDM involvement.

# 2	References

[1]	3GPP TR 33.857:” Study on enhanced security support for Non-Public Networks”

[2]			3GPP TR 23.700-07:” Study on enhanced support of non-public networks”

# 3	Rationale

Solution 19 of TR [1] contains following editor's note:

Editor's note: The impact on the authentication procedure if the UDM is not involved in the choice of authentication method is ffs.

From a security point of view, it does not make a difference, if the decision is taken by the AUSF or the UDM. Since the onboarding UE does not have default credentials, there is also no information only known to the UDM, which would mandate involvement of the UDM for selection of authentication method.

SA2 has been working on an onboarding mechanism without UDM and is in the process of normative specification. It should be agreed with SA2, if UDM should be involved in onboarding  xz<process or not. There is no security impact related to this.

It is proposed to remove the Editor's Note and to add a Note that decision should be taken during normative phase based on options provided by SA2.

Proposal It is proposed to remove the Editor's Note and to add a Note that decision should be taken during normative phase based on options provided by SA2.

# 4	Detailed proposal

It is proposed that SA3 agree the below pCR for inclusion in the TR [1].

**** START OF CHANGES ****

## 6.19	Solution #19: Secure onboarding without client authentication

### 6.19.1	Introduction

This solution addresses key issue#4 Securing initial access for UE onboarding between UE and SNPN. The scope of the solution is limited to cases, in which the subsequent onboarding shall be executed using a restricted PDU session.

In this solution one-way authentication including 5G key hierarchy is executed. The main difference to other solutions is that the network does not authenticate the UE, e.g., no peer authentication is applied during EAP-TLS authentication, The main difference of the modified variant with respect to EAP-TLS is that it does include server authentication only, but no client authentication.

That is, no default credentials or default credential server needs to be involved. Default credentials will be used only during the actual provisioning step, which is outside the scope of this solution.

### 6.19.2	Solution details

Figure 6.19.2-1 shows a generalisation of the solution.

Figure 6.19.2-1: initial access and sharing of identity.

1.	The UE sends a Registration Request including a SUCI to the network.

2.	AMF / SEAF forwards request to AUSF.

3.	Based on the received SUCI the AUSF concludes that the UE wants to execute unauthenticated access and selects a corresponding EAP-TLS method configured without client authentication. The selection of the EAP method might be carried out by the AUSF, or the AUSF might invoke the UDM for this (not shown in Figure 6.19.2-1)

Note: Decision of whether UDM needs to be involved can be taken during normative work based on available options from SA2.

4.	UE and AUSF execute EAP based authentication using the selected EAP-TLS method. This is following the procedure in TS 33.501 [2] described for EAP-TLS except that the selected EAP-TLS method without client authentication.

5.	Before the last step of the EAP procedure the AUSF calculates KAUSF and KSEAF as defined in TS 33.501 [2], i.e., The EMSK resulting from the executed EAP session is used as input for the derivation of KAUSF.

6.	The AUSF returns response message including EAP Success message, KSEAF and SUPI. The SUPI is set to a predefined constant value, which indicates to the SEAF that the UE has not been authenticated.

7.	AMF / SEAF finalizes the EAP session towards the UE.

8.	SEAF calculates the KAMF as specified in 3GPP TS 33.501 [2] with the difference that not a real SUPI, but a reserved string is used as input to the key derivation function. The calculation of the remaining 5G keys is according to 3GPP TS 33.501 [2].

9.	UE calculates all 5G keys according to the definitions in TS 33.501 [2], with the difference that not a real SUPI but the same reserved string also used by the SEAF is used as input to the key derivation function.

After the one-way authentication has been executed, the UE can request a restricted PDU Session as studied in TR 23.007-7 [3] and currently standardized in TS 23.501 [4]. The actual provisioning of the Subscriber profile is executed subsequently and outside the scope of this solution.

Editors note: The security implications of skipping client authentication is FFS.

Editors note: It is ffs how the root certificate used to verify the O-SNPN server cerifificate is pre-provisioned in the UE.

Editors note: Interoperability between different authentication method and identification of potential bidding down attacks is FFS.

Editors note: The construction of SUCI based on information provisioned to the UE is FFS.

### 6.19.3	System impact

TBD

### 6.19.4	Evaluation

Editor’s Note: Each solution should motivate how the potential security requirements of the key issues being addressed are fulfilled.

****** END OF CHANGES ****