3GPP TSG-SA3 Meeting #103e 	S3-211807

# e-meeting, 17 - 28 May 2021														    Revision of S3-21xxxx

# Source:	Qualcomm Incorporated

# Title:	Add an evaluation for solution #10

# Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

This contribution proposes an update to add evaluation.

# 2	References

[1]	TR 33.847 v0.5.0

# 3	Rationale

This contribution proposes to add an evaluation of solution #10 based on the architectural impacts on the core network components.

# 4	Detailed proposal

***** START OF CHANGES *****

### 6.10.3	Evaluation

This solution requires a new relayed primary authentication procedure to enable Remote UE to perform primary authentication with the AUSF of Remote UE via the AMF of Relay UE, which further requires new core network procedures in managing the remote UE context.

The Remote UE needs to perform primary authentication whenever it establishes a connection with a relay UE if the Remote UE is not registered to the 5GS.

The Remote UE cannot perform the procedure described in subclause 6.10.2.2 if the remote UE is not already registered to the 5GS.

### To support the PC5 link establishement procedure based on the Remote UE’s 5G security context, the AMF needs to support a new 5GMM context management mechanism for the Remote UE. Furthermore, it requires either AMF upgrade to support both normal UE and ProSe UE procedures (for both Remote and Relay) or dedicated AMFs that support the ProSe UE procedures. For example,  the AMF should be able to process Remote UE’s registration update and 5G-GUTI reallocation when Remote UE’s security context is transferred to the relay UE’s AMF (option 2).

***** END OF CHANGES *****