3GPP TSG SA WG 3 Meeting SA3#103e		TDoc S3-211899

Electronic meeting, Online, 17 - 28 May 2021

Title:	Draft LS on TAU reject issue during MME handover

Response to:

Release:	Rel-15 onwards

Work Item:

Source:	SA3

To:	CT4

Cc:

Contact person:	Bo Zhang

Zhangbo6@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S3-210397

- Overall description

When a fast moving UE attaches to 5GS, then moves to MME-1 in EPS and shortly thereafter moves to MME-2 the TAU request message sent from the UE to the MME-2 will always fail caused by the misalignment on the EPS security context, i.e. the security context stored in the UE is the mapped type, and the security context stored in the MME-2 is the native type. Details of the issue and analysis can be found in the attachment S3-210397.

SA3 has discussed this issue, and identified that this issue can be easily avoided by including the TSC part indicating the mapped or native security context during the MME-1 to MME-2 reallocation as defined in clause 7.2.8.4.3 of TS 33.401. Therefore, it is required that CT4 should align with SA3 in this case.

2	Actions

To 3GPP TSG SA CT1

ACTION: 	SA3 kindly asks CT4 to take the above information into account, and revise the EPS Security Context and Quadruplets forwarded from the source MME to the target MME.

3	Dates of next TSG SA WG 3 meetings

SA3#103bis-e	5 - 9 July 2021		Electronic meeting

SA3#104e	16 - 27 August 2021	Electronic meeting