3GPP TSG-SA3 Meeting #103-e 	S3-212119

# e-meeting, 17 - 28 May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 33.501 | CR | <CR#> | rev | <Rev#> | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Annex A.13 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | S3 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2021-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | SA3 has specified in TS 33.501[3], clause 6.9.4.3, the NAS key refresh as follows: #### 6.9.4.3 NAS key refresh If the AMF determines that NAS key refresh is required due to e.g. uplink or downlink NAS counter in the current security context is about to wrap around or based on a local operator policy to refresh the NAS keys after a certain time, the AMF may trigger a primary authentication run or may derive a new KAMF key using horizontal KAMF derivation upon the reception of an initial NAS message, e.g. a Registration Request or a Service Request using the uplink NAS COUNT value in the initial NAS message as described in clause 6.9.3 for mobility update registration. The AMF resets the corresponding uplink and downlink NAS counters and derive new NAS keys from the new KAMF key and the algorithms in use. The AMF activates the new KAMF key by running a NAS SMC with UE according to clause 6.7.2. When the new KAMF key is horizontally derived, the UE shall use the uplink NAS COUNT value that was sent in the initial NAS message to derive the same KAMF key as the AMF, reset the corresponding uplink and downlink NAS counters and then derive new NAS keys from the KAMF and the algorithms in use. In this case, if AS security is also established between the UE and gNB/ng-eNB, then the AMF and the UE shall derive a new initial KgNB from the new KAMF as specified in Annex A.9. Further, the AMF and the UE shall associate the derived new initial KgNB with a new NCC value equal to zero. Further, the derived new initial KgNB/KeNB is sent by the AMF to the gNB/ng-eNB triggering the gNB/ng-eNB to perform the AS key re-keying as described in clause 6.9.4.4. Clause 6.9.4.3 refers to clause 6.9.3 which does not describe the case when a Service Request message is used. Clause 6.9.3 refers to Annex A.13. Annex A.13 in TS 33.501, describes that the COUNT shall be the uplink NAS COUNT of the 3GPP access used in the Registration Request when performing the horizontal Kamf derivation. But for the case when UE has initiated a Service Request, then A.13 does not describe that the COUNT shall be the uplink COUNT of the 3GPP access corresponding to the Service Request message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause A.13 needs to be updated to describe that when when UE has initiated a Service Request, then the COUNT shall be the uplink COUNT of the 3GPP access corresponding to the Service Request message as input to horizontal Kamf derivation for the case when NAS COUNT’s are about to wrap around. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Horizontal Kamf derivation in Service Request procedure may fail between UE and AMF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** START OF CHANGES ****

# A.13	KAMF to KAMF'  derivation in mobility

Derivation of KAMF'  from KAMF during mobility shall use the following input parameters.

-	FC = 0x72

-	P0 =  DIRECTION

-	L0 = length of DIRECTION (i.e. 0x00 0x01)

-	P1 = COUNT,

-	L1 = length of COUNT (i.e. 0x00 0x04)

The input key KEY shall be KAMF.

When KAMF' is derived in handover, DIRECTION shall be 0x01 and COUNT shall be the downlink NAS COUNT of the 3GPP access.

When KAMF' is derived in idle mode mobility (i.e., mobility registration update), DIRECTION shall be 0x00 and COUNT shall be the uplink NAS COUNT of the 3GPP access used in the Registration Request.

When KAMF' is derived in Service Request procedure, DIRECTION shall be 0x00 and COUNT shall be the uplink NAS COUNT of the 3GPP access used in the Service Request.

**** END OF CHANGES ****