3GPP TSG-SA3 Meeting #103-e 	S3-212307

# e-meeting, 17 – 28 May 2021	                         revision S3-211641

# Source:	Huawei, HiSilicon, China Mobile, Nokia, Nokia Shanghai Bell

# Title:	Execution Step for Finding the right NF instance serving the UE

# Document for:	Approval

Agenda Item:	4.5

- Decision/action requested

This contribution proposes to add execution step for test case in TR 33.521

- References

- Rational

Pre-condition, execution steps and expected results of test case for Finding the right NF instance serving the UE are not complete. Thus, the contribution proposes to add pre-condition, execution steps and expected results for the test case.

- Detailed proposal

*************** Start of 1st Change ****************

4.2.2	Finding the right NF instance are serving the UE

Requirement Name: Finding the right NF instance are serving the UE

Requirement Reference: TS 23.288[2], clause 6.2.2.1.

Requirement Description: "To retrieve data related to a specific UE, the NWDAF shall first determine which NF instances are serving this UE as stated in table 6.2.2.1-2 unless the NWDAF has already obtained this information due to recent operations related to this UE.

Table 6.2.2.1-2: NF Services consumed by NWDAF to determine which NF instances are serving a UE

| Type of NF instance (serving the UE) to determine | NF to be contacted by NWDAF | Service | Reference in TS 23.502 [3] |
| --- | --- | --- | --- |
| UDM | NRF | Nnrf_NFDiscovery | 5.2.7.3 |
| AMF | UDM | Nudm_UECM | 5.2.3.2 |
| SMF | UDM | Nudm_UECM | 5.2.3.2 |
| BSF | NRF | Nnrf_NFDiscovery | 5.2.7.3 |
| PCF | BSF | Nbsf_Management | 5.2.13.2 |
| NEF | NRF | Nnrf_NFDiscovery | 5.2.7.3 |

." as specified in TS 23.288 [2], clause 6.2.2.1.

Threat Reference:  TS 33.926 [4], clause X.Y.

Test Name: TC_NWDAF_FIND_NF_SRVING_UE

Purpose:

Verify that the NWDAF always find a recent NF from operations related to the UE.

Editor’s Note: Purpose of test to be clarified.

Pre-Conditions:

Test environment with UE, source AMF, target AMF and UDM. UE, source AMF, target AMF and UDM may be simulated.

The UE is registrated on the source AMF and the UDM, and the NWDAF subscribes analytics A which needs to collect the UE’s information on the source AMF.

Execution Steps

TBD

Expected Results:

TBD

Expected format of evidence:

TBD

*************** End of 1st Change ****************