3GPP TSG-SA3 Meeting #104-e ad-hoc 	draft_S3-213355-r1

- e-meeting, 27 – 30 September 2021	Revision of S3-21xxxx

- Source:	InterDigital

- Title:	Analysis for PC5 root key anchor function selection for CP based approach

- Document for:	Discussion

Agenda Item:	5.2

# 1	Decision/action requested

This contribution proposes an analysis for the selection of the function responsible for PC5 root key derivation/distribution for the CP based approach following up on agreed L3 U2N Relay conclusion in TR 33.847

# 2	References

[1] TR 33.847 v0.7.0

# 3	Rationale

TR 33.847 [1], conclusion for L3 U2N relay in clauses 7.3 and 7.4 have the following NOTE:

NOTE:	Which Network Function (e.g. AMF or AUSF) is responsible for PC5 key derivation and distribution will be decided during normative phase. The detailed procedure to enable authorization for Remote UE/Relay will be determined accordingly during normative phase.

This paper provides a comparative analysis and recommendation for the selection of the function, i.e., AMF or AUSF,  responsible for PC5 root key derivation/distributionfor the normative phase.

The draft_S3-213355-r1 aims to provide comments regards the original discussion paper (S3-213355).

# 4	Discussion

In the CP based solutions for L3 U2N Relay authorization and security, the network function candidates for PC5 key derivation/distribution are AUSF (Sol#1, #15/#39, #30), AMF (Sol#10).

The following key common principles have been identified for these solutions:

- The PC5 root key is derived from a key derived from the primary authentication:

  - #10: PC5 root key is derived from a primary authentication that takes place during PC5 link establishment.

  - Sol#1 and #30: PC5 root key can be derived from a primary authentication during PC5 link establishment when SUCI is carried in DCR. PC5 root key can be derived based on security materials from a primary authentication before PC5 link establishment if the remote UE has existing 5G-GUTI.

  - Sol#15/#39: PC5 root key is derived from a primary authentication that takes place before PC5 link establishment.

- The PC5 root key is derived by the Remote UE, the same PC5 root key is derived by the network (AMF or AUSF) and sent securely to the Relay UE by AMF.

  - Sol#1: PC5 root key (REAR Key) is derived by AUSF, the next layer key KNR_ProSe is derived by AMF.

  - Sol#10: PC5 root key is derived by AMF.

  - Sol#15/#39, #30: PC5 root key is derived by AUSF.

- Remote UE and Relay UE base on the PC5 root key to verify relaying authorization and to establish the PC5 link security context.

- Remote UE can optionally connect with the Relay UE using its 5G-GUTI in DCR to bypass primary authentication run (sol#1, #10, #30).

The following table provides a summary comparison between using AUSF or AMF as a function responsible for PC5 key derivation/distribution, with respect to the key security features/KIs below. In the table, elements that are supported only by some of the solutions using AUSF for PC5 key management are indicated between "[ ]".

|  | AUSF |  | AMF |
| --- | --- | --- | --- |
| PC5 keys derivation & distribution (KI#3, #9) | KAUSF  PC5 root key (1) Sol#1: AUSF derives PC5 root key (REAR key), AMF derives lower layer key KNR_ProSe. Sol#15/39: AUSF derives PC5 root key directly from KAUSF |  | KAMF  PC5 root key NOTE: AMF derives the PC5 root key KRelay from KAMF, the KAMF is derived from the AUSF based on KAUSF. The KAMF may change during the context transfer which may impact on 5G NAS security context. |
| procedure impacts (KI#4) | Relay UE (A&A), AMF([key gen], A&A), AUSF (A&A, key gen), UDM([key/id store]) |  | Relay UE (A&A), AMF (A&A, key gen), AUSF (A&A), UDM |
|  | UE | Sol #15, #39: P-KID derivation Sol #1: REAR Key derivation, additional logic to re-send DCR if indicates by AMF. Sol #1, #30: Additional processing logic to carry either 5G-GUTI or SUCI in DCR | Additional processing logic: - select the ID carried in DCR (5G-GUTI, or SUCI, or none) - Both Remote and Relay UE needs to check whether PLMN IDs are identical or not. - Relay UE needs to ask for the Remote UE’s ID if PLMN IDs are not same, this requires additional unprotected signalling message. - Which ID is carried, or no ID is carried when UEs with differen PLMN IDs, it’s still FFS. |
|  | AMF | Sol#1, #30: key gen, A&A, additional processing logic to execute different procedures with 5G-GUTI or SUCI in DCR Sol #1: Relay AMF manages security context for Remote UE, and checks the authorisation of the Remote UE. Sol #15, #39: A&A (only Remote AMF) NOTE: There is no impact on AMF in sol#15,#39 | Sol #10: key gen, A&A, additional processing logic to execute different procedures with 5G-GUTI or SUCI in DCR. Relay AMF manages security context for Remote UE, and checks the authorisation of the Remote UE NOTE: Relay AMF has to stroe remote UE’s context which is a big change to SA2. |
|  | AUSF | Sol#1, #15, #30, #39: key gen, A&A | Sol #10: A&A NOTE: There is no impact on AUSF |
|  | UDM | Sol #1, #15, #39: Key/ID storage | Sol #10: A&A (gets AV) |
| MM Context impact | Sol #1: Yes, new UE MM context Sol #15, #30, #39: None |  | Sol #10: Yes, new UE MM context in both Remote AMF and Relay AMF |
| Identifier(s) in DCR & privacy (KI#5) | SUCI, [5G-GUTI], [Root Key ID], [P-KID] Support for identifiers privacy protection using existing mechanisms (5G-GUTI) including PC5 key/PC5 root key ID privacy re-using TS 33.536 mechanisms for KNRP ID 5G-GUTI reallocation is FFS (sol#1, #30) 5G-GUTI reallocation in sol#15, #39 can reuse the mechanism defined in TS 33.501. Because the key request can happen during the registration procedure. Sol #30: Whether or not Remote UE can use its NAS key to protect the PC5 messages is FFS. Root Key ID and P-KID privacy are FFS (sol#30, #15/#39) Sol #1, #30: Privacy risk happens when Remote UE uses 5G-GUTI to connect more than one Relays. |  | SUCI, 5G-GUTI, PC5 root key id Support for identifiers privacy protection using existing mechanisms (option 1) including PC5 root key id privacy re-using TS 33.536 mechanisms for KNRP ID 5G-GUTI reallocation is FFS if option 2 is considered (2) NOTE: Whether or not Remote UE can use its NAS key to protect the PC5 messages is FFS. Sol #10: Privacy risk happens when Remote UE uses 5G-GUTI to connect more than one Relays. |
| Reconnection with Relay (KI#3) | Reuse PC5 root key sending fresh PC5 root key id using existing TS 33.536. Reuse of KNRP ID i.e. no additional signaling for authorization towards the network required Or Sol #1, #30: [May use 5G-GUTI in DCR with signaling with the network for new PC5 root key retrieval/derivation] |  | Reuse PC5 root key sending fresh PC5 root key id using existing TS 33.536. Reuse of KNRP ID i.e. no additional signaling for authorization towards the network required Or May use 5G-GUTI in DCR |
| Key lifetime & storage (KI#9) | FFS Sol #1, #30: Sol #15, #39: A fresh PC5 root key (equvialent to KNRP) is derived when relay send request to AMF by using freshness parameter as KDF input. Sol #15, #39: A fresh PC5 root key can be derived using current KAUSF when the Relay sends a request to Remote UE’s AMF (Remote UE SUCI or 5G-GUTI in DCR). P-KID stored in UDM (sol#15/#39) Root Key and Root Key ID stored in UDM (sol#30) Root Key stored in AUSF (sol#1, #30) |  | N/A A fresh PC5 root key is derived using current KAMF when the Relay sends a request to AMF (Remote UE SUCI or 5G-GUTI in DCR) Relay AMF manages security context for Remote UE. NOTE: Detailed inputs of key derivation are FFS. NOTE: How to refresh PC5 root key is FFS. Maybe additional round of primary authentication is needed. |
| NOTES | - Root key naming: sol#1:REAR key; sol#15/39: KR; sol#30: 5GPRUK. - Option1: no Remote UE context transfer to Relay AMF with 5G-GUTI in DCR. Option 2: with context transfer support. |  |  |

# 5	Conclusion

In summary:

- Using AUSF or AMF presents impact to the network, specific impacts please see the Table above.

-  Using AUSF or AMF as a function responsible for PC5 key derivation/distribution presents several open issues (i.e., marked as "FFS" for the security features/KIs listed above), and that would still need to be addressed.

- Using AMF as the function responsible for PC5 key derivation/distribution presents a solution with new impact on relay UE’s AMF which is a new thing to SA2.

# 6	Proposal

.