3GPP TSG-SA3 Meeting #104-e ad-hoc 	draft_S3-213537-r1

- e-meeting, 27 - 30 September 2021											Revision of S3-20xxxx

- Source:	ZTE Corporation

- Title:	Security for 5G ProSe Direct Discovery

- Document for:	Approval

Agenda Item:	4.8

# 1	Decision/action requested

This document proposes the security for 5G ProSe direct discovery, including open discovery and restricted discovery, to to Clause 6.1 of the TS 33.XXX “Security Aspects of Proximity based Services (ProSe) in the 5G System (5GS) (Release 17)” . SA3 is kindly requested to approve this document.

# 2	References

[1]	3GPP TS 33.XXX, v 0.0.0. “Security Aspects of Proximity based Services (ProSe) in the 5G System (5GS) (Release 17)”

[2]	3GPP TR 33.847, v 0.7.0. “Study on security aspects of enhancement for proximity based services in the 5G System (5GS) (Release 17)”

# 3	Rationale

This contribution proposes the security for 5G ProSe direct discovery, including open discovery and restricted discovery, to to Clause 6.1 of the TS 33.XXX “Security Aspects of Proximity based Services (ProSe) in the 5G System (5GS) (Release 17)”.

The procedure in Solution #3 of the TR 33.847 [2] is used for open ProSe direct discovery.

The procedures in Solution #4 of the TR 33.847 [2] are used for restricted ProSe direct discovery.

# 4	Detailed proposal

***	BEGIN OF CHANGE	***

- 6.1	Security for 5G ProSe Direct Discovery

- 6.1.1	General

- 6.1.2	Security requirements

- 6.1.3	Security procedures

#### 6.1.3.1	Open discovery security procedure

The open discovery security procedure is described as follows:

- The announcing UE sends a Discovery Request message containing the ProSe Application ID to the DDNMF in its HPLMN in order to be allowed to announce a code on its serving PLMN (either VPLMN or HPLMN).

- If the announcing UE wants to send announcements in the VPLMN, it needs to be authorised from the VPLMN ProSe Function. The DDNMF in the HPLMN requests authorization from the VPLMN DDNMF by sending Announce Auth.() message.

- VPLMN DDNMF responds with an Announce Auth. Ack () message, if authorization is granted. There are no changes to these messages for the purpose of protecting the transmitted code for open discovery. If the Announcing UE is not roaming, these steps do not take place.

- The DDNMF in HPLMN of the announcing UE returns the ProSe App Code that the announcing UE can announce and a Discovery Key associated with it. The DDNMF stores the Discovery Key with the ProSe App Code. In addition, the DDNMF provides the UE with a CURRENT_TIME parameter, which contains the current UTC-based time at the DDNMF, a MAX_OFFSET parameter, and a Validity Timer. The UE sets a clock which is used for ProSe authentication (i.e. ProSe clock) to the value of CURRENT_TIME and the UE stores the MAX_OFFSET parameter, overwriting any previous values. The announcing UE obtains a value for a UTC-based counter associated with a discovery slot based on UTC time. The counter is set to a value of UTC time in a granularity of seconds. The UE may obtain UTC time from any sources available, e.g. the RAN via SIB16, NITZ, NTP, GPS, via Ub interface (in GBA) (depending on which is available).

NOTE 1:	The UE may use unprotected time to obtain the UTC-based counter associated with a discovery slot. This means that the discovery message could be successfully replayed if a UE is fooled into using a time different to the current time. The MAX_OFFSET parameter is used to limit the ability of an attacker to successfully replay discovery messages or obtain correctly MICed discovery message for later use. This is achieved by using MAX_OFFSET as a maximum difference between the UTC-based counter associated with the discovery slot and the ProSe clock held by the UE.

NOTE 2:	A discovery slot is the time at which an announcing UE sends the announcement.

- The UE starts announcing, if the difference between UTC-based counter provided by the system associated with the discovery slot and the UE’s ProSe clock is not greater than the MAX_OFFSET and if the Validity Timer has not expired. For each discovery slot it uses to announce, the announcing UE calculates a 32-bit Message Integrity Check (MIC) to include with the ProSe App Code in the discovery message. Four least significant bits of UTC-based counter are transmitted along with the discovery message. The MIC is calculated as described in clause A.2 of TS 33.303 [6] using the Discovery Key and the UTC-based counter associated with the discovery slot.

- The Monitoring UE sends a Discovery Request message containing the ProSe Application ID to the DDNMF in its HPLMN in order to get the Discovery Filters that it wants to listen for.

- The DDNMF in the HPLMN of the monitoring UE sends Monitor Req. message to the DDNMF in the HPLMN of the announcing.

- The DDNMF in the HPLMN of the announcing UE sends Monitor Resp. message to the DDNMF in the HPLMN of the monitoring.IfMIC needs to be checked by the DDNMF in the HPLMN of the monitoring UE or the monitoring UE, the Discovery Key should be contained in the response message.

- The DDNMF returns the Discovery Filter containing either the ProSe App Code(s), the ProSe App Mask(s) or both along with the CURRENT_TIME and the MAX_OFFSET parameters. The UE sets its ProSe clock to CURRENT_TIME and stores the MAX_OFFSET parameter, overwriting any previous values. The monitoring UE obtains a value for a UTC-based counter associated with a discovery slot based on UTC time. The counter is set to a value of UTC time in a granularity of seconds. The UE may obtain UTC time from any sources available, e.g. the RAN via SIB16, NITZ, NTP, GPS (depending on which is available).IfMIC needs to be checked by the monitoring UE, the Discovery Key should be contained in the response message.

- The Monitoring UE listens for a discovery message that satisfies its Discovery Filter, if the difference between UTC-based counter associated with that discovery slot and UE’s ProSe clock is not greater than the MAX_OFFSET of the monitoring UE's ProSe clock.If the monitoring UE has the Discovery Key, the MIC check may be performed locally, and steps 11 to 15 may be omitted.

- On hearing such a discovery message, and if the UE needs to check the MIC for the discovered ProSe App Code, the Monitoring UE sends a Match Report message to the DDNMF in the HPLMN of the monitoring UE. The Match Report contains the UTC-based counter value with four least significant bits equal to four least significant bits received along with discovery message and nearest to the monitoring UE’s UTC-based counter associated with the discovery slot where it heard the announcement, and other discovery message parameters including the ProSe App Code and MIC.If the DDNMF in the HPLMN of the monitoring UE has the Discovery Key, the MIC check is performed locally, and steps 12 to 14 are omitted.

- The DDNMF in the HPLMN of the monitoring UE passes the discovery message parameters including the ProSe App Code and MIC and associated counter parameter to the DDNMF in the HPLMN of the announcing UE in the Match Report message.

- The DDNMF in the HPLMN of the announcing UE should check the MIC is valid. The relevant Discovery Key is found using the ProSe App Code.

- The DDNMF in the HPLMN of the announcing UE should acknowledge a successful check of the MIC to the DDNMF in the HPLMN of the monitoring UE in the Match Report Ack message. The DDNMF in the HPLMN of the announcing UE include a Match Report refresh timer in the Match Report Ack message. The Match Report refresh timer indicates how long the UE will wait before sending a new Match Report for the ProSe App Code.

- The DDNMF in the HPLMN of the monitoring UE acknowledges the check result to the monitoring UE. The DDNMF returns the parameter ProSe Application ID to the UE. It also provides the CURRENT_TIME parameter, by which the UE (re)sets its ProSe clock The DDNMF in the HPLMN of the monitoring UE may optionally modify the received Match Report refresh timer based on local policy and then include the Match Report refresh timer in the message to the Monitoring UE.

Figure 6.1.3.1-1: Open discovery security procedure

#### 6.1.3.2	Restricted discovery security procedure

##### 6.1.3.2.1 	Model A restricted discovery

The security procedure for Model A restricted discovery is described as follows:

Steps 1-4 refer to an Announcing UE.

- Announcing UE sends a Discovery Request message containing the RPAUID to the DDNMF in its HPLMN in order to get the ProSe Code to announce and to get the associated security material.

- The DDNMF may check for the announce authorization with the ProSe Application Server.

- If the Announcing UE is roaming, the DDNMFs in the HPLMN and VPLMN of the Announcing UE exchange Announce Auth.

- The DDNMF in the HPLMN of the Announcing UE returns the ProSe Code and the corresponding Code-Sending Security Parameters, along with the CURRENT_TIME and MAX_OFFSET parameters. The Code-Sending Security Parameters provide the necessary information for the Announcing UE to protect the transmission of the ProSe Code and are stored with the ProSe Code. The Announcing UE takes the same actions with CURRENT_TIME and MAX_OFFSET as described for the Announcing UE in step 4 of subclause 6.3.2 of the current specification.

Steps 5-10 refer to a Monitoring UE

- The Monitoring UE sends a Discovery Request message containing the RPAUID to the DDNMF in its HPLMN in order to be allowed to monitor for one or more Restricted ProSe Application User IDs.

- The DDNMF in the HPLMN of the Monitoring UE sends an authorization request to the ProSe Application Server. If, based on the permission settings, the RPAUID is allowed to discover at least one of the Target RPAUIDs contained in the Application Level Container, the ProSe Application Server returns an authorization response.

- If the Discovery Request is authorized, and the PLMN ID in the Target RPAUID indicates a different PLMN, the DDNMF in the HPLMN of the Monitoring UE contacts the indicated PLMN’s DDNMF i.e. the DDNMF in the HPLMN of the Announcing UE, by sending a Monitor Request message.

- The DDNMF in the HPLMN of the Monitoring UE may exchange authorization messages with the ProSe Application Server.

- The DDNMF in the HPLMN of the Announcing UE responds to the DDNMF in the HPLMN of the Monitoring UE with a Monitor Response message including the ProSe Code, the corresponding Code-Receiving Security Parameters and an optional Discovery User Integrity Key (DUIK). The Code-Receiving Security Parameters provide the information needed by the Monitoring UE to undo the protection applied by the announcing UE. The DUIK be included as a separate parameter if the Code-Receiving Security Parameters indicate that the Monitoring UE use Match Reports for MIC checking. The DDNMF in the HPLMN of the Monitoring UE stores the ProSe Code and the Discovery User Integrity Key (if it received one outside of the Code-Receiving Security Parameters).

NOTE 1:	There are two configurations possible for integrity checking, namely, MIC checked by the DDNMF, and MIC checked at the UE side. Which of the configuration is used is decided by the DDNMF that assigned the ProSe Code being monitored, and signalled to the Monitoring UE in the Code-Receiving Security Parameters.

- The DDNMF in the HPLMN of the Monitoring UE returns the Discovery Filter and the Code-Receiving Security Parameters, along with the CURRENT_TIME and MAX_OFFSET parameters. The Monitoring UE takes the same actions with CURRENT_TIME and MAX_OFFSET as described for the Monitoring UE in step 9 of subclause 6.3.2 of the current specification. The UE stores the Discovery Filter and Code-Receiving Security Parameters.

Steps 11 and 12 occur over PC5.

- The UE starts announcing, if the UTC-based counter provided by the system associated with the discovery slot is within the MAX_OFFSET of the announcing UE's ProSe clock and if the Validity Timer has not expired. The UE forms the discovery message and protects it. The four least significant bits of UTC-based counter are transmitted along with the protected discovery message.

- The Monitoring UE listens for a discovery message that satisfies its Discovery Filter, if the UTC-based counter associated with that discovery slot is within the MAX_OFFSET of the monitoring UE's ProSe clock. In order to find such a matching message, it processes the message. If the Monitoring UE was not asked to send Match Reports for MIC checking, it stops at this step from a security perspective. Otherwise it proceeds to step 13.

NOTE 2:	The UE checking the integrity of the discovery message on its own does not prevent the UE from sending a Match Report due to requirements in TS 23.303 [5]. If such a Match Report is sent, then there is no security functionality involved.

Steps 13-16 refer to a Monitoring UE that has encountered a match.

- If the UE has either not had the DDNMF check the MIC for the discovered ProSe Code previously or the DDNMF has checked a MIC for the ProSe Code and the associated Match Report refresh timer (see step 15 for details of this timer) has expired, then the Monitoring UE sends a Match Report message to the DDNMF in the HPLMN of the monitoring UE. The Match Report contains the UTC-based counter value with four least significant bits equal to four least significant bits received along with discovery message and nearest to the monitoring UE’s UTC-based counter associated with the discovery slot where it heard the announcement, and other discovery message parameters including the ProSe Code and MIC. The DDNMF checks the MIC.

- The DDNMF in the HPLMN of the Monitoring UE may exchange an Auth Req/Auth Resp with the ProSe App Server to ensure that Monitoring UE is authorised to discover the Announcing UE.

- The DDNMF in the HPLMN of the monitoring UE returns to the Monitoring UE an acknowledgement that the integrity check passed. It also provides the CURRENT_TIME parameter, by which the UE (re)sets its ProSe clock. The DDNMF in the HPLMN of the Monitoring UE include the Match Report refresh timer in the message to the Monitoring UE. The Match Report refresh timer indicates how long the UE will wait before sending a new Match Report for the ProSe Code.

- The DDNMF in the HPLMN of the Monitoring UE may send a Match Report Info message to the DDNMF in the HPLMN of the Announcing UE.

Figure 6.1.3.2.1-1: Model A restricted discovery security procedure

##### 6.1.3.2.2 	Model B restricted discovery

The security procedure for Model B restricted discovery is described as follows:

Steps 1-4 refer to a Discoveree UE.

- Discoveree UE sends a Discovery Request message containing the RPAUID to the DDNMF in its HPLMN in order to get Discovery Query Filter(s) to monitor a query, the ProSe Response Code to announce and associated security materials. The command indicates that this is for ProSe Response (Model B) operation, i.e. for a Discoveree UE.

- The DDNMF may check for the announce authorization with the ProSe Application Server depending on DDNMF configuration.

- The DDNMFs in the HPLMN and VPLMN of the Discoveree UE exchange Announce Auth. messages. If the Discoveree UE is not roaming, these steps do not take place.

- The DDNMF in the HPLMN of the Discoveree UE returns the ProSe Response Code and the Code-Sending Security Parameters, Discovery Query Filter(s) and their Code-Receiving Security Parameters corresponding to each discovery filter along with the CURRENT_TIME and MAX_OFFSET parameters. The Code-Sending Security Parameters provide the necessary information for the Discoveree UE to protect the transmission of the ProSe Response Code and are stored with the ProSe Response Code. The Code-Receiving Security Parameters provide the information needed by the Discoveree UE to undo the protection applied to the ProSe Query Code by the Discoverer UE. The Code-Receiving Security Parameters indicate a Match Report will not be used for MIC checking. The UE stores each Discovery Filter with its associated Code-Receiving Security Parameters. The Discoveree UE takes the same actions with CURRENT_TIME and MAX_OFFSET as described for the Announcing UE in step 4 of subclause 6.3.2 of the current specification.

Steps 5-10 refer to a Discoverer UE

- The Discoverer UE sends a Discovery Request message containing the RPAUID to the DDNMF in its HPLMN in order to be allowed to discover one or more Restricted ProSe Application User IDs.

- The DDNMF in the HPLMN of the Discoverer UE sends an authorization request to the ProSe Application Server. If the RPAUID is allowed to discover at least one of the Target RPAUIDs contained in the Application Level Container, the ProSe Application Server returns an authorization response.

- If the Discovery Request is authorized, and the PLMN ID in the Target RPAUID indicates a different PLMN, the DDNMF in the HPLMN of the Discoverer UE contacts the indicated PLMN’s DDNMF i.e. the DDNMF in the HPLMN of the Discoveree UE, by sending a Discovery Request message.

- The DDNMF in the HPLMN of the Discoveree UE may exchange authorization messages with the ProSe Application Server.

- The DDNMF in the HPLMN of the Discoveree UE responds to the DDNMF in the HPLMN of the Discoverer UE with a Discovery Response message including the ProSe Query Code(s) and their associated Code-Sending Security Parameters, ProSe Response Code and its associated Code-Receiving Security Parameters, and an optional Discovery User Integrity Key (DUIK) for the ProSe Response Code. The Code-Receiving Security Parameters provide the information needed by the Discoverer UE to undo the protection applied by the Discoveree UE. The DUIK be included as a separate parameter if the Code-Receiving Security Parameters indicate that the Discoverer UE use Match Reports for MIC checking. The DDNMF in the HPLMN of the Discoverer UE stores the ProSe Response Code and the Discovery User Integrity Key (if it received one outside of the Code-Receiving Security Parameters). The Code-Sending Security Parameters provide the information needed by the Discoverer UE to protect the ProSe Query Code.

NOTE 1:	There are two configurations possible for integrity checking, namely, MIC checked by the DDNMF, and MIC checked at the UE side; this is decided by the DDNMF that assigned the ProSe Code being monitored, and signalled to the Monitoring UE in the Code-Receiving Security Parameters.

- The DDNMFs in the HPLMN and VPLMN of the Discoverer UE exchange Announce Auth. messages. If the Discoverer UE is not roaming, these steps do not take place.

- The DDNMF in the HPLMN of the Discoverer UE returns the Discovery Response Filter and the Code-Receiving Security Parameters, the ProSe Query Code and the Code-Sending Security Parameters along with the CURRENT_TIME and MAX_OFFSET parameters. The Discoverer UE takes the same actions with CURRENT_TIME and MAX_OFFSET as described for the Monitoring UE in step 9 of subclause 6.3.2 of the current specification. The UE stores the Discovery Response Filter and its Code-Receiving Security Parameters and the ProSe Query Code and its Code-Sending Security Parameters.

Steps 12 to 15 occur over PC5.

- The Discoverer UE sends the ProSe Query Code and also listens for a response message, if the UTC-based counter provided by the system associated with the discovery slot is within the MAX_OFFSET of the announcing UE's ProSe clock and if the Validity Timer has not expired. The Discoverer UE forms the discovery message and protects it. The four least significant bits of UTC-based counter are transmitted along with the protected discovery message.

- The Discoveree UE listens for a discovery message that satisfies its Discovery Filter, if the UTC-based counter associated with that discovery slot is within the MAX_OFFSET of the Discoverer UE's ProSe clock. In order to find such a matching message, it processes the message.

NOTE 2:	Match Reports are not used for the MIC checking of ProSe Query Codes.

- The Discoveree sends the ProSe Response Code associated with the discovered ProSe Query Code. The Discoveree UE forms the discovery message and protects it. The four least significant bits of UTC-based counter are transmitted along with the protected discovery message.

- The Discoverer UE listens for a discovery message that satisfies its Discovery Filter. In order to find such a matching message, it processes the message. If the Discoverer UE was not asked to send Match Reports for MIC checking, it stops at this step from a security perspective. Otherwise it proceeds to step 16.

NOTE 3:	The UE checking the integrity of the discovery message on its own does not prevent the UE from sending a Match Report due to requirements in TS 23.303 [5]. If such a Match Report is sent, then there is no security functionality involved.

NOTE 4:	The security keys in the Code-Sending Security Parameters of discover UE and the security keys in the Code-Sending Security Parameters of discoveree UE need to be generated independently and randomly. This ensures that the impersonation of the discoveree UE is not feasible when the discoverer UEs make use of match reports.

Steps 16-19 refer to a Discoverer UE that has encountered a match.

- If the Discoverer UE has either not had the DDNMF check the MIC for the discovered ProSe Response Code previously or the DDNMF has checked a MIC for the ProSe Response Code and the associated Match Report refresh timer (see step 18 for details of this timer) has expired, then the Discoverer UE sends a Match Report message to the DDNMF in the HPLMN of the Discoverer UE. The Match Report contains the UTC-based counter value with four least significant bits equal to four least significant bits received along with discovery message and nearest to the monitoring UE’s UTC-based counter associated with the discovery slot where it heard the announcement, and other discovery message parameters including the ProSe Response Code and MIC. The DDNMF checks the MIC.

- The DDNMF in the HPLMN of the Discoverer UE may exchange an Auth Req/Auth Resp with the ProSe App Server to ensure that Discoverer UE is authorised to discover the Discoveree UE.

- The DDNMF in the HPLMN of the Discoverer UE returns to the Discoverer UE an acknowledgement that the integrity check passed. It also provides the CURRENT_TIME parameter, by which the UE (re)sets its ProSe clock. The DDNMF in the HPLMN of the Discoverer UE include the Match Report refresh timer in the message to the Discoverer UE. The Match Report refresh timer indicates how long the UE will wait before sending a new Match Report for the ProSe Response Code.

- The DDNMF in the HPLMN of the Discoverer UE may send a Match Report Info message to the DDNMF in the HPLMN of the Discoveree UE.

Figure 6.1.3.2.2-1: Model B restricted discovery security procedure

***	END OF CHANGE	***