3GPP TSG-SA3 Meeting #104-e 	S3-213061

# e-meeting, 16 - 27 August 2021					Revision of S3-212880

# Source:	Nokia, Nokia Shanghai Bell

# Title:	Way forward eSBA

# Document for:	Endorsement

Agenda Item:	5.20

# 1	Decision/action requested

Discussion on way forward, eSBA study and normative work

# 2	References

[1]	3GPP 33.875

# 3	Rationale

Introduction

The work on eSBA study was initiated to address open points after Rel-16 freezing and allow for a thorough analysis of key issues and solutions, which can lead to conscious decisions on whether security requirements need to be fulfilled and require an update of existing specification text or the addition of new features.

Several key issues and solution have been added so far. But Rel-17 is ending fast and decision on how to continue with the study need to be made.

The purpose of this discussion document is as follows:

- Identify key issues for urgent treatment in Rel-17 and establish a WID for those

- Allow for further analysis of key issues in Rel-18 and potential other eSBA aspects that need to be addressed and do not need immediate treatment or are still in a stage of analysis, i.e. continue eSBA study in Rel-18.

-r1 includes all inputs contributed in this meeting  6 new KIs(!)

-final revision includes notes from the discussion, no WID was endorsed.

Status of eSBA study and proposed way forward for KI resolution

| Key issues | KI/Solutions details | Comments | Way forward |
| --- | --- | --- | --- |
| #1: Authentication of NRF and NF Service Producer in indirect communication | Threat scenario: MitM cooperating with NFp 2 solution candidates sol#1: Service response verification in indirect communication without delegated discovery (Nokia) sol#6: Verification of Service Response from a NF Service Producer at the expected NF Set (Samsung) | Several concerns were raised for the Nokia solution Samsung solution updates / generalizes the solution for introduction of CCA_NFp Can we agree on intro of CCA_NFp in normative work? HW: Ensure the authenticity of NFp Nokia: thinks it is covered, solution for providing authenticity of NFs is there. Reselection case is important, this is achieved by including NF Set id (if also part of the certificate) MVNR: not needed, no need to carry over to rel-18 Samsung prefers Rel-17 HW: no need to postpone, but sees technical problems. | No conclusion |
| #2: SCP security domains | Trust assumptions not yet clear, list of ENs to be resolved in KI No solution so far | Solution pending | Postpone analysis to Rel-18, depending on trust assumptions |
| #3: Service access authorization in the “Subscribe-Notify” scenarios | Threat – URI not authorized No requirement yet Proposal in S3-212653] No solution so far accepted in TR, but several proposals under discussion (but not yet included in TR) | KI clarifications & solution pending Nokia: Despite KI clarifications and solution pending, it is suggested to address this KI in Rel-17, since the threat of URI being not authorized should be addressed. MVNR prefers to finalize either way in Rel-17, no carry over ERI: why would NFc provide a malicious URI? If NFc would be malicious, then it does need to modify URI, it can just send it to another NF | No conclusion 2 requirements were added, but questioned by Ericsson |
| #4: Authorization of SCP to act on behalf of an NF or another SCP | Threat: SCP acting without authorization and thus, compromising a NFc 2 solution candidates sol#2: Authorization between NFs and SCP (Nokia) sol#3: Using existing procedures for authorization of SCP to act on behalf of an NF Consumer (Ericsson) | KI & solutions available Nokia: Important to address this KI, since otherwise SCP could be malicious and act on behalf of NFcs that have not requested an access token or service. Resolve in Rel-17. Existing mechanisms are not sufficient. Propose to conclude with sol. #2 (adding inside the CCA_NFc the SCP id or SCP domain info (list of SCPs) such that NRF or NFp knows whether SCP was requested by NFc) ERI prefers #3 (SCP receiving CCA_NFc implies that SCP is authorized by NFc) Discussion on whether authentication between NF and SCP is sufficient as authorization of SCP to act on behalf of NFc (ERI/MVNR say yes) Nokia: maybe okay in service mesh, but in SCP standalone case not true SCP has access to the data! Need to establish trust. How to decide that SCP is actually authorized to provide the service response. NFp should only a response via an authorized SCP. New addition in sol#3 – if SCP is malicious, what would the new part on safe storage in SCP bring? | No conclusion |
| #5: End-to-end integrity protection of HTTP messages | sol#4: Service request authenticity verification in indirect communication sol#5: End-to-end integrity protection of HTTP body and method New proposed in this meeting: LS reply by CT4: S3-212418 S3-212928 - Integrity protection of HTTP message in consideration of update by SCP | 2 solution candidates Pending CT4 response Need to address backwards compatibility with Rel-16 MVNR: Stop this KI DCM: In general, it should be SA3 to decide on, not CT groups | LS response from CT4 to discourages KI, but this should be not decided by another group than SA3 Postpone to Rel-18 |
| #6: Access token usage by all NFs of an NF set | sol#7: Access token request for NF Set | Nokia: Existing concept (NF set with any NFp of NFp set would provide the service with the presented token). KI is about NFc set. Any NFc of a NFc set should be allowed to use a token provided for the NFc set. access token usage has not been clarified for NFc set so far and should be addressed in Rel17 Ericsson: is it best practice to do this? VZ: support KI, ability to request a token for a NF set, good to share the token HW: in certificate no NF set ID Currently only the UE context is shared VZ: How to use the authorization framework to use NF set is important to clarify. But cost model consideration to be taken into account. No need to request token for unique NF only. | No conclusion |
| Tdocs / KIs brought up for this meeting |  |  |  |
| Key issues | KI/Solutions details | Comments | Way forward |
| NEW #C S3-212879 (Nokia) vNRF – hNRF mutual authentication in service access authorization | KI was proposed in last meeting, here updated and resubmitted by Nokia in this meeting how to ensure the trust in the information provided by the vNRF to the hNRF during the access token get request | Pending acceptance of KI/sol vNRF hNRF mutual authentication is not possible via SEPP, clarification is needed if trust via SEPP can be achieved better Cablelabs: maybe not Rel-17 yet. Seems not to be the same understanding by all parties. VZ: Rel-18 ok DCM – GSMA needs it now, if Rel-18, maybe to late. | No KI added, no conclusion, maybe sufficient by CR |
| Also roaming NEW #B ??? S3-212651 (HW) New Key issue on authorization mechanism negotiation | KI / solution (2652) proposed Proposal: HPLMN decides on authorization method. if the Final authorization mechanism indicates static authorization, then the NFc could use the static authorization to access the NFp service. If the Final authorization mechanism indicates OAuth authorization, then the NFc could get the token from the NRF before consuming the service from the NFp. NFc could get the token from the NRF before consuming the service from the NFp . | Nokia comment: Static authorization as described in 33.501 is based on local authorization policy at the NRF and the NF Service Producer if token-based authorization is not used within one PLMN from 13.3.0: “During the Nnrf_NFDiscovery procedure, the NRF ensures that the NF Service Consumer is authorized to discover the NF Service Producer service(s)” i.e. without presenting an OAuth token Nokia: Isn’t this against the concept of 5G SBA in core? KI/Sol suggest usage of non-roaming concept in roaming ??? MVNR – solution already exists. Bootstrapping takes care of it – what are different features and releases ERI home operator decides. | KI/solution added Key issue #7: Authorization mechanism determination |
| NEW #Y: Proposal for KI on NRF deployment / handling S3-212878 (Nokia) | KI / solution presented in this meeting Deployment options for NRFs: - one NRF can serve the entire PLMN - one NRF can serve a set of network slices - one NRF can serve a single network slice - several NRFs can be deployed in a PLMN, optionally using a hierarchical structure • Proposal to add in TS 33.501 clarification text for NF Service Consumer behaviour along the lines The NF Service Consumer may have discovered a specific NRF in advance, e.g. a slice specific NRF, and can send its request directly to that NRF. If the NF Service Consumer requests an NRF, where the NF Service Producer is not registered (see NRF deployment options in 13.4.1.1.1a), the requested NRF needs to redirect/forward the service request to that NRF. • Proposal to add in TS 33.501 clarification text for local NRF deployments along the lines NOTE: In a local NRF deployment, the NF Service Producer only gets the certificate of the local NRF. Thus, the local NRF would need to check if the NF Service Consumer is authorized and the NRF where the NF Service Producer is registered would need to trust the NRF which has verified the NF Service Consumer. | Pending acceptance of KI/sol There is an ambiguous understanding among companies about hierarchical deployments / client registration & token issuing instances Clarification needed in Rel-17, because NRF deployments in intra-PLMN need to be addressed. | KI/solution added Key issue #8: Service access authorization requirements in intra-PLMN scenarios for PLMN deploying multiple NRFs (in OAuth2.0 AS role) |
| Earlier discussed KI, not yet agreed: S3-212641 (HW) New KI #Z: NF Domain granularity authorization | resubmission Issue: An attacker may control an NF Service Consumer to request a token using the real FQDN of the NF Service Consumer. EG. A stateless UDM sends its FQDN as the callbackReference URI to UDR for subscribe to the notifications. If the UDM as Consumer sends a tamper FQDN in the subscription request to the UDR, and the UDR doesn’t check the authentic of the FQDN, the UDR may response the notification to the malicious FQDN, resulting sensitive information leaked to unauthorized NFs. | Pending acceptance of KI/sol Merge with 2890? | KI was noted |
| NEW #W: S3-212890 (Nokia) KI and Solution for verification of NFc by NF producers | New KI/sol proposal Threat: If the requester fqdn/domain in the service request is tampered, the NF Service Producer can return a service response or a service notification to the malicious FQDN, resulting sensitive information leaked to unauthorized NFs. Sol: The NF Service Consumer adds fqdn or domain in the CCA. This guarantees that the consumer fqdn/domain is not manipulated while “travelling” to the NF Service Producer. | Pending acceptance of KI/sol Same threat as above??? Merge with 2641? | KI was noted |
| NEW KI: #A Proposed in this meeting S3-212519 (CableLabs) Authorization of IPX by PLMN in indirect roaming | New KI/sol proposal Proposes: Authorization of IPX to represent PLMN in indirect roaming Proposed requirement: The 5GS should provide a mechanism for a PLMN to verify that an IPX has been authorized by another PLMN as its roaming proxy. | Pending acceptance of KI/sol Nokia comment: Usage of indirect comm term is different than in SBA/SCP. If iPX is used, each PLMN has a dedicated contract with an IPX, one per other PLMN. This is the reason for having only 2 IPX in the PRINS protocol that can modify (on for outgoing PLMN, one for incoming PLMN), all other IPX on the route would just proxy the traffic. IPX has not been in focus of 3GPP. Guidance for this provided in GSMA?? | KI was noted |
| KI NEW #V S3-212916 (Samsung) Authorization for Inter-Slice Access | New KI/solution proposals Issue: NSSAI is neither a part of the certificate profile of NF Service Consumer, nor can it be assured that the NRF receiving the access token request contains the profile of the NF Service Consumer from which the request is received. S3-212917 - New Solution to KI #X: Authorization for Inter-Slice Access Solution proposed: The 5GS should provide a mechanism that allows NRF validate the S-NSSAIs of the NF-Service Producer that an NF-Service-Consumer is allowed to access. Using individual or combination of IEs like allowedNfTypes and allowedNssais, NRF can already filter the requests from NF Service Consumers not allowed to access any services of an NF Service Producer. Additionally, to allow NF Service Producers (themselves) validate the “Inter-Slice” access requests from the NF Service consumers, it is proposed to include “Requester-NSSAI” in the access-grant, indicating the NRF validated NF-Consumer’s S-NSSAIs. | Pending acceptance of KI/sol | KI added Key issue #9: Authorization for Inter-Slice Access |

# 4	Proposal

It is proposed to follow the way forward for eSBA SID and eSABA WID as discussed in the Rational.