- 3GPP TSG SA WG3 Meeting #104-e                            	S3-212707

- e-meeting, 16 – 27 August 2021	                         revision S3-21abcd

- Source:	Huawei, HiSilicon, Nokia, Nokia Shanghai Bell

- Title:	General Conclusion on Generic Requirement for the Procedures for User Consent Revocation

- Document for:	Approval

Agenda Item:	5.14

# Decision/action requested

This contribution proposes a new conclusion in TR 33.867

# References

# Rational

The contribution proposes a new conclusion on how to handle revocation of user consent. The highlighted text in yellow is approved in S3-212123 in the last SA3 meeting.

# Detailed proposal

*************** Start of 1st Change ****************

- 8.5.D	Generic Requirement for the Procedures for User Consent Revocation

The following generic aspects are concluded for normative work:

- How to handle user consent revocation shall be specified in a generic way. This is to provide guideline for new use cases that need notification of revocation of user consent.

- If a service is invoked by NF service consumer directly, NF service provider subscribes UDM servicewhich can be notified that user consent is changed.

- If user consent is revoked, the NF service provider halts processing of data subject to the user consent.

- If there is an intermediate NF which is involved in service invocation, e.g. NWDAF, NEF, etc., the intermediate NF subscribes UDM service which can be notified that user consent is changed

- If user consent is revoked, the intermediate NF halts processing of data subject to the user consent, if available.

- If user consent is revoked, the intermediate NF notifies NF service consumers to halt processing of data subject to the user consent.

- Editor’s Note: How to delete data subject to the user consent is ffs.

*************** End of 1st Change ****************