3GPP TSG-SA3 Meeting #104-e 	S3-212913-r1

- e-meeting, 16 - 27 August 2021											Revision of S3-20xxxx

- Source:	Samsung

- Title:	[EDGE] Conclusion for KI#1

- Document for:	Approval

Agenda Item:	5.8

# 1	Decision/action requested

It is requested to approve the pCR to add conclusion for KI#1 of TR 33.839.

# 2	References

[1]	3GPP TR 33.839 Study on Security Aspects of Enhancement of Support for Edge Computing in 5GC.

# 3	Rationale

This pCR is proposes to add Conclusion for KI#1.

# 4	Detailed proposal

******Start of Change******

- 7	Conclusions

## 7.x	Conclusion for Key Issue #1

Authentication and authorization between EEC and EES:

- Solution #3 and Solution#17 is recommended as the baseline for the normative work to conclude that EEC and the EES establish a secure TLS connection using EES (server-side) certificate. For authentication of EEC, ID token by OpenID connect 1.0 is used. TLS provides integrity protection, replay protection, and confidentiality protection over the EDGE-1 interface. EEC produces the access token and ID token to the EES, over the EDGE-1 interface securely.

- Authorization of the EEC is based on the access token generated by the ECS for the EES service which is described in solution #3 and #17. It is recommended to use solution #3 and solution#17 as the baseline for the normative work.

******End of Change******