3GPP TSG-SA3 Meeting #104-e 	draft_S3-212963-r4

- e-meeting, 16 - 27 August 2021														Revision of S3-212963

- Source:	Xiaomi, CATT, Qualcomm Incorporated

- Title:	ProSe: Conclusion for Key Issue #13

- Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

SA3 is kindly asked to approve the proposed conclusion for key issue #13 of TR 33.847.

# 2	References

[1]	3GPP TS 23.304 v1.0.0		Proximity based Services (ProSe) in the 5G System (5GS)

[2]	3GPP TR 33.847 v0.6.0		Study on security aspects of enhancement for proximity based services in the 5G System (5GS)

# 3	Rationale

As per TS 23.304 [1], the ProSe Layer-2 Group ID can be pre-configured for the Application Layer Group ID provided by the application layer in the UE. In this case, the UE uses the ProSe Layer-2 Group ID as the Destination Layer-2 ID, hence there is no privacy issue in converting the Application Layer Group ID into a Destination Layer-2 ID.

As per TS 23.304 [1], when ProSe Layer-2 Group ID is not pre-configured for the Application Layer Group ID in the UE, the conversion from the Application Layer Group ID into a Destination Layer-2 ID is done by the UE. As the identifier is converted in the UE itself and not transmitted over any interface, it is unclear how linkability and traceability attacks can be performed against the UE. Therefore, it is recommended that the privacy protection of the ID conversion is left to UE implementation, which does not need to be standardized.

This pCR proposes to make a conclusion for key issues #13 in TR 33.847 [2].

# 4	Detailed proposal

*************** Start of the Change ****************

## 7.13	Key Issue #13: Security and privacy of groupcast communication

As per TS 23.304 [16], the ProSe Layer-2 Group ID can be pre-configured for the Application Layer Group ID provided by the application layer in the UE. In this case, the UE uses the ProSe Layer-2 Group ID as the Destination Layer-2 ID.

As per TS 23.304 [16], when ProSe Layer-2 Group ID is not pre-configured for the Application Layer Group ID in the UE, the conversion from the Application Layer Group ID into a Destination Layer-2 ID is done by the UE. Thus, it is not transmitted over any interface.

For protection of one-to-many communications between ProSe-enabled UEs, it is concluded that the principle security mechanisms defined in clause 6.2 of TS 33.303 [2] are reused for 5G ProSe public safety case scenarios. Further enhancements or adjustments can be discussed and added during normative phase.

NOTE: The agreement for normative work is on key common principles and not on all details of the solutions

*************** End of the Change ****************