3GPP TSG-SA3 Meeting #105-e 	S3-213896

- e-meeting, 8 - 19 November 2021

- Source:	Nokia, Nokia Shanghai Bell

- Title:	New Solution Using Attestation for Key Issue #13

- Document for:	Approval

Agenda Item:	5.3

# 1	Decision/action requested

It is requested to approve this contribution for a new solution in TR 33.848

# 2	References

[1]	3GPP TR 33.848: “Study on Security Impacts of Virtualisation”

# 3	Rationale

Key Issue 13 states that without a full attestation chain from the 3GPP function level to the hardware level, a 3GPP Network at the application layer is not able to verify the trustworthiness of VNFs or the NFVI.

Specifically, Key Issue 13 states that possession of a 3GPP level identity / certificate is not in itself a means to prove authenticity of a VNF, unless there is a full attestation chain back to hardware.

The solution tries to fill this gap by outlining how the provisioning of 3GPP level identities and corresponding cryptographic material can be bound to remote attestation.

# 4	Detailed proposal

It is proposed to add the following changes to TR 33.848:

****Start of 1st Change ****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	ETSI GS NFV 002: "Network Functions Virtualisation (NFV); Architectural Framework".

[3]	ETSI GS NFV-SEC 009: "Network Functions Virtualisation (NFV); NFV Security; Report on use cases and technical approaches for multi-layer host administration".

[4]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security"

[5]	ETSI GS NFV-SEC 001: "Network Functions Virtualisation (NFV); NFV Security; Problem Statement"

[6]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[7]	"Virtualization Technology: Cross-VM Cache Side Channel Attacks make it Vulnerable"; Shahzad and Litchfield 2015; https://arxiv.org/ftp/arxiv/papers/1606/1606.01356.pdf

[8]	"OpenStack"; https://www.openstack.org/

[9]	ETSI GR NFV-SEC 016: "Network Functions Virtualisation (NFV); Security; Report on location, timestamping of VNFs".

[10]	ETSI GS NFV-SEC 012: "Network Functions Virtualisation (NFV) Release 3; Security; System architecture specification for execution of sensitive NFV components".

[11]	ETSI GR NFV-SEC 011: " Network Functions Virtualisation (NFV); Security; Report on NFV LI Architecture".

[12]	ETSI GS NFV-SEC 013: "Network Functions Virtualisation (NFV) Release 3; Security;  Security Management and Monitoring specification".

[X]	IETF: RATS Working Group: Remote Attestation Procedures Architecture, https://www.ietf.org/archive/id/draft-ietf-rats-architecture-12.txt

[Y]	Intel Labs: Integrating Remote Attestation with Transport Layer Security, https://github.com/cloud-security-research/sgx-ra-tls/blob/master/whitepaper.pdf

****End of 1st Change****

****Start of 2nd Change ****

## 6.Y Solution Y: Solution Using Attestation for Key Issue 13

This solution addresses Key Issue 13: Attestation at 3GPP Function Level.

The solution proposes to bind the provisioning of 3GPP identities and corresponding cryptographic material to 3GPP NFs to the process of remote attestation.

### 6.Y.1	Introduction

3GPP has defined an own security framework on NF level, which is based on the Service Based Architecture.

A central element of the security is transport layer security based on X.509 certificates.

If a NF acting as a service consumer requests a service from another NF acting as a service producer, the consumer is authenticating the producer by means of verifying the producer's server certificate. Vice versa, the producer authorizes the consumer by means of verifying an OAuth 2.0 token included in the service request.

The consumer requests and receives the OAuth 2.0 token from the NRF. During the token request the NRF authenticates the NF requesting the token. The authentication is based on a X.509 certificates and a corresponding private key, which together are acting as the consumer's OAuth Client Id and Client Secret.

There are two possible approaches, how the concept of remote attestation could be linked to the existing security concept:

1. Remote Attestation is executed during each interaction between consumer and NRF and between consumer and producer.

2. Remote Attestation is executed during enrolment of client and server certificates to the NFs. Afterwards trust is based on these certificates and their corresponding private keys.

The proposed solution focuses on the 2nd approach, because the impact on the NFs is expected to be smaller.

This approach assumes that the NF can protect the private key linked to the certificate from any attack. That is the cryptographic material needs to reside within a trusted execution environment and must not be visible outside of the trusted execution environment at any time, not even during provisioning.

It is up to implementation and specific security requirements, if the NF is using a trusted execution environment based on enclave technologies for handing of the private key and certificate, or if the entire NF is executed in a trusted execution environment.

### 6.Y.2 Solution Details

The following description provides a high-level overview about a possible solution focussing on the concept and requirements. The individual steps are only indicative. A detailed specification is FFS or up to normative work.

Figure 6.Y.2-1: Combining provision of 3GPP identities and certificates to the process of remote attestation. In terms of an attestation architecture [12] the Network Function NF is acting as Attester, the Provisioning Server PS as the Relying Party, and the Attestation Server AS as the Verifier.

1.	NF is requesting secure channel from the provisioning server.

Depending on the exact specification of the protocol in this step the NF might also verify the identity of the provisioning server. For instance, if TLS is used as a secure channel, the NF could verify the server certificate of the provisioning server using a preconfigured root certificate.

2.	The provisioning server is replying with a request for evidence. The request includes a nonce, which will be used to ensure the freshness of the evidence provided in the subsequent steps.

3-	The NF collects claims and prepares the signed evidence, which includes the Nonce

The claims included in the evidence are for further study and might be out of scope of 3GPP. Claims in the evidence might include claims related to the NF Software and to the NF configuration, but also claims related to the Hardware and the Cloud Software stack. If NF is deployed in the form of several components, the evidence might also include claims from the individual components.

Furthermore, the evidence needs to include a claim, which binds the secure channel to the attestation process. For instance, an ephemeral public key, whose corresponding private key is only known to the NF and protected by the NF by means of a trusted execution environment, could be included in the evidence.

4.	The NF sends the evidence to the provisioning server

5.	The provisioning server requests verification of the evidence from the attestation server.

6.	The attestation server appraises the provided evidence using endorsements and refence values. The way, how this appraisal is executed and how the attestation server receives endorsements and reference values might be out of scope of 3GPP.

Note: In case of a negative outcome of the appraisal the attestation server might inform the management system about this incident. The management might take immediate action on this incident including identifying, isolating, and shutting down of the potentially malicious NF. Furthermore, the incident might cause a root cause analysis with the target of closing the security gap, which was causing the incident.

7.	The attestation server returns the result of the appraisal to the provisioning server

8.	Based on the attestation results and other policies the provisioning server decides whether the NF is eligible for provisioning or not.

9.	The establishment of the secure channel is completed.

Note: The establishment of the secure channel includes usage of the claim related to the channel, which was included in the evidence provided in step 3 from the NF to the provisioning server.

10.	NF and provisioning server use the secure channel to enrol necessary identities and certificates to the NF.

### 6.Y.3	Evaluation

This solution proposes linking the provisioning of 3GPP identities and certificates to the process of remote attestation. That is, the identities and certificates should only be provisioned to NFs, which could successfully provide evidence about the integrity of their software and hardware.

The solution outlines basic steps on a protocol level, which need to be fulfilled for linking remote attestation to provisioning. Most importantly, the protocol needs to ensure the freshness of the evidence provided by the NF and, secondly, the evidence needs to include a claim about a secret, which verifiably is only known to the NF and which is used to form the secure channel.

The solution does not provide a detailed protocol specification. Nevertheless, it is recommended that 3GPP is providing a specification of this protocol. This can be done either in a subsequent study or during normative work.

The benefit of a standardized protocol linking remote attestation and provisioning is not necessarily interoperability, but general availability of a secure and audited solution.

The standardization work could partly or fully make use of existing or currently developed solution approaches in the industry, like those related to the linkage of remote attestation and TLS [Y].

****End of 2nd Change****