3GPP TSG-SA3 Meeting #105-e 	draft_S3-214035-r2

- e-meeting, 8 - 19 November 2021														 Revision of S3-214035

- Source:	Huawei, HiSilicon

- Title:	Delete Editor's Note in EN-DC

- Document for:	Approval

Agenda Item:	4.13

# 1	Decision/action requested

Endorse the proposal

# 2	References

[1]	S3-213698 Living CR on 33.401 for UP IP for LTE

# 3	Rationale

The gNB does not need an indication on whether the UE support UP IP. Because all gNBs support UP IP, and if the UP IP policy is provided to the gNB, then it shall decide whether to activate the UP IP based on the UP IP policy.

# 4	Proposal

****************The first change****************

- E.2.3	Activation of encryption/decryption

The DRB offload procedure with activation of encryption/decryption and integrity protection follows the steps outlined on the Figure E.2.3-1.

Figure E.2.3-1. SeNB encryption/decryption activation

1.	The UE and the MeNB establish the RRC connection.

2.	The MeNB decides to offload the DRB(s) to the SeNB. The MeNB sends SeNB Addition Request to the SeNB over the X2-C to negotiate the available resources, configuration, and algorithms at the SeNB. The MeNB computes and delivers the S-KeNB to the SeNB as necessary. UE EPS security capability should also be sent to SeNB. The SeNB Addition Request message shall additionally include UP integrity protection policy (either the one received from other network entities or the locally configured one if no UP integrity protection policy is received from other network entities).

3.	The SeNB allocates the necessary resources and chooses the ciphering algorithm which has the highest priority from its configured list and is also present in the UE EPS security capability. If the UE supports user plane integrity protection, then the SeNB shall use the UP IP policy received from the MeNB together with the UE EPS security capabilities (i.e. bit EIA7) to determine whether to activate UP integrity protection. The SeNB shall activate UP integrity protection per DRB according to the UP integrity protection policy if it is received and shall indicate that to the UE.

4.	The SeNB sends SeNB Addition Request Acknowledge to the MeNB indicating availability of requested resources and the identifiers for the selected ciphering algorithm and integrity algorithm to serve the requested DRB for the UE.

5.	The MeNB sends the RRC Connection Reconfiguration Request to the UE instructing it to configure a new DRB for the SeNB. The MeNB shall include the SCG Counter parameter to indicate that the UE shall compute the S-KeNB for the SeNB, the KUPenc and the KUPint associated with the assigned bearer. The MeNB forwards the UE configuration parameters (which contains the algorithm identifier received from the SeNB in step 4) to the UE (see section E.2.4.3 for further details).

NOTE: Since the message is sent over the RRC connection between the MeNB and the UE, it is integrity protected using the KRRCint of the MeNB. Hence the SCG Counter cannot be tampered with, and the UE can assume that it is fresh.

6.	The UE accepts the RRC Connection Reconfiguration Command and shall compute the S-KeNB for the SeNB. The UE shall also compute the KUPenc and the KUPint for the associated assigned DRB on the SeNB. The UE sends the RRC Reconfiguration Complete to the MeNB. The UE activates encryption/decryption and integrity protection once S-KeNB and KUPenc are derived.

7. MeNB sends SeNB Reconfiguration Complete to the SeNB over the X2-C to inform SeNB configuration result. On receipt of this message, SeNB may activate encryption/decryption and integrity protection with UE. If SeNB does not activate encryption/decryption or integrity protection with the UE at this stage, SeNB shall activate encryption/decryption and integrity protection upon receiving the Random Access request from the UE.

************The end of first change************

****************The second change****************

- E.3.3	Activation of encryption/decryption/integrity protection of DRBs and encryption/decryption/integrity protection of SRB

The dual connectivity procedure with activation of encryption/decryption and integrity protection of Split and/or Non-Split SgNB terminated DRB(s) (i.e. a DRB for which PDCP is located in the SgNB)  and/or activation of encryption/decryption and integrity protection of an SgNB terminated SRB (i.e. a SRB for which PDCP is located in the SgNB)  follows the steps outlined on the Figure E.3.3-1.

Figure E.3.3-1. SgNB encryption/decryption and integrity protection activation

1.	The UE and the MeNB establish the RRC connection.

2.	Before the MeNB decides to use dual connectivity for some DRB(s) and/or an SRB with the SgNB, the MeNB shall check whether the UE has NR capability and is authorized to access NR. The MeNB sends SgNB Addition Request to the SgNB over the X2-C to negotiate the available resources, configuration, and algorithms at the SgNB. . The MeNB computes and delivers the S-KgNB to the SgNB if a new key is needed. The UE NR security capability shall also be sent to SgNB. The SgNB Addition Request message shall additionally include UP integrity protection policy (either the one received from other network entities or the locally configured one if no UP integrity protection policy is received from other network entities) and an indication that the UE supports UP integrity protection with EPS.

NOTE 1: Void.

NOTE 2: Void.

3.	The SgNB allocates the necessary resources and chooses the ciphering algorithm and integrity algorithms for the DRB(s) and SRB , if an SRB is to be established, which has the highest priority from its configured list and is also present in the UE NR security capability. If a new S-KgNB was delivered to the SgNB, then the SgNB calculates KSgNB-UP-int (if needed) and KSgNB-UP-enc as well as KSgNB-RRC-int and KSgNB-RRC-enc if an SRB is to be established. If the UE supports user plane integrity protection with EPS based on the received indication, then the SgNB shall use the UP IP policy received from the MeNB to determine whether to activate UP integrity protection. The SgNB shall activate UP integrity protection per DRB according to the UP integrity protection policy if it is received and shall indicate that to the UE. If the SgNB does not receive the UP IP policy and the indication, then the SgNB does shall not activate UP IP.

4.	The SgNB sends SgNB Addition Request Acknowledge to the MeNB indicating availability of requested resources and the identifiers for the selected algorithm(s) to serve the requested DRBs and/or SRB for the UE.

5.	The MeNB sends the RRC Connection Reconfiguration Request to the UE instructing it to configure the new DRBs and/or SRB for the SgNB. The MeNB shall include the SCG Counter parameter to indicate that the UE shall compute the S-KgNB for the SgNB if a new key is needed. The MeNB forwards the UE configuration parameters (which contains the algorithm identifier(s) and UP integrity indication received from the SgNB in step 4) to the UE (see section E.3.4.3 for further details).

NOTE 3: Since the message is sent over the RRC connection between the MeNB and the UE, it is integrity protected using the KRRCint of the MeNB. Hence the SCG Counter cannot be tampered with, and the UE can assume that it is fresh.

6.	The UE accepts the RRC Connection Reconfiguration Command. The UE shall compute the S-KgNB for the SgNB if an SCG Counter parameter was included. The UE shall also compute KSgNB-UP-enc and and KSgNB-UP-int (if needed) as well as KSgNB-RRC-int and KSgNB-RRC-enc for the associated assigned DRBs and/or SRB. The UE sends the RRC Reconfiguration Complete to the MeNB. The UE activates the chosen encryption/decryption and integrity protection at this point.

7. MeNB sends SgNB Reconfiguration Complete to the SgNB over the X2-C to inform the SgNB of the configuration result. On receipt of this message, SgNB may activate the chosen encryption/decryption and integrity protection with UE. If SgNB does not activate encryption/decryption and integrity protection with the UE at this stage, SgNB shall activate encryption/decryption and integrity protection upon receiving the Random Access request from the UE.

************The end of secondchange************

****************The third change****************

- E.3.10.2	Creating the mapped UE NR security capabilities

The MeNB that does not have the UE NR security capabilities shall create them as follow:

-	Set the support of NEA0, 128-NEA1, 128-NEA2, 128-NEA3, 128-NIA1, 128-NIA2, 128-NIA3 to the same as EEA0, 128-EEA1, 128-EEA2, 128-EEA3, 128-EIA1, 128-EIA2, 128-EIA3 respectively; and

-	Set the rest of the bits to 0.

This mapping of E-UTRAN security algorithms support to NR security algorithms support means that for the purposes of dual connectivity to SgNB, the UE shall have the same support for 128-NEA1 as 128-EEA1, 128-NEA2 as 128-EEA2, 128-NEA3 as 128-EEA3, 128-NIA1 as 128-EIA1, 128-NIA2 as 128-EIA2 and 128-NIA3 as 128-EIA3.

**************The end****************