3GPP TSG-SA3 Meeting #105-e 	draft_S3-214041-r1

- e-meeting, 08 - 19 November 2021											Revision of S3-20xxxx

- Source:	Huawei, HiSilicon

- Title:	resolving the EN on Kausf desych and key lifetime

- Document for:	Approval, Information, Discussion

Agenda Item:	5.7

# 1	Decision/action requested

This contribution proposes to resolve the EN on kausf desych and key lifetime.

# 2	References

[1]	3GPP TR 33.847 "Study on Security Aspects of Enhancement for Proximity Based Services in 5GS".

# 3	Rationale

This contribution proposes to resolve the following two Editor Notes:

Editor’s Note: Kausf desync should be further studied.

Editor’s Note: How to determine the key lifetime is FFS.

For the issue of Kausf desync, it has defined only one KAUSF is maintained in the AUSF and in the UE in TS33.501.  The AUSF stores only the latest KAUSF upon successful authentication procedure and deletes any stored old KAUSF. UE stores only the latest KAUSF upon successful authentication procedure and deletes any stored old KAUSF.  Therefore, the existing mechanism can be used to keep kausf synchronized.

For the key lifetime, there is already a NOTE said “The key lifetime issue will further align with the other normative work to address key lifetime”, we can delete the EN.

# 4	Detailed proposal

*************** Start of 1st Change ****************

## 6.39	Solution #39: Key management in UE-to-Network Relay based on shared key generated during primary authentication

### 6.39.1	Introduction

This solution addresses the KI #3, KI#4 and KI#9. This solution provides a mechanism to set up PC5 link security between a remote UE and UE-to-network relay based on primary authentication.

### 6.39.2	Solution details

#### 6.39.2.1	Procedure

The procedure for key management in UE-to-Network relay shared key generated using primary authentication is depicted in Figure 6.15.2-1.

1.	During the primary authentication procedure, the AUSF interacts with the UDM in order to fetch authentication information such as subscription credentials (e.g. AKA Authentication vectors) and the authentication method using the Nudm_UEAuthentication_Get Request service operation.

2.	In the response, the UDM may also indicate to the AUSF whether ProSe Key Identifier (P-KID) needs to be generated for the UE. If the ProSe Indicator is included, the UDM shall also include the RID of the UE.

3. 	If the AUSF receives the ProSe indication from the UDM, the AUSF shall store the KAUSF and generate the P-KID from KAUSF after the primary authentication procedure is successfully completed.

P-KID is in NAI format as specified in clause 2.2 of IETF RFC 7542 [6], i.e. username@realm. The username part includes the Routing Identifier and the P-TID (ProSe Temporary UE Identifier), and the realm part includes Home Network Identifier.

The P-TID is derived from KAUSF as specified in 6.39.2.2.

4. AUSF sends the P-KID via Nudm_UEAuthenticationResultConfirmation request to UDM, the UDM stores the P-KID with AUSF instance ID and sends Nudm_UEAuthenticationResultConfirmation response to the AUSF.

5. Authorization and provisioning are performed for the Remote UE.

6. During the Registration procedure, authorization and provisioning are performed for the ProSe UE-to-NW relay.

7. The Remote UE discovers the UE-to-network Relay using either model A or model B discovery.

8. The Remote UE derives P-KID using the same method as the remote AUSF.

9. The Remote UE sends a Direct Communication Request. The Direct Communication Request contains the Relay Service Code, P-KID and Nonce1.

10. The UE-to-Network relay triggers the PC5 root key (i.e PC5 root key for communicating with the remote UE) retrieval procedure. The Relay sends NAS remote Key request with P-KID, Relay service code, Nonce 1.

11. The relay AMF that serves the relay UE first check whether the relay UE is authorized to be a relay. If authorized, The AMF discovers the UDM based on the P-KID, the relay AMF sends Nudm_AUSFID_Get request with P-KID to the UDM.

12. UDM response to relay AMF via Nudm_AUSFID_Get with AUSF instance ID of the AUSF serving remote UE.

13. AMF further sends Nausf_ProSe_RemoteKey request with P-KID, Relay service code, Nonce 1 to the remote AUSF.

14. The remote AUSF generates KR freshness parameter and derives KR using at least Kausf, relay service code, KR freshness parameter and Nonce 1 as input.

15. The remote AUSF sends Nausf_ProSe_RemoteKey response with KR, KR freshness parameter to the relay AMF.

16. The relay AMF sends NAS remote Key response KR, KR freshness parameter to the UE-to-network relay.

NOTE: This solution requires integrity and confidentiality protected NAS messages.

17. The UE-to-network relay sends a Direct Security Mode Command message to the Remote UE, KR freshness parameter and Nonce 2 are included in the message.

18. The Remote UE derives the KR key in the same way as the remote AUSF in step 14. The DSMC message is integrity and confidentiality protected as in TS 33.536[8].

NOTE: If the normative key lifetime related solution exists, the key lifetime issue will further align with the normative work to address key lifetime

Editor’s Note: How to protect privacy of P-TID is FFS.

*************** End of 1st Change ****************