- 3GPP TSG-SA3 Meeting #105e 	S3-214067-r7

- e-meeting, 8 - 19 November 2021

- Source:	Huawei, HiSilicon

- Title:	EC: Authentication and Authorization between EEC and ECS

- Document for:	Approval

Agenda Item:	4.18

# 1	Decision/action requested

This contribution proposes add the Authentication and Authorization between EEC and ECS.

# 2	References

[1] TS 33.558 v020

# 3	Rationale

It is proposed to add the Authentication and Authorization between EEC and ECS according to the conclusion of key issue #1 of TR 33.839 clause 7.1.

# 4	Detailed proposal

It is suggested to approve the following changes.

***************************Start of change ****************************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[3]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[4]	3GPP TS 33.187: "Security aspects of Machine-Type Communications (MTC) and other mobile data applications communications enhancements".

[xx]	3GPP TS 23.558: "Architecture for enabling Edge Applications."

[yy]	3GPP TS 33.535: "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)."

[zz]	3GPP TS 33.222: "Generic Authentication Architecture (GAA); Access to network application functions using Hypertext Transfer Protocol over Transport Layer Security (HTTPS)".

***************************End of change****************************

## 6.2	Authentication and Authorization between EEC and ECS

6.2.1 General

The detailed service procedures between EEC and ECS are described in the TS 23.558 [xx]. There are two options that can be used for the authentication and authorization between EEC and ECS, i.e. TLS with AKMA [yy], TLS with GBA [zz]. Those options are described in the following clause.

### 6.2.2 Procedure for the Authentication and Authorization between EEC and ECS

Pre-requisite:

- If the TLS with AKMA shared key is used, the AKMA mechanism is assumed to be supported by the EEC and ECS.

- If the TLS with GBA shared key is used, the GBA mechanism isassumed to be supported by the EEC and ECS.

- The ECS optionally supports the GBA mechanism or AKMA mechanism.

Editor’s Note: TLS server authentication in case the configuration of the EEC doesn't depend on the UE identity is FFS

Figure 8.3.3.2.2-1: Service provisioning – Request/Response

1. TLS shall be used between EEC and ECS. Two options are applicable for the TLS establishment.

Option A:

The mutual authenticated TLS with shared AKMA key is based on the mechanism defined in TS 33.535 [yy].

Option B: Alternatively, the mutual authenticated TLS with shared GBA key is based on the mechanism defined in TS 33.222 [zz].

2. The EEC sends the service request to the ECS, which may include the GPSI.

Editor’s Note: It is FFS what other paramers are included in serveice request message.

Editor’s Note: It is FFS how to address the key collision issue when there is multiple EECs in one UE.

3. The ECS shall authorize the EEC by its local authorization policy.

Editor’s Note: It is FFS whether IP address translation is required for GPSI verification and if it is required then related steps are FFS.

If the authorization successes, the ECS processes the request.

The ECS may generate an EES service token for the EES service authorization, and send the EES service token to the EEC. The claims of the EES service token shall include the ECS ID (issuer), EEC ID (subject), expected EES service name(s) (Scope), expiration time (expiration).

Editor’s Note: The definition of ECS ID is FFS.

Editor’s Note: It is FFS whether multiple EES identities are included in the token.

4. The ECS sends the service response back the EEC, which may include the EES service token and a capability indication, where the capability indication indicates the EES support to use AKMA or GBA.

***************************End of change****************************