- 3GPP TSG-SA3 Meeting #105e 	draft_S3-214069-r1

- e-meeting, 8 - 19 November 2021

- Source:	Huawei, HiSilicon

- Title:	EC: Authentication and Authorization in EES capability exposure for non-CAPIF scenario

- Document for:	Approval

Agenda Item:	4.18

# 1	Decision/action requested

This contribution proposes to add the Authentication and Authorization in EES capability exposure for non-CAPIF scenario.

# 2	References

[1] TR 33.558 v020

# 3	Rationale

It is proposed to clarify authentication and Authorization in EES capability exposure for non-CAPIF scenario.

# 4	Detailed proposal

It is suggested to approve the following changes.

***************************Start of change ****************************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[3]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[4]	3GPP TS 33.187: "Security aspects of Machine-Type Communications (MTC) and other mobile data applications communications enhancements".

[xx]	IETF RFC 5246: "The Transport Layer Security (TLS) Protocol Version 1.2"..

[yy]	IETF RFC 8446: “The Transport Layer Security (TLS) Protocol Version 1.3".

[zz]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

***************************Next change****************************

## 6.5	Authentication and Authorization in EES capability exposure

According to clause 8.7.3 of TS 23.558 [5], the EES may re-expose the network capabilities of the 3GPP core network to the EAS(s) as per the CAPIF architecture specified in 3GPP TS 23.222 [6]. If the CAPIF architecture is used, the CAPIF functional security model specified in TS 33.122 [7] shall be used for Authentication and authorization in EES capability exposure.

If CAPIF is not used, mutual authentication with TLS certificates using TLS defined in RFC 5246[xx] and RFC 8446 [yy] shall be used. The TLS and certificates shall follow the profiles defined in TS 33.210 [2] and TS 33.310 [zz], and the authorization is based on local authorization policy at the EES.

NOTE: Adaptations of the security protocol dependent on the lower layer protocols could be necessary, e.g. HTTPS instead of TLS if HTTP is used.

***************************End of change****************************