3GPP TS 33.520 V0.3.0 (2021-11)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Security Assurance Specification for Non-3GPP InterWorking Function (N3IWF)

(Release 17 )

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

SCAS, 5G,gNodeB,network product class, security

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions of terms and abbreviations	5

3.1	Terms	5

3.2	Abbreviations	6

4	N3IWF-specific security requirements and related test cases	6

4.1	Introduction	6

4.2 	N3IWF-specific security functional adaptations of requirements and related test cases	6

4.2.1	Introduction	6

4.2.2	Security functional requirements on the N3IWF deriving from 3GPP specifications and related test cases	6

4.2.3	Technical Baseline	7

4.2.4	Operating Systems	7

4.2.5	Web Servers	8

4.2.6	Network Devices	8

4.3	N3IWF-specific adaptations of hardening requirements and related test cases.	8

4.3.1	Introduction	8

4.3.2	Technical Baseline	8

4.3.3	Operating Systems	8

4.3.4	Web Servers	8

4.3.5	Network Devices	8

4.4	N3IWF-specific adaptations of basic vulnerability testing requirements and related test cases	8

Check whether the N3IWF sends the EAP-Identity Request message	9

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document contains objectives, requirements and test cases that are specific to the Non-3GPP InterWorking Function (N3IWF) network product class. It refers to the Catalogue of General Security Assurance Requirements and formulates specific adaptions of the requirements and test cases given there, as well as specifying requirements and test cases unique to the N3IWF network product class.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]  	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]  	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[xx]	3GPP TR 33.926: "Security Assurance Specification (SCAS) threats and critical assets in 3GPP network product classes".

[yy]	3GPP TS 33.117: "Catalogue of general security assurance requirements".

# 3	Definitions of terms and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

# 4	N3IWF-specific security requirements and related test cases

## 4.1	Introduction

N3IWF specific security requirements include both requirements derived from N3IWF-specific security functional requirements in relevant specifications as well as security requirements introduced in the present document derived from the threats specific to N3IWF network product classes as described in TR 33.926 [xx].

## 4.2 	N3IWF-specific security functional adaptations of requirements and related test cases

### 4.2.1	Introduction

Present clause contains N3IWF-specific security functional adaptations of requirements and related test cases.

### 4.2.2	Security functional requirements on the N3IWF deriving from 3GPP specifications and related test cases

#### 4.2.2.1	Security functional requirements on the N3IWF deriving from 3GPP specifications – TS 33.501 [2]

##### 4.2.2.1.1	Handling of EAP-Identity Request message

Requirement Name: Handling of EAP-Identity Request message

Requirement Reference: TS 33.501 [2], clause 7.2.1

Requirement Description: "The N3IWF shall refrain from sending an EAP-Identity request. The UE may ignore an EAP Identity request or respond with the SUCI it sent in the Registration Request." as specified in TS 33.501 [2], clause 7.2.1.

Threat Reference:  TBD

Test Name: TC_N3IWF_REFRAIN_SEND_IDENTITY_REQUEST

Purpose:

Verify that the N3IWF will not send an EAP-Identity request message, and if the N3IWF sends EAP-Identity Request message, whether the N3IWF will conintue to proceed the EAP-5G procedure.

Pre-Conditions:

-	The AMF network product shall be connected in emulated/real network environments. UE may be simulated.

- 	A document describes whether the N3IWF will send an EAP-Identity Request message.

Execution Steps

1. The UE initiates IKE_SA_INIT procedure without UE identity.

2. The N3IWF responses an EAP message

Expected Results:

If the EAP message is EAP-5G start, then it is ok.

If the EAP message is EAP-Identity Request meesage, then check whether the N3IWF sends a N2 msg to the AMF, and whether the procedure in 7.2 of TS 33.501[2] runs successfully.

Expected format of evidence:

Save the logs and the communication flow in a .pcap file.

4.2.3	Technical Baseline

#### 4.2.3.1	Introduction

The present clause provides baseline technical requirements.

#### 4.2.3.2	Protecting data and information

##### 4.2.3.2.1	Protecting data and information – general

There are no N3IWF-specific additions to clause 4.2.3.2.1 of TS 33.117 [yy].

##### 4.2.3.2.2	Protecting data and information – unauthorized viewing

There are no N3IWF-specific additions to clause 4.2.3.2.2 of TS 33.117 [yy].

##### 4.2.3.2.3	Protecting data and information in storage

There are no N3IWF-specific additions to clause 4.2.3.2.3 of TS 33.117 [yy].

##### 4.2.3.2.4	Protecting data and information in transfer

There are no N3IWF-specific additions to clause 4.2.3.2.4 of TS 33.117 [yy].

##### 4.2.3.2.5	Logging access to personal data

There are no N3IWF-specific additions to clause 4.2.3.2.5 of TS 33.117 [yy].

#### 4.2.3.3	Protecting availability and integrity

There are no N3IWF-specific additions to clause 4.2.3.3 of TS 33.117 [yy].

#### 4.2.3.4	Authentication and authorization

There are no N3IWF-specific additions to clause 4.2.3.4 of TS 33.117 [yy].

#### 4.2.3.5	Protecting sessions

There are no N3IWF-specific additions to clause 4.2.3.5 of TS 33.117 [yy].

#### 4.2.3.6	Logging

There are no N3IWF-specific additions to clause 4.2.3.61 of TS 33.117 [yy].

### 4.2.4	Operating Systems

There are no N3IWF-specific additions to clause 4.2.4 of TS 33.117 [yy].

### 4.2.5	Web Servers

There are no N3IWF-specific additions to clause 4.2.5 of TS 33.117 [yy].

### 4.2.6	Network Devices

There are no N3IWF-specific additions to clause 4.2.6 of TS 33.117 [yy].

### 4.3	N3IWF-specific adaptations of hardening requirements and related test cases.

### 4.3.1	Introduction

The present clause contains N3IWF-specific adaptations of hardening requirements and related test cases.

### 4.3.2	Technical Baseline

There are no N3IWF-specific additions to clause 4.3.2 of TS 33.117 [yy].

### 4.3.3	Operating Systems

There are no N3IWF-specific additions to clause 4.3.3 of TS 33.117 [yy].

### 4.3.4	Web Servers

There are no N3IWF-specific additions to clause 4.3.4 of TS 33.117 [yy].

### 4.3.5	Network Devices

There are no N3IWF-specific additions to clause 4.3.5 of TS 33.117 [yy].

### 4.4	N3IWF-specific adaptations of basic vulnerability testing requirements and related test cases

There are no N3IWF-specific additions to clause 4.4 of TS 33.117 [yy].

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-05 | SA3#99e | S3-200985 |  |  |  | skeleton | 0.0.0 |
| 2020-05 | SA3#99e | S3-201434 |  |  |  | Updated skeleton | 0.0.1 |
| 2020-05 | SA3#99e | S3-201434 |  |  |  | Skeleton of SCAS_5G_N3IWF | 0.1.0 |
| 2021-02 | SA3#102-e | S3-210802 |  |  |  | - Check whether the N3IWF sends the EAP-Identity Request message | 0.2.0 |
| 2021-02 | SA3#105-e | S3-214537 |  |  |  | - Implementing S3-214114. | 0.3.0 |