3GPP TSG-SA WG3 Meeting #106-e	S3-220068-r5

e-meeting, 14 – 25 February 2022                                                           revision of S3-20xabc

Source:	InterDigital

Title:	TR 33.870 – Informative Annex Y

Document for:	Approval

Agenda Item:	5.14 - Study of privacy of identifiers over radio access

Work Item / Release:	n/a

This contribution proposes Informative Annex Y for TR 33.870, Study of privacy of identifiers over radio access.

# 1	Decision/action requested

It is requested to approve this proposed content for Informative Annex Y for TR 33.870, Study of privacy of identifiers over radio access

# 2	References

[1]	SP-211364	New WID for Study of privacy of identifiers over radio access

# 3	Rationale

SP-211364 [1] in section 3, Justification, necessitates providing, “…examples of identifiers that require a comprehensive privacy evaluation” and lists SUPI, SUCI, S-NSSAI, 5G-GUTI as examples. To systematically study the potential privacy impacts/risks of 3GPP identifiers it is proposed to list them.

This contribution proposes Informative Annex Y for TR 33.870, Study of privacy of identifiers over radio access.

# 4	Detailed proposal

START OF CHANGE 1

Annex <Y> (informative):
List of 3GPP identifiers with potential privacy sensitivity.

The following table provides a non-exhaustive list of 3GPP identifiers and parameters transmitted over the air. These identities are provided for information only (e.g. inclusion neither suggests that the identity is in scope of study nor that there is a privacy issue with that identity.

| No | Name of 3GPP Identifier | Description | Specified in 3GPP document |
| --- | --- | --- | --- |
| 1 | SUCI (concealing SUPI in NAI format) | SUbscription Concealed Identifier | TS 23.003 [7], TS 23.501 [2] |
| 2 | S-NSSAI | Single Network Slice Selection Assistance Information | TS 23.003 [7], TS 23.501 [2] |
| 3 | 5G-GUTI | 5G Globally Unique Temporary Identifier 5G-GUTI provides an unambiguous identification of the UE that does not reveal the UE or the user's permanent identity. 5G-GUTI has two main components: - one that identifies the AMF(s) which allocated the 5G-GUTI; and - one that uniquely identifies the UE within the AMF(s). | TS 23.003 [7] |
| 4 | CAG Identifier | A Closed Access Group (CAG) within a PLMN is uniquely identified by a CAG-Identifier | TS 23.003 [7], TS 23.501 [2] |
| 5 | C-RNTI | Cell Radio Network Temporary Identifier C-RNTI is a unique identifier dedicated to a particular UE and used for identifying RRC Connection and scheduling. C-RNTI can be reallocated when a UE accesses a new cell with the cell update procedure. | TS 38.300 [8], TS 38.321 [9] |

END OF CHANGE 1