3GPP TSG-SA3 Meeting #106e	S3-220180-r4

- e-meeting, 14 – 25 February 2022  							Merger of S3-220373 Revision of S3-22xxxx

- Source:	Huawei; HiSilicon; Ericsson

- Title:	Security procedures for L2 UE-to-Network relay

- Document for:	Approval

Agenda Item:	4.13

# 1	Decision/action requested

Kindly Request this contribution can be accepted by the group

# 2	References

N/A

# 3	Rationale

As specified in clause 6.2, the PC5 direct communication security for relay services is specified in clause 6.3. However in clause 6.3.4 the PC5 connection security between the remote UE and relay UE is circular referred back to clause 6.2. Moreover, the PC5 connection security between the remote UE and relay UE cannot be covered by the procedures in clause 6.2 as the key establishment needs help from the network side in the UE-to-Network scenario. This contribution aims to address the above issues.

Merger of S3-220373.

# 4	Detailed proposal

**** 1st change****

### 6.3.4	Security for 5G ProSe Communication via 5G ProSe Layer-2 UE-to-Network Relay

Connection establishment for 5G ProSe Communication via 5G ProSe Layer-2 UE-to-Network Relay is specified in clause 6.5.2.2 of TS 23.304 [2]. During the connection establishment, the Remote UE and NG-RAN node shall establish AS security as specified in TS 33.501 [3].

The remote UE and the relay UE shall establish security for PC5 connection using either User Plane based solution as specified in clause 6.3.3.2 or Control Plane based solution as specified in clause 6.3.3.3.

**** End of changes****