3GPP TSG-SA3 Meeting #106-e 	S3- 220200

- e-meeting, 14 - 25 February 2022

- Source:	Xiaomi

- Title:	eNS2: Key Issue #2 update

- Document for:	Approval

- Agenda Item:	5.12

# 1	Decision/action requested

SA3 is kindly asked to approve the proposed update to key issue #2 in TR 33.874.

# 2	References

- 3GPP TR 33.874 TR V0.5.0 Technical Specification Group Services and System Aspects; Study on enhanced security for Network Slicing Phase 2 (Release 17)

- S2-2107062	Reply LS on NSAC procedure

# 3	Rationale

In one of the issues in key issue #2 of the current version of TR 33.874 [1], it is stated that the UE will go through NSSAA procedure twice if the UE is rejected due to NSAC. However, according to the reply LS from SA2 [2], the NSSAA status is kept in the UE context in AMF (also transferred between AMFs), i.e. the AMF may avoid NSSAA when NSSAA has been successful for an S-NSSAI. If none of the S-NSSAIs of Requested can be registered due to NSAC, then AMF may decide to include Default Subscribed S-NSSAIs in the Allowed NSSAI, if they are not subject for NSAC. Therefore, the UE does not need to go through the same NSSAA procedure again when NSSAA is successful but the quota has been reached.

Regarding the issue on EAC, according to LS from SA2 [2], EAC mode can be activated by the operator based on the threshold which is also defined by the operator. This implies that proper setting of the threshold by the operator can avoid the risk of burst in the number of UEs attempting to register under EAC mode inactive. Therefore, there is no issue with EAC procedure upon a burst of UE registration.

This pCR proposes to update key issue #2 in the current version of TR 33.874 [1] based on the above analysis.

- 4	Detailed proposal

*************** Start of the 1st Change ****************

## 5.2	Key Issue 2: DoS to NSAC procedure

### 5.2.1	Key issue details

A new Network Slice Admission Control (NSAC) procedure has been introduced in TS23.501 [2] and TS23.502 [3], where the number of registered UEs is monitored for a network slice (i.e. S-NSSAI) and a UE will be rejected to access if the number of UE registered in the requested S-NSSAI has reached its quota.

- In the current NSAC procedure, the number of registered UE in an S-NSSAI is updated independently from other S-NSSAIs during the registration procedure. In other words, the granularity level at registration is S-NSSAI. However, it is not the case in the de-registration procedure. The numbers are only updated when the UE exits from all network slices, i.e. de-registered. Since a UE may access multiple slices, e.g. eight, the UE would still be counted against quota usage of ALL S-NSSAIs even the UE is not using some or most of slices (“idly occupied” by the UE). This may lead to the quota reached fast which does not reflect the real usage of a slice. Other legitimate UEs will suffer from DoS – “dog in the mager”. It is notable that an attacker can use legitimate UEs to launch such attacks.

- The Early Admission Control (EAC) mode has been introduced where the admission control can be inactive if the number of UE bellows a preconfigured threshold. This may pose a security risk that exceeds the slice quota when a sudden increase in the slice registration requests, maliciously or accidentally.

- *************** Start of the 2nd Change ****************

# 7	Conclusions

Editor’s Note: This clause contains the agreed conclusions that will form the basis for any normative work.

## 7.X	Conclusions for KI#2

Key issue #2 does not need any normative work.

The issue related to slice usage will not be studied in Rel 17.

It is up to operators to set a proper threshold to activate EAC mode, which can avoid the risk of bursting in the number of UEs attempting to register under EAC mode inactive. The threshold setting is a matter of operator deployment. So in SA3 we may not need to specify anything.

*************** End of the Change ****************