- 3GPP TSG-SA3 Meeting #106-e	draft_S3-220209-r2

- e-meeting, 14 – 25 February 2022	revision of S3-220209

- Source:	CATT

- Title:	pCR to TS33.503 Clause 4.2 Add new reference point between PKMF and UDM

- Document for:	Approval

Agenda Item:	4.13

# 1	Decision/action requested

Approve the pCR below

# 2		Rationale

The Npc4* reference point is defined for the interaction between 5G PKMF and UDM, e.g. de-concealing SUCI or obtaining AVs from the UDM.

# 3	Detailed proposal

#### *** BEGIN CHANGES ***

## 4.2	Reference points and functional entities

Architectural reference model is specified in clause 4.2.1, 4.2.2, 4.2.3, and 4.2.7 of TS 23.304 [2].

In addition to the reference points are specified in clause 4.2.5 of TS 23.304 [2], the 5G Prose architectural reference model shall support the following reference points:

PC8:	The reference point between the UE and the 5G ProSe Key Management Function (5G PKMF). PC8 relies on 5GC user plane for transport (i.e. an "over IP" reference point). It is used to transport security material to UEs for 5G ProSe UE-to-Network Relay Communication.

Npc9:	The reference point between the 5G PKMF of the 5G ProSe Layer-3 Remote UE and the 5G PKMF of the 5G ProSe Layer-3 UE-to-Network Relay. It is used to transport security material between two PKMFs.

Npcxx:	The reference point between the UDM and 5G PKMF. It is used to de-conceal SUCI to gain SUPI, obtain an Authentication Vector (AV) for a UE, or request relay service authorization information from the UDM.

#### *** END OF CHANGES ***