3GPP TSG-SA3 Meeting #106-e 	draft_S3-220244-r2

- e-meeting, 14 - 25 February 2022

- Source:	Ericsson, Nokia, Nokia Shanghai Bell

- Title:	Multiple PLMN-IDs in the SEPP interconnect certificate profile

- Document for:	Approval

Agenda Item:	4.19

# 1	Decision/action requested

Approve the proposed changes to the draftCR S3-220243.

# 2	References

[1]	S3-220243 "Clarification on the certificate profile for SCP and SEPP", re-submission of S3-214440 that was approved at SA3#105-e

# 3	Rationale

The approved draft-CR [1] with the certificate profile for the SCP and SEPP contains an Editor's Note in the SEPP interconnect certificate profile:

Editor's Note: How to consider multiple PLMN-IDs in the subject alt name is ffs.

This contribution attempts to resolve the Editor's Note.

# 4	Detailed proposal

- *** BEGIN CHANGES ***

- 6.1.3c.y.3 SEPP interconnect certificate profile

The same requirements to the NF certificate profile as listed in clause 6.1.3c.3 apply, except for the following requirements that are not applicable to the SEPP interconnect certificate profile:

-	The following requirement does not need to be supported: "subjectAltName should (in TLS server certificates) contain URI-IDs with the HTTPS URI(s) for the apiRoot of a Network Function producer instance for the NF service API(s) that it provides; using wildcard URIs should be avoided";

-	The following requirement does not need to be supported: "subjectAltName should (in TLS server certificates) contain URI-IDs with the HTTPS URI(s) for the apiRoot of a Network Function consumer instance for the NF service callback URI(s) that it provides; using wildcard URIs should be avoided".

The following additions to the NF certificate profile shall hold for the SEPP interconnect certificate profile:

-	subjectAltName shall (in TLS client and server certificates) contain the PLMN-ID(s) or SNPN-ID(s) as DNS-ID(s) (that is, using dNSName subjectAltName) of the same PLMN or same SNPN where the SEPP is located, using the PlmnId as a string according to clause 5.4.4.3 of TS 29.571 [57] or PlmnIdNid according to clause 5.4.4.33 of TS 29.571 [57].

- *** END CHANGES ***