3GPP TSG-SA3 Meeting #106-e 	S3-220278-r1

- e-meeting, 14 - 25 February 2022

- Source:	Xiaomi

- Title:	33.503: PC5 Security Policy Privisioned by PKMF

- Document for:	Approval

Agenda Item:	4.13

# 1	Decision/action requested

SA3 is kindly asked to approve the proposed text for PC5 security policies privisioned by PKMF in TS 33.503.

# 2	References

[1]	3GPP TS 33.503 v0.2.0		Security Aspects of Proximity based Services (ProSe) in the 5G System (5GS)

[2]	3GPP TS 23.304 v17.1.1	Proximity based Services (ProSe) in the 5G System (5GS)

# 3	Rationale

In TS 33.503 [1] clause 6.3.3.1, there is a security requirement on 5G PKMF for provisioning PC5 security policies to the UE as follows:

-	For L3 relay security established over user plane, the 5G PKMF shall be able to provision the PC5 security policies to the UE per ProSe relay service, during security materials provisioning procedure defined in clause 6.3.3.2.

However, in current security materials provisioning procedure over user plane in TS 33.503 [1] clause 6.3.3.2, the PC5 security policy provisioning by the 5G PKMF is not defined. It is assumed that a list of PC5 security policies of the ProSe UE-to-Network relay services is configured in the PKMF, from which the PC5 security policies of the specific relay service requested by the Remote UE is provisioned by the PKMF to the UE during the security materials provisioning procedure based on the RSC sent by the Remote UE.

In addition, there is a note in TS 33.503 [1] clause 6.3.3.2 stating that:

NOTE 3: The PKMF may be locally configured with the UE’s authorization information. Otherwise, the PKMF interacts with the UDM to retrieve the UE’s authorization information

However, according to 23.304 [2] clause 5.1.4, UE authorization and provisioning for 5G ProSe UE-to-Network relay services is mainly provided by the PCF rather than the UDM.

Therefore, this pCR proposes to add new text in TS 33.503 [1] clause 6.3.3.2 to define how PC5 security policies of ProSe relay service are provisioned to the UE by the 5G PKMF during security materials provisioning procedure. The pCR also proposes to replace the UDM with the PCF to be aligned with SA2 requirements.

# 4	Detailed proposal

*************** Start of the Change ****************

#### 6.3.3.1	Security requirements

Both user-plane (UP) based and control-plane (CP) based the procedure can be used for 5G ProSe Layer-3 UE-to-Network Relay authorization and security establishment. The UP based procedure uses a UP connection to the 5G PKMF, while the CP based procedure uses the primary authentication for PC5 key establishment.

The following are the security requirements for 5G ProSe L3 U2N relay communication:

-	For L3 relay security established over control plane, the PCF shall be able to provision the PC5 security policies to the Remote UE and L3 U2N relay respectively per ProSe relay service, during service authorization and information provisioning procedure as defined in TS 23.304 [2].

-	For L3 relay security established over user plane, the 5G PKMF shall be able to provision the PC5 security policies to the Remote UE and L3 U2N relay respectively per ProSe relay service, during security materials provisioning procedure defined in clause 6.3.3.2.

-	The PC5 UP security policies for protecting 5G ProSe relay communication shall be configured per ProSe relay service based on the security requirements of the specific relay service.

-	The activation of PC5 signalling security is based on PC5 CP security policies of the specific ProSe relay service.  PC5 signalling integrity security policy is set to “REQUIRED” for the Remote UE and 5G ProSe Layer-3 UE-to-Network Relay.

-	The activation of PC5 user plane security is based on PC5 UP security policies of the specific ProSe relay service.

-	The remote UE shall establish a different PC5 security context with each different relay UE and for each different Relay Service Code. It shall also be possible to establish a security context when the remote UE is out of coverage.

#### 6.3.3.2	Security procedure over User Plane

##### 6.3.3.2.1	General

This clause describes a mechanism to setup a PC5 link between a remote UE and UE-to-network relay. The mechanism includes how a Remote UE and UE-to-network relay get authorized by the ProSe Key Management Function (PKMF) and verify each other’s role.

Editor’s Note: Co-existence with CP based solution is FFS

##### 6.3.3.2.2	Remote UE attaching to a ProSe UE-to-network relay

Figure 6.3.3.2.2-1: Authorization and secure PC5 link establishment procedure for UE-to-network relay

The remote UE is provisioned with the discovery security materials (see clause 6.1.3.2) and Prose Remote User Key (PRUK) when it is in coverage. These security materials are associated with an expiration time, after which they become invalid. If the UE does not have valid discovery security materials, the Remote UE needs to connect to the PKMF and obtain fresh ones to use the UE-to-Network relay services.

The 5G PKMF is configured with a list of PC5 security policies of the ProSe UE-to-Network relay services it supports, which need to be provisioned to both the Remote UE and Relay UE as required in clause 6.3.3.1.

NOTE 1: The procedure is described for the scenario that the PKMF of the remote UE is different from the PKMF of the UE-to-network relay. If both the remote UE and the UE-to-network relay are served by a single PKMF, the PKMF takes the role of the PKMF of the remote UE and the PKMF of the UE-to-network relay and the inter-PKMF message exchanges are not needed.

NOTE 2: Steps 0a, 0b, 1a, 1b are performed when the remote UE is in coverage.

0a. The Remote UE gets the ProSe Key management function (PKMF) address from the 5G DDNMF of its HPLMN. Alternatively, the Remote UE may be provisioned with the PKMF address by PCF. If the Remote UE is provisioned with the PKMF address, the Remote UE may access the PKMF directly without requesting it from the 5G DDNMF. In case that the UE cannot access the PKMF using the provisioned PKMF address, the UE may request the PMKF address from the 5G DDNMF.

0b. The remote UE shall establish a secure connection with the PKMF via PC8 reference point. Security for PC8 interface relies on Ua security if GBA [8] is used (see clause 5.2.3.4) or Ua* security if AKMA [5] is used (see clause 5.2.3.5). The PKMF of the Remote UE shall check whether the Remote UE is authorized to receive UE-to-network relay services and if the UE is authorized, the PKMF of the Remote UE provides the discovery security materials to the Remote UE. The PKMF of the remote UE shall request the discovery security materials from the PKMFs of the potential relay UEs from which the remote UE gets the relay services, if the Remote UE provided the list of the visited networks.

NOTE 3: The PKMF may be locally configured with the UE’s authorization information. Otherwise, the PKMF interacts with the UDM to retrieve the UE’s authorization information.

NOTE 4: The remote UE is provisioned by PCF with the list of the potential visited networks for the UE-to-network relay service (which is identified by RSC).

0c. The UE-to-network relay gets the ProSe Key management function (PKMF) address from its HPLMN in the same way as described in step 0a.

0d. The UE-to-network relay shall establish a secure connection with the PKMF via PC8 reference point as in step 0b. The PKMF of the Relay UE shall check whether the UE-to-network relay is authorized to act as a relay and if authorized, the PKMF of the Relay UE provides the discovery security materials to the UE-to-network relay.

1a. The Remote UE sends a PRUK Request message to its PKMF. The message indicates that the Remote UE is requesting a PRUK from the PKMF. If the Remote UE already has a PRUK from this PKMF, the message shall also contain the PRUK ID of the PRUK.

1b. The PKMF checks that the Remote UE is authorised to receive UE-to-network Relay services. This is done by using the Remote UE’s identity associated with the key used to establish the secure connection between the Remote UE and PKMF in step 0b. If the Remote UE is authorised to receive the service, the PKMF sends a PRUK and PRUK ID to the Remote UE. If a PRUK and PRUK ID are included, the Remote UE shall store these and delete any previously stored ones for this PKMF.

2. The discovery procedure is performed between the Remote UE and the UE-to-network Relay using the discovery parameters and discovery security material as described in clause 6.1.3.2.

3. The Remote UE sends a Direct Communication Request (DCR) that contains the PRUK ID, Relay Service Code (RSC) of the requested UE-to-network relay service and KNRP freshness parameter 1 to the UE-to-network relay. If PRUK ID does not contain the HPLMN ID of the Remote UE or the routing information to the PKMF of the Remote UE (e.g., realm part when the NAI format of PRUK ID is used), the DCR message shall include the HPLMN ID of the Remote UE. The PC5 security establishment procedure between the Remote UE and the UE-to-network relay including security parameters and security policy negotiation and protection of messages hereafter shall follow the one-to-one security establishment described in clause 6.2.3 of the present document. Only additional parameters required for the Layer-3 UE-to-network relay scenario are described in this subclause.

Editor’s Note: privacy of PRUK ID is FFS.

4a. The UE-to-network relay sends a Key Request message that contains PRUK ID, RSC and KNRP freshness parameter 1 to its PKMF. The Key Request message shall also include the HPLMN ID of the Remote UE if it is included in the DCR.

4b. On receiving the Key Request message, the PKMF of the UE-to-network relay shall check if the UE-to-network relay is authorized to act as a relay for the relay service requested by the Remote UE based on the UE-to-network relay’s identity associated with the key used to establish the secure PC8 connection and the received RSC. If the UE-to-network relay is authorized to provide the requested relay service, the PKMF of the UE-to-network relay sends the Key Request with the PRUK to the PKMF of the remote UE. The PKMF of the Relay UE identifies the PKMF address of the Remote UE based on the PRUK ID or HPLMN ID of the Remote UE if it is included in the Key Request message.

Editor’s Note: PKMF’s authorization check of the UE with UDM requires alignment with SA2.

4c. On receiving the Key Request message from the PKMF of the UE-to-network relay, the PKMF of the Remote UE shall check if the Remote UE is authorized to use the requested relay service based on the PRUK ID and RSC included in the Key Request message.

If the PKMF determines a PRUK to be refreshed, the PKMF shall perform the one of the following procedures:

-	If the PKMF of the Remote UE supports the Zpn interface to the BSF of the Remote UE, the PKMF of the Remote UE may request a GBA Push Info (GPI – see TS 33.223[xx]) for the Remote UE from the BSF. When requesting the GPI, the PKMF shall include a PRUK ID in the P-TID field. On receiving the GPI, the 5G PKMF shall use Ks(_ext)_NAF as the PRUK.

-	If the PKMF supports the SBI interface to the BSF of the Remote UE, the PKMF may request the GPI via SBI interface as described in TS 33.223[xx]. On receiving the GPI, the 5G PKMF shall use Ks(_ext)_NAF as the PRUK.

-	If the PKMF of the Remote UE supports the PC4a interface to the HSS of the UE, then the PKMF of Remote UE may request a GBA Authentication Vector (AV) for the Remote UE from the HSS. On receiving the AV, the PKMF locally forms the GPI including a PRUK ID in the P-TID field. The PKMF shall use Ks(_ext)_NAF as the PRUK.

-	If the PKMF is co-located or integrated with BSF functionality and supports the SBI interface to the UDM/HSS of the Remote UE, the PKMF may request the GBA AV via SBI interface as described in TS 33.220 [8]. On receiving the AV, the PKMF locally forms the GPI including a PRUK ID in the P-TID field. The PKMF shall use Ks(_ext)_NAF as the PRUK.

Editor’s Note: GBA push info (GPI) aspects are still under discussion in TR 33.847 [x] therefore additional details for GPI handling are FFS.

The PKMF of the remote UE shall generate KNRP freshness parameter 2 and derive KNRP using the PRUK identified by PRUK ID, RSC, KNRP freshness parameter 1 and KNRP freshness parameter 2. The PKMF shall also select the PC5 security policies for the requested relay service based on the RSC and its configured list of PC5 security policies for all UE-to-Network relay services it supports. Then, the PKMF of the remote UE sends a Key Response message that contains KNRP and KNRP freshness parameter 2 and the PC5 security policies of the requested relay service to the PKMF of the UE-to-network relay. This message shall include GPI if generated.

4d. The PKMF of the UE-to-network relay sends the Key Response message to the UE-to-network relay, with which the relay UE is provisioned the PC5 security policies of the requested relay service.

5a. The UE-to-network relay shall derive the session key (KNRP-SESS) from KNRP and then derive the confidentiality key (NRPEK) (if applicable) and integrity key (NRPIK) based on the PC5 security policies as specified in TS 33.536 [6]. The UE-to-network relay sends a Direct Security Mode Command message to the Remote UE. This message shall include the KNRP Freshness Parameter 2 and the PC5 security policies of the requested relay service, which shall be protected as specified in TS 33.536 [6].

5b. If the Remote UE receives the message containing the GPI, it processes the GPI as described in TS 33.223[xx]. The Remote UE shall derive the PRUK and obtain the PRUK ID from the GPI.

The Remote UE shall derive KNRP from its PRUK, RSC, KNRP Freshness Parameter 1 and the received KNRP Freshness Parameter 2. It shall then derive the session key (KNRP-SESS) in the same manner as the UE-to-network relay and process the Direct Security Mode Command. The Remote UE shall also store and apply the PC5 security policies of the requested relay service received from the UE-to-network relay, based on which the Remote UE shall derive the confidentiality key (NRPEK) (if applicable) and integrity key (NRPIK). Successful verification of the Direct Security Mode Command assures the Remote UE that the UE-to-network relay is authorized to provide the requested relay service.

5c. The Remote UE responds with a Direct Security Mode Complete message to the UE-to-network relay protected as specified in TS 33.536 [6].

5d. On receiving the Direct Security Mode Complete message, the UE-to-network relay shall verify the Direct Security Mode Complete message.  Successful verification of the Direct Security Mode Complete message assures the UE-to-network relay that the Remote UE is authorized to get the relay service.

6. The remote UE and UE-to-network relay continues the rest of procedure for the relay service over the secure PC5 link.

*************** End of the Change ****************