3GPP TSG-SA3 Meeting #106-e 	S3-220287-r4

- e-meeting, 14 – 25 February 2022

- Source:	Samsung

- Title:	Evaluation and Conclusion for Key Issue #9

- Document for:	Approval

Agenda Item:	5.11

# 1	Decision/action requested

This contribution proposes to introduce evaluation and conclusion for key issue #9 in TR 33.875.

# 2	References

NA

# 3	Rationale

Conclusion on KI #9 is proposed.

The parameter "allowedOperationsPerSnssai" allowes finer control of scopes allowed to a consumer from a different slice. Hence, it is proposed to utilize this parameter instead.

Additionally, use of parameter "Requester-NSSAI" is a minor enhancement to existing Access-Token Grant, and allows NF-producers validate the NF-Consumer's S-NSSAI on its own. Hence, it is proposed to allow inclusion of this parameter in the Access-Token grant.

# 4	Detailed proposal

*** First Change ***

## 6.11	Solution #11: Registered NF Profile changes for Inter-Slice Access

### 6.11.1	Introduction

This solution proposes to address following security requirement of KI #9:

- 	The 5GS should provide a mechanism that allows the NF Service Producer not to provide service to NF consumers, which are not authorized to access a slice.

This solution gives the option for NF Service Producers to indicate which consumer’s S-NSSAIs are allowed to access which producer’s S-NSSAIs, and by which operations in its profile.

### 6.11.1	Solution Details

Using individual or combination of IEs like allowedNfTypes and allowedNssais, NRF can already filter the requests from NF Service Consumers not allowed to access any services of an NF Service Producer.

To address the issue of static configurations at NRF for restricted Inter-slice access, it is proposed that:

-	When an NF Service Producer registers its profile in NRF, it includes a new information element, “allowedSnssaisPerSnssai”, which specifies, for each NF-Service Consumer’s S-NSSAI, the list of NF-Service Producer’s S-NSSAIs it is allowed to access.

-	When an NF Service Producer registers its profile in NRF, it includes a new information element, “allowedOperationsPerSnssai”, which specifies allowed operations on its resources, for NF Consumers belonging to the given S-NSSAI.

These attributes can then be used by NRF to determine the “producerSnssaiList” to be included in the AccessTokenClaims. Additionally, NRF can determine whether a given resource/operation-level scope can be granted to an NF Service Consumer that requested an Oauth2 access token with a specific scope, or limit the scope or authorization irrespective of the request.

Additionally, to allow NF Service Producers (themselves) validate the “Inter-Slice” access requests from the NF Service consumers, it is proposed to include “Requester-NSSAI” in the access-grant, indicating the NRF validated NF-Consumer’s S-NSSAIs.

The NF Service Producer can thus check the decision of the NRF to assign an access token, but is still dependent on the NRF validating the consumer’s S-NSSAI and including it in the access token.

Editor’s Note: Definition of Slice Resource is FFS.

### 6.11.3	Evaluation

This solution proposes to address the following security requirement of KI #9:

- 	The 5GS should provide a mechanism that allows the NF Service Producer not to provide service to NF consumers, which are not authorized to access a slice.

The solution gives the option for NF Service Producers to indicate which consumer’s S-NSSAIs are allowed to access which producer’s S-NSSAIs, and by which operations, in its profile.

Editor’s Note: Further evaluation is FFS.

*** End of Changes ***