3GPP TSG-SA3 Meeting #106-e 	draft_S3-220300-r3

e-meeting, 14 - 25 February 2022

{Guidance text shown in curly brackets, in italics. All guidance text is to be deleted before WID submission.

Note that this form is to be used for all types of Work Item, covering both normative work and/or studies. A Work Item covering only studies is also known as "Study Item", as stated in TR 21.900.}

- Source:	Nokia

- Title:	R18 SID on Standardising Automated Certificate Management in SBA

- Document for:	Approval

Agenda Item:	5.15 Study on Standardising Automated Certificate Management in SBA

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	Study on Standardising Automated Certificate Management in SBA

## Acronym: FS_ACM_SBA

## Unique identifier: 	 920022

Potential target Release: {Rel-18}.

Note that this field above indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan.

## 1	Impacts { For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study.}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  |  |
| Don't know |  |  |  |  | X |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a …

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

### 2.2	Parent Work Item

{Not applicable for a Study Item}

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

## 3	Justification

According to TS33.501, use of mutual TLS for authentication of NF require compliance to 3GPP TS33.310 section 6.1.3c for TLS client and TLS server certificate profiles in addition to TLS profile compliance with section 6.2a of TS33.310.

Use of TLS certificates in 5G SBA is ubiquitous. Also as per section 9.9 of TS33.501, certificate based NDS/IP is to be used for protection of non-SBI interfaces. E.g. N4, N9.

However, unlike standardised model using CMPv2 in RAN, SBA does not have a standardised model and set of procedures for automated certificate management.

SBA also does not have a standardised protocol for managing life cycle events of the certificates. e.g. bootstrap, request, issue, enrolment, revocation, renewal etc.

- Lack of standardisation has resulted into number of bespoke methodologies and varying choices of certificate management protocols resulting into inconsistent model.

- Once service slicing and NPN are introduced in service provider network, manual management or lack of standardised procedures for life cycle management of TLS certificates belonging to separate legal entities could further complicate the architecture.

All the above have potential of increasing the security risk and impact the deployment and to availability of operators’ 5G SBA network.

RAN has benefitted from the standardisation of CMPv2 to be used for eNodeB/gNodeB automated certificate management. The specification defined a bootstrap procedure based on the use of vendor certificate for requesting an operator certificates for the set-up of IPSec IKE2 towards the SeGW. 5G SBA is within the operator core network domain that could benefit from a study that leads to the standardisation of an automated certificate management procedure using a standardised protocol that is fit for purpose to serve the 5G Core Network.

## 4	Objective

The objectives of this study are to identify key issues, potential security and privacy requirements and solutions with respect to

- Standardise the use of a single automated certificate management protocol and procedures for certificate life cycle events within intra-PLMN 5G SBA (i.e. to be used by all 5GC NFs including NRF, SCP, SEPP etc.)

- Study the impact of virtualization in 5G Core Network (e.g. cloud native architectures) in certificate management within 5G SBA.

- Study which lifecycle events (e.g. enrolment, renewal, revocation, status monitoring) of a certificate need to be covered.

- Study the relation between certificate management lifecycle and NF management lifecycle.

- Study to reference at minimum following principles:

- Principle to be reusable when 5G SBA is for NPN (standalone and PNI)

- Principles standardised to be able to support NFs doing mutual TLS in Slicing

- Principles standardised to support both intra and inter PLMN SBA scenarios

- Principles involving ‘Chain of Trust’ of Certificate Authorities hierarchies

- Principles for security of CA’s cryptographic private key

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| TR | 33.xxx | Study on Standardising Automated Certificate Management in SBA |  |  |  |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

## 6	Work item Rapporteur(s)

Nokia, German Peinado, german.peinado@nokia.com

## 7	Work item leadership

SA3

## 8	Aspects that involve other WGs

## 9	Supporting Individual Members

Supporting IM name

Nokia

Ericsson

BT

Mavenir

Deutsche Telekom

NTT DOCOMO

CableLabs

Charter Communications

Samsung

Verizon