3GPP TSG-SA3 Meeting #106-e 	S3-220314r1

- e-meeting, 14 - 25 February 2022

- Source:	Qualcomm Incorporated

- Title:	Details of pairing in EPS

- Document for:	Approval

Agenda Item:	4.12

# 1	Decision/action requested

This contribution provide text for pairing in 4G.

# 2	References

[1]	3GPP TS 33.256

# 3	Rationale

This contribution provide text for pairing in 4G. The below pCR proposes text to complete this.

# 4	Detailed proposal

It is proposed that SA3 approve the below pCR for inclusion in the TS [1].

**** START OF CHANGES ****

- 5.4.1	General

Pairing authorization in 5GS is performed during either a PDU Session Establishment procedure or a PDU Session Modification procedure. Pairing authorization in EPS is performed during either a PDN Connection Establishment procedure or a PDN Connection Modification procedure.

Editor's Note:	UAV pairing authorization during PDU Session Modification is FFS.

- **** NEXT CHANGE ****

- 5.4.X		UAV pairing Authorization with UAVC in EPS

Pairing authorization may be performed during a PDN Connection Establishment/PDN Connection  Modification procedure after a successful UUAA between the UAV and the USS/UTM. If no successful UUAA has been performed, then the pairing authorization can occur during the during the UUAA  procedure (see clause 5.2.5.3.0of TS 23.256 [3] for full details). This procedure follows the clause 5.X.Y.Z with the following additions:

the UE provides pairing information (if available) in a C2 authorization payload and this is forwarded to the USS in steps 2 and 3; and

after a successful authentication and before sending the message in step 5, the USS performs C2 authorization considering the included pairing information, the CAA-Level UAV ID and 3GPP UAV ID/GPSI. The USS includes a C2 authorization payload that contains C2 session security information if the USS has such information to send to be passed to the UE in steps 5-7. The content of C2 session security information (e.g., key material to help establish security between the UAV and UAV-C) is not in 3GPP scope

UAV pairing authorization during the PDN Connection Establishment/ Modification procedure is described as follows. Full details of the procedures are given in TS 23.256 [3].

Figure 5.4.X-1: UAV pairing authorization during PDN Connection Establishment/Modification

1. When the UAV needs a new dedicated PDU session for connectivity to the UAV-C, the UE initiates a PDN Connection Session Establishment procedure.  When the UAV needs to use an existing PDN connection for connectivity to the UAV-C, the UE initiates a PDN Connection Modification procedure. The UE shall include the following IEs in the PDN connection establishment/modification request: a CAA-Level UAV ID, a DNN/S-NSSAI implying dedicated connectivity to UAV-C, and UAV pairing information, which includes any needed authorization information, if available.

The pairing information includes the CAA-level UAV IDs of the requesting UAV and identification information of UAV-C to pair. The USS may also use its locally configured pairing information for UAV and UAV-C pairing authorization which takes precedence over UAV provided pairing information.

NOTE: The integrity protection of pairing information is recommended. It is performed by the USS, and is not in scope of 3GPP system.

2. The SMF+PGW-C determines whether the UAV pairing authorization is required based on UAV’s aerial subscription, presence of CAA-Level UAV ID, and DNN/S-NSSAI indicating the UAV service,

The SMF+PGW-C invokes the authorization procedure with the USS via UAS-NF. The USS will perform C2 authorization taking account of the included pairing information, which includes any needed authorization information, if available, the CAA-Level UAV ID, and GPSI etc.

The USS informs the SMF+PGW-C via the UAS NF of the authorization results. The authorization information includes the IP address of the UAV-C and a C2 authorization payload that contains C2 session security information plus possibly other non-security specific information (e.g. C2 authorization result, i.e., whether the UAV is allowed to be paired with the UAV-C) if the USS has such information to send. The content of C2 session security information (e.g., key material to help establish security between the UAV and UAV-C) is not in 3GPP scope. The other information contained in this message is given in TS 23.256 [3].

3. The SMF+PGW-C sends the UE the C2 authorization payload paring authorization result and may also send a new CAA-level UAV ID. The UE shall store the Pairing authorization result and authorization information.

The PDN Connection Establishment/Modification continues and completes as described in TS 23.256 [3].

The UAV pairing authorization can be revoked by the USS at any time.

Besides, the paired UAV-C can be replaced by a new UAV-C by the USS at any time.

**** END OF CHANGES ****