3GPP TSG-SA3 Meeting #107e AdHoc 	S3-221369

- e-meeting, 27 June - 1 July 2022

- Source:	China Telecom

- Title:	New solution on topology hiding in data and analytics exchange in roaming case

- Document for:	Approval

Agenda Item:	5.8

# 1	Decision/action requested

This contribution proposes to add new solution on topology hiding in data and analytics exchange in roaming case.

# 2	References

[1]	3GPP TS 33.501 Security architecture and procedures for 5G system

# 3	Rationale

The contribution proposes to add new solution on topology hiding in data and analytics exchange in roaming case.

# 4	Detailed proposal

It is suggested to approve the following change.

*************** Start of the change ****************

## 6.Y	Solution #Y: Topology hiding in data and analytics exchange in roaming case

### 6.Y.1	Introduction

This solution is addressing KI#1. It proposes that SEPP performs topology hiding for data and analytics exchange in roaming case.

### 6.Y.2	Solution details

As per clause 13.1.1.1 of TS 33.501, SEPP generates a telescopic FQDN and rewrites the original FQDN for the following messages received over N32-f: Nnrf_NFDiscovery_Get response HTTP message with FQDNs of a set of the discovered NF or NF service instance(s), subscription message with the Callback URI in the payload of the message, Nsmf_PDUSession_POST HTTP message from a V-SMF with PduSessionCreateData containing the URI representing the PDU session in the V-SMF.

As per clause 28.5.2 of TS 23.003, the structure of telescopic FQDN is<Label representing FQDN from other PLMN>.<FQDN of the SEPP in the request initiating PLMN>, and FQDN from other PLMN may be rewritten by the other PLMN SEPP for topology hiding.

To perform topology hiding when the HPLMN/VPLMN collect data or consume analytics from the VPLMN/HPLMN, the SEPP generates telescopic FQDN for messages involve internal network information (e.g., FQDN of application server) and rewrite the label representing FQDN from other PLMN in the telescopic FQDN.

### 6.Y.3	Evaluation

TBD.

*************** End of the change ****************