3GPP TSG-SA3 Meeting #107Adhoc-e 	draft_S3-221331-r1

- e-meeting, 27th June – 1st July, 2022 	merger of S3-221418, S3-221421, S3-221427, S3-221495

- Source:	Interdigital, Huawei, HiSilicon, China Telecomunication, ZTE, CATT

- Title:	Key Issue on Authorization in the UE-to-UE Relay Scenario

- Document for:	Approval

Agenda Item:	5.3

# 1	Decision/action requested

This contribution proposes to add Key issue on authorization in the UE-to-UE relay scenario in TR 33.740.

# 2	References

# 3	Rationale

This contribution proposes to add a new key issue on authorization in the UE-to-UE relay scenario.

# 4	Detailed proposal

**** 1st CHANGE ****

## 5.X	Key issue #X: Authorization in the UE-to-UE Relay Scenario

### 5.X.1	Key issue details

TR 23.700-33 [Y], key issue #1 describes its key Issue regarding support of UE-to-UE Relay:

"-	Whether and how the network can control UE-to-UE Relay operation, at least including how to:

-	Authorize the UE-to-UE Relay, e.g. authorize a UE as UE-to-UE Relay.

-	Authorize Source/Target UEs to use a UE-to-UE Relay.

…

NOTE 3:	For security/privacy protection aspects, coordination with SA WG3 is needed."

From a security point of view, whether the UE can act as a UE-to-UE Relay should be assured by the Source UE or Target UE. Similarly, whether the UE can act as a Source UE or Target UE should be assured by the UE-to-UE relay.

3GPP system should be able to authorize a UE to perform as UE-to-UE Relay and a UE to communicate with another UE via a UE-to-UE Relay.

### 5.X.2	Security threats

An attacker may impersonate the UE-to-UE Relay. If the Source/Target UE cannot verify if the UE acting as UE-to-UE relay is authorized, the attacker UE may impersonate the UE-to-UE relay. The attacker may then deny the UE services between the two UEs (e.g., arbitrary discard messages).

Similarly, an attacker may impersonate the Source UE or the Target UE.

### 5.X.3	Potential security requirements

The 5GS shall support authorization of the UE as a UE-to-UE relay in the UE-to-UE relay scenario.

The 5GS shall support authorization of the UE as a Source UE or a Target UE in discovering the UE-to-UE relay in the UE-to-UE relay scenario.

3GPP system shall provide means to authorize a UE to communicate with another UE via a UE-to-UE Relay.

#### *** NEXT CHANGE ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[Y]	3GPP TR 23.700-33: "Study on System enhancement for Proximity based Services (ProSe) in the 5G System (5GS); Phase 2".

**** END OF CHANGES ****