3GPP TSG-SA3 Meeting #107Adhoc-e 	draft_S3-221332-r3

- e-meeting, 27th June – 1st July, 2022 	merger of S3-221383, S3-221422, S3-221428, S3-221491, S3-221549, S3-221503

- Source:	Interdigital, Huawei, HiSilicon, China Telecomunication, ZTE, CATT, Xiaomi, OPPO

- Title:	Key Issue on Security of UE-to-UE Relay

- Document for:	Approval

Agenda Item:	5.3

# 1	Decision/action requested

This contribution proposes to add Key issue on Security of UE-to-UE Relay in TR 33.740.

# 2	References

# 3	Rationale

This contribution proposes to add a new key issue on Security of UE-to-UE Relay.

# 4	Detailed proposal

**** 1st CHANGE ****

## 5.X	Key Issue #X: Security of UE-to-UE Relay

### 5.X.1	Key issue details

3GPP system has to be able to protect security (i.e., the integrity and confidentiality) of information between the peer UEs over the UE-to-UE Relay. Failure to protect integrity and confidentiality of information exchanged between the peer UEs over the UE-to-UE Relay will open vulnerability in 5GS and allow various attacks such as unauthorized disclosure and modification of information. Protection of communications between the peer UEs should take into consideration that the UE-to-UE Relay is an untrusted node.

TR 23.700-33 [xx], key Issue #1: Support of UE-to-UE Relay, has the following key issue:

-	How to enhance the system architecture to provide security/privacy protection for a relayed connection.

...

NOTE 3:	For security/privacy protection aspects, coordination with SA WG3 is needed.”

### 5.X.2	Security threats

Failure to protect integrity and confidentiality of information exchanged between the peer UEs over the UE-to-UE Relay will open vulnerability in 5GS and allow various attacks such as unauthorized disclosure and modification of information.

When considering the UE-to-UE Relay as an untrusted node the security (i.e., the integrity and confidentiality) of information exchanged between the peer UEs may be compromised. Therefore, secure exchange of end-to-end information between the peer UEs communicating over the UE-to-UE Relay is needed.

A malicious Relay UE that can establish a unicast link with the source UE, as well as the target UE may conduct an MITM attack.

Failure to protect integrity and confidentiality of information during path change will open vulnerability in 5GS and allow various attacks resulting in unauthorized disclosure and modification of information.

### 5.X.3	Potential security requirements

The 5GS shall support the secure establishment of the connection between the source UE and the target UEs via UE-to-UE Relay.

3GPP system shall provide means to protect security (i.e., confidentiality, integrity, and replay protection) of user plane data and signalling information between the peer UEs during communications over the UE-to-UE Relay.

3GPP system shall provide means to protect security (i.e., the integrity, confidentiality, and replay protection) of user plane data and signalling information during UE-to-UE Relay path switch.

**** END OF CHANGES ****