3GPP TSG-SA3 Meeting #107Adhoc-e 	draft V1 S3-22xxxx

- e-meeting, 27th June – 1st July 2022                                                      Revision of S3-221349

- Source:	Nokia, Nokia Shanghai Bell

- Title:	Key issue on securing AIML operation

- Document for:	Approval

Agenda Item:	5.20

# 1	Decision/action requested

It is requested to approve the new KI on securing AIML operation.

# 2	References

[X]		S3-221188 "New SID on Security and Privacy of AI/ML-based services and applications in 5G"

[Y]		3GPP TR 23700-80 "Study on 5G System Support for AI/ML-based Services"

- 3	Rationale

This contribution introduces a new KI in the SID on Security and Privacy of AI/ML-based services and applications in 5G.

It is related to objective #3 in SID S3-221188 [1] which is defined as

3.   UE and 5G system to secure AI/ML based services and operations.

- 4	Detailed proposal

**** START OF CHANGE ****

- 6.X	KI #X: Securing AIML operations

- 6.X.1	Key issue details

As a part of the AIML study, the following kinds of AIML operations are defined:

-	AI/ML operation splitting between AI/ML endpoints

-	AI/ML model/data distribution and sharing over 5G system

-	Distributed/Federated Learning over 5G system

The UE and AF perform these AIML operations. Especially in the last 2 operations, AIML-related data is transferred between the AF and UEs. Some of these operations are AF initiated as well. For example (FL use case), AF selects the UEs and sends a request to the UEs for model transfer. The UE trains the model and provides interim training results to the AF, and then the AF aggregates the parameters from different UEs and prepares the global model. However, it is not clear how the 5G system will secure these AIML operations between AF and UE.

Please note that AKMA provides a framework where AF can authenticate a UE (i.e. UE initiated communication via A-KID) and secure the communication between UE and AF. However, current AKMA functionality will not work in this case where AF wants to initiate communication with different UEs.

- 6.X.2	Security threats

If UE and AF data transmission is not protected, the interim training results sent to the AF may not be accurate and could lead the AF into developing a model that is not useful. A persistently incorrect training model will lead to waste of 5G system and UE resources.

- 6.X.3	Potential requirements

The interface between UE and AF shall be confidentiality, integrity, and replay protected.

**** END OF CHANGE ****