- 3GPP TSG-SA3 Meeting #107e AdHoc	S3-221436-r1

- e-meeting,  27 June - 1 July 2022

- Source:	ZTE

- Title:	Home network triggered authentication solution for 4G to 5G interworking on Key issue #1

- Document for:	Approval

- Agenda Item:	5.7

# 1	Decision/action requested

This document proposes to add a new solution on key issue #1: Ability of the home network to trigger primary authentication. SA3 is kindly requested to approve this doc.

# 2	References

[1]	3GPP TR 33.741 v0.1.0

# 3	Rationale

When the UE moves from EPS to 5GS, AMF may use the mapped 5G security context constructed from the EPS security context to protect the subsequent messages, which does not include a KAUSF key [1]. This solution provides a home network triggered primary authentication mechanism for interworking from EPS to 5GS with N26.

# 4	Detailed proposal

**** START OF CHANGE ****

## 6.Y	Solution #Y: Home network triggered authentication solution for 4G to 5G interworking

### 6.Y.1	Introduction

This solution addresses the KI #1: Ability of the home network to trigger primary authentication. And it relates to use case #1: Security of Interworking

### 6.Y.2	Solution details

During the mobility and handover procedures from EPS to 5GS, the UE sends Registration Request to 5G network with registration type set to "Mobility Registration Update".

- In the registration procedure, after the AMF registers with the UDM using Nudm_UECM_Registration, UDM checks if it supports home network services requiring KAUSF, for example, UPU, SoR and AKMA. UDM also checks for the corresponding AUSF instance ID. If UDM supports the home network services but doesn’t have AUSF instance (or doesn’t keep track of AUSF that stores the latest KAUSF generated after successful completion of the latest primary authentication reported to the UDM), it replies error information to AMF and indicates AMF to initiate UE (re-)authentication before continuing the registration procedure. For example, it responds with 403 Forbidden with the cause “REAUTHENTICATION_REQUIRED”.

- Editor’s Note: Impact on current system on reusing the error code is FFS.

- NOTE: No AMF’s reaction on this error code in stage 3.

- NOTE: this error code is only used for interworking.

### 6.Y.3	Evaluation

Editor’s Note: Evaluation is FFS.

**** END OF CHANGE ****