3GPP TSG-SA3 Meeting #107e AdHoc 	draft_S3-221470-r2

- e-meeting, 27 June - 1 July 2022

- Source:	Nokia, Nokia Shanghai Bell

- Title:	Solution for AI/ML model authorization and retrieval.

- Document for:	Approval

Agenda Item:	5.8

# 1	Decision/action requested

It is requested to approve the new Key issue for FS_eNA_SEC_Ph3

# 2	References

# 3	Rationale

In the last SA3 meeting, key issue #3 was approved under tdoc S3-221221. This solution addresses the threats and security requirements of the key issue, specifically the authorization aspects in the retrieval of the AI/ML process.

# 4	Detailed proposal

************ START OF CHANGES ************

- 6.y	Solution #Y: AI/ML model authorization and retrieval

- 6.y.1	Introduction

This addresses KI#3 on security for AI/ML model storage and sharing, specifically the authorization aspects for the access to the AI/ML models, i.e.:

- ADRF (Analytical Data Repository Function), or any other network function which may store the AI/ML model, shall be able to authorize the NFc to retrieve that AI/ML model

- NF Service consumers shall be authorized to access to the AI/ML models in the ADRF (or any other NF which may store the ML model, for instance NWDAF MtLF).

- 6.y.2	Solution details

The solution defines an authorization schema for retrieval of AI/ML models in 5GS based on OAuth 2.0.

The procedure is as follows:

6.y.2-1: AI/ML model authorization procedure

- NF producer which trains the model (e.g., NWDAF MtLF), while storing the ML model in the ADRF, it also appends its metadata. The metadata with respect to each model may contain attributes such as Model Info/ analytics Id(s) supported by the model, NF Type / Instance Id and vendor Id that can eventually retrieve a particular model, etc.

Editor’s note: The content of the metadata is ffs

- NF producer shall also register the model specific information including the model metadata when registering its profile in the NRF. This information includes the Model Info/Analytics ID, allowed NF Type/Id and allowed NF Id per Model.

- NFc when requesting the access token includes at least the Model Id and/or Analytics Id for which a trained model is needed.

- NRF when receiving the access token request, verifies that the NFc is authorized to retrieve the model from ADRF.

- In case of valid authorization, the NRF provides the token with access token claims including the Model Id, and optionally also the Analytics Id, identifying the type of analytics that the model may be providing.

- NFc now provides the access token with the model retrieval service request to the ADRF.

- ADRF verifies the access token the ensures that the NFc is indeed authorized for the requested model by verifying the access token claims and then also updates the metadata info of the Model (received in Step 1) to also include the authorized NFc info (NFc Instance ID, NFc type).

- In case of successful verification ADRF provides the NFc a URI to download the model as a service response.

- NFc initiates the download of the ML model using the URI and providing its credential information as part of the request (e.g., CCA).

Editor’s note: if CCA or other user credentials are needed is ffs.

Editor’s note: Implication of ADRF, a repository/database, supporting credential management is ffs.

- ADRF verifies that the NFc, which is trying to download the model, is indeed authorized by verifying the credentials against the metadata updated in Step 7).

- ADRF after a successful verification finally initiates the model download at the NFc.

Editor’s note: Alignment with SA2 procedure is ffs.

- 6.y.3	Evaluation

TBD

************ END OF CHANGES ************