3GPP TSG-SA3 Meeting #107Adhoc -e	S3-221530-r1

- e-meeting, 27th June – 1st July, 2022

- Source:	Samsung

- Title:	New solution on HN initiated Re-authentication via AUSF

- Document for:	Approval

Agenda Item:	5.7

# 1	Decision/action requested

This contribution proposes a new solution to address KI#1 in TR 33.741.

# 2	References

[1]	3GPP TR 33.741 Security aspects of home network triggered re-authentication.

# 3	Rationale

This contribution proposes a solution for the KI#1, Ability of the home network to trigger primary authentication.

# 4	Detailed proposal

*****Start of Change*****

## 6.Y	Solution #Y:  AMF initiated primary authentication based on AUSF request

### 6.Y.1	Introduction

This solution addresses KI#1. A new primary authentication may require for certain events at the network, resulting in refresh of the latest home key KAUSF. In such scenarios, an internal Network Function (AUSF) requests the AMF to trigger the re-authentication procedure. The AMF initiates re-authentication of the UE, if an internal NF requests (AUSF) to initiate re-authentication to refresh the UE specific home key (KAUSF).

### 6.Y.2	Solution details

Figure 6.Y.2-1: AMF initiated primary authentication

- The primary authentication is performed as specified in TS 33.501 [3]. After successful authentication, KAUSF is derived at the AUSF and at the UE.

- AUSF determines (for e.g., due to long time availability of same key, etc) the need of refreshing the KAUSF key.

- If the AUSF determines that there is a need to refresh the KAUSF, it decides to perform primary authentication to refresh the home key KAUSF. AUSF requests UDM to provide UE’s current AMF by sending Nudm_UECM_Get request. Prior to sending the request to UDM, the AUSF checks whether the primary authentication for the UE to be initiated or request to be rejected, based on the operator policy.  Operator policy includes the details of the wait period for the new request, after the last successful authentication.

- Upon receiving the request for the details of the current serving AMF from the AUSF, UDM provides UE’s current AMF details in Nudm_UECM_Get response message.

- The AUSF prepares to indicate re-authentication and requests the AMF to initiate primary authentication for the UE by invoking Namf_UEAuthentication_Authenticate service operation.

- Upon receiving the request from the AUSF, the AMF (SEAF) initiates the primary authentication as described in clause 6.1.2 of TS 33.501 [3], resulting in generation of fresh key material in the UE and in the network as described in clause 6.2 of TS 33.501 [3], if the primary authentication is performed successfully.

### 6.Y.3	Solution Evaluation

TBD

*****End of Change*****