3GPP TSG-SA3 Meeting #107Adhoc -e	S3-221533-r1

- e-meeting, 27th June – 1st July, 2022

- Source:	Samsung

- Title:	Key issue on Cyber-attack detection

- Document for:	Approval

Agenda Item:	5.8

# 1	Decision/action requested

This contribution proposes a new KI on the cyber-attack detection in TR 33.738.

# 2	References

[1]	3GPP TR 33.738 Study on security aspects of enablers for Network Automation for 5G - phase 3.

# 3	Rationale

In 105e SA3 meeting, SID S3-220563 was agreed to address security aspects of enablers for Network Automation for 5G – phase 3 (study approved in SA2 Rel-18: SP-211330, TR 23.700-81[6] ). One of the objectives of the SID refers to study KIs from Release 17 in TR 33.866, which did not have enough time to proceed, e.g., NWDAF detection related issues. This key issue brings back Cyber-attack detection key issue to Release 18.

# 4	Detailed proposal

*****Start of Change*****

## 5.X	Key issues on Cyber-attack

### 5.X.1	Key Issue Details

TR 23.700-91 has identified the use case of NWDAF detecting cyber-attacks by monitoring events and data packets in the UE and the network, with the support of machine-learning algorithms. To achieve cyber-attacks detection, the NWDAF can collaborate with UE and any other NFs to collect related data as inputs and providing alerts of anomaly events as outputs to OAM and other NFs which have subscribed to them so that they could take proper actions.

This key issue describes what kind of cyber-attacks can be detected. In order to mitigate the identified cyber-attacks, the data/parameters collected by the NWDAF or any other NF need to be studied.

The specific cyber-attacks for which an analytics function may provide detection support include but are not limited to the following examples:

(1) MitM attacks on the radio interface: MitM attacks or fraudulent relay nodes may modify or change messages between the UE and the RAN, resulting in failures of higher layer protocols such as NAS or the primary authentication.

(2) DoS attacks: 5G has high performance requirements for system capacity and data rate, improved capacity and higher data rate may lead to much higher processing capability cost for network entities, which may make some network entities (e.g. RAN, Core Network Entities) to suffer from DDoS attack. The NFs may also enable the detection of DDoS attacks. .

### 5.X.2	Security Threats

Cyber-attack may not be detected by the 5G network; thus further attacks could be conducted.

Anomaly events may not be detected by the 5G network; thus further attacks could be conducted.

### 5.X.3	Potential Security Requirements

The 3GPP system shall support the detection of cyber-attacks by providing related inputs or collecting output analytics using an analytics function (for e.g., NWDAF).

*****End of Change*****